package regulationOfFloodControl;

import regulationOfFloodControl.function.Param;

import tools.arry;

import tools.matrix;

import tools.writeTxt;

import java.io.File;

import java.util.Map;

import java.text.DecimalFormat;

import static regulationOfFloodControl.function.floods.floodData;

import static regulationOfFloodControl.function.plans.plan;

/*

1、汛限水位起调；

2、转换水位以下，江坪河、江垭满发，皂市按石门9500m3/s补偿

3、转换水位以上，转按各自基础方案

*/

public class FloodControl {

public static void main(String[] args) {

System.out.println("程序开始运行...");

double Qcontrol_SJK = 12000;                  //****三江口安全泄量

double Qcontrol_SJK_low = 9500;              //****各水库调度转换水位以下，三江口流量控制值

double timeLag = 14;                          //江坪河到三江口（石门）滞时

double deltaT = 2;           //流量序列时段长，单位h

Param DM=new Param();

System.out.println("水库参数初始化完毕！");

double plan[][] = plan();

System.out.println("方案组合完毕！");

Map<String, double[][]> floods_Map =floodData();

System.out.println("洪水序列读取完毕！");

int calculationTimes = plan.length * 2 + 1;

String[][] data_statistical = new String[calculationTimes][29];

int count = 1;

data_statistical[0][0] = "年份";

data_statistical[0][1] = "重现期";//

data_statistical[0][2] = "江坪河末水位";//

data_statistical[0][3] = "江垭末水位";//

data_statistical[0][4] = "皂市末水位";//

data_statistical[0][5] = "石门最大流量";//

data_statistical[0][6] = "江坪河起调水位";//；

data_statistical[0][7] = "江坪河转换水位";//；

data_statistical[0][8] = "江垭起调水位";//；

data_statistical[0][9] = "江垭转换水位";//；

data_statistical[0][10] = "皂市起调水位";//；

data_statistical[0][11] = "皂市转换水位";//；

data_statistical[0][12] = "江坪河拦洪量";//

data_statistical[0][13] = "江垭拦洪量";//；

data_statistical[0][14] = "皂市拦洪量";//；

data_statistical[0][15] = "石门洪水削减量";//；

data_statistical[0][16] = "洪量复核";//；

data_statistical[0][17] = "皂市上限水位";//；

data_statistical[0][18] = "调洪后石门最小流量";//；

data_statistical[0][19] = "江坪河最小出库流量";//；

data_statistical[0][20] = "江垭最小出库流量";//；

data_statistical[0][21] = "皂市最小出库流量";//；

data_statistical[0][22] = "江坪河关停时段数";//；

data_statistical[0][23] = "江垭关停时段数";//；

data_statistical[0][24] = "皂市关停时段数";//；

data_statistical[0][25] = "调洪后江垭最小入库流量";

data_statistical[0][26] = "调洪后江垭最小入库流量（秒级）";

data_statistical[0][27] = "调洪后石门最小流量（秒级）";

data_statistical[0][28] = "石门天然洪峰";

System.out.println("开始防洪调度计算...");

for (int i_paln = 0; i_paln < plan.length; i_paln++) {//plan.length

double Hchange_JPH = plan[i_paln][2];

double Hchange_JY = plan[i_paln][3];                   //****江垭水库固定下泄转换水位，210.6

double Hchange_ZS = plan[i_paln][4];

double H_initial_JPH = plan[i_paln][5];  //初始江垭水库水位

double H_initial_JY = plan[i_paln][6];  //初始江垭水库水位

double H_initial_ZS = plan[i_paln][7];//初始皂市水库水位

String strfloodYear = String.valueOf(plan[i_paln][0]);

String floodYear_temp = strfloodYear.substring(0, 4);

String strReturnPreio = String.valueOf(plan[i_paln][1]);

String returnPreio_temp = strReturnPreio.substring(0, 2);

int timeLag_JPH_SM = (int) (timeLag / deltaT); //江坪河到三江口（石门）滞时时段数，7

int timeLag_JPH_JY = timeLag_JPH_SM - 4; //江坪河到三江口（石门）滞时时段数

double QFixed1_JPH = 1400; //江坪河转换水位以上的第一级控泄流量，为1400

double QFixed2_JPH = 1400;//江坪河转换水位以上的第二级控泄流量，最后定位1400

double QFixed2_JY = 1700;   //江垭控制下泄

double QFixed1_JY = 425;

double QFixed_enhace_JY = 425;

double Qgen_ZS = 273.6;//皂市满发流量

double QFixed_enhance_ZS = 273.6;//皂市转换水位以上的第二级最小下泄

if ((floodYear_temp.equals("2016") && returnPreio_temp.equals("50")) || floodYear_temp.equals("2020") && returnPreio_temp.equals("20")) {

QFixed_enhace_JY = 0;

QFixed_enhance_ZS = 0;//皂市

}

if ((floodYear_temp.equals("1954") || floodYear_temp.equals("1964") || floodYear_temp.equals("2020")) && returnPreio_temp.equals("50")) {

QFixed1_JY = 1700;

QFixed_enhace_JY = 1700;

}

double[][] FloodS = floods_Map.get("input_Q_" + floodYear_temp + "_" + returnPreio_temp + ".txt");;           //洪水过程

// 江坪河数据

double[] inflow_JPH = arry.columns(FloodS, 4);                 //该水库入库洪水

double[] preQSeries_JPH_SJK = arry.columns(FloodS, 3);         //三江口（石门）天然洪水

//江垭数据

double[] inflow_JY = arry.columns(FloodS, 1);                            //江垭水库入库洪水

//皂市数据

double[] inflow_ZS = arry.columns(FloodS, 0);                 //该水库入库洪水

double floodPeakFlowOf20year_SJK = 19700; //****三江口二十年一遇洪峰流量19700立方米/s,2023年批复的是19200立方米/s

double[] HupperLimit = {140, 0};//****皂市正常蓄水位140m,

if (returnPreio_temp.equals("dx")) {

if (new matrix().matrixMax(preQSeries_JPH_SJK) > floodPeakFlowOf20year_SJK) {

HupperLimit[0] = 143.5;  //石门流量超20年一遇，则皂市可防洪蓄水至143.5m;

}

} else {

if (returnPreio_temp.equals("50")) {

HupperLimit[0] = 140;

HupperLimit[1] = 143.5;

}

}

for (int i_HupperLimit = 0; i_HupperLimit < HupperLimit.length; i_HupperLimit++) {// HupperLimit.length

if (HupperLimit[i_HupperLimit] != 0) {

boolean flag = true;//三库调洪后三江口（石门）流量依然超12000

int[] index_JPH = new int[inflow_JPH.length];

int[] index_JPH_enhance = new int[inflow_JPH.length];

int[] index_JY = new int[inflow_JPH.length];

int[] index_JY_enhance = new int[inflow_JPH.length];

int[] index_ZS = new int[inflow_JPH.length];

int[] index_ZS_enhance = new int[inflow_JPH.length];

for (int i = 0; i < index_JPH.length; i++) {

index_JPH[i] = -1;

index_JPH_enhance[i] = -1;

index_JY[i] = -1;

index_JY_enhance[i] = -1;

index_ZS[i] = -1;

index_ZS_enhance[i] = -1;

}

//江坪河调洪

double[][] regulationOfFloodControl_JPH = new regulationOfFloodControl_JPH().JPH(inflow_JPH, preQSeries_JPH_SJK, DM.getH_V_JPH(),DM.getH_qXie_JPH_high(), DM.getH_qXie_JPH_low(), H_initial_JPH, deltaT, Hchange_JPH, index_JPH, index_JPH_enhance, timeLag, QFixed1_JPH, QFixed2_JPH, Qcontrol_SJK_low);

double[][] dataout_F_JPH = new dataStatistics().dataout_F(regulationOfFloodControl_JPH, deltaT);

String[] title_JPH = {"江坪河时段初水位", "江坪河时段初蓄水量", "江坪河入库流量过程", "江坪河时段初出库流量", "江坪河时段平均出库流量", "江坪河时段初泄流能力", "江坪河调蓄前三江口（石门）流量", "江坪河调蓄后三江口（石门）流量", "江坪河调蓄后三江口（石门）流量(平均)"};

String[][] dataPrint_JPH = new dataStatistics().data_print(dataout_F_JPH, title_JPH);

//江垭调洪

double[] inflow_JY_mid = new double[inflow_JY.length * (int) (deltaT * 3600)];

double[] preQSeries_JPH_SJK_mid = new double[inflow_JY.length * (int) (deltaT * 3600)];

for (int i = 0; i < inflow_JY_mid.length; i++) {

int j = (int) (i / (deltaT * 3600));

inflow_JY_mid[i] = inflow_JY[j];

preQSeries_JPH_SJK_mid[i] = preQSeries_JPH_SJK[j];

}

for (int i = (int) (timeLag_JPH_JY * deltaT * 3600); i < inflow_JY_mid.length; i++) {

inflow_JY_mid[i] = inflow_JY_mid[i] - (regulationOfFloodControl_JPH[i - (int) (timeLag_JPH_JY * deltaT * 3600)][2] - regulationOfFloodControl_JPH[i - (int) (timeLag_JPH_JY * deltaT * 3600)][3]);

}

double[] preQSeries_JY_SJK = arry.columns(regulationOfFloodControl_JPH, regulationOfFloodControl_JPH[0].length - 2);

double[][] regulationOfFloodControl_JY = new regulationOfFloodControl_second().JY(inflow_JY_mid, preQSeries_JY_SJK, DM.getH_V_JY(), DM.getH_qXie_JY(), H_initial_JY, QFixed2_JY, deltaT, Hchange_JY, index_JY, index_JY_enhance, QFixed1_JY, QFixed_enhace_JY, Qcontrol_SJK_low);

double[][] dataout_F_JY = new dataStatistics().dataout_F(regulationOfFloodControl_JY, deltaT);

String[] title_JY = {"江垭时段初水位", "江垭时段初蓄水量", "江垭入库流量过程", "江垭时段初出库流量", "江垭时段平均出库流量", "江垭时段初泄流能力", "江垭调蓄前三江口（石门）流量", "江垭调蓄后三江口（石门）流量", "江垭调蓄后三江口（石门）流量(平均)"};

String[][] dataPrint_JY = new dataStatistics().data_print(dataout_F_JY, title_JY);

//皂市调洪

double[] preQSeries_ZS_SJK = arry.columns(regulationOfFloodControl_JY, regulationOfFloodControl_JY[0].length - 2);                     //皂市调蓄前三江口（石门）洪水

double[][] regulationOfFloodControl_ZS = new regulationOfFloodControl_second().ZS_buChangAndbuChang(inflow_ZS, preQSeries_ZS_SJK, DM.getH_V_ZS(), DM.getH_qXie_ZS(), H_initial_ZS, Hchange_ZS, deltaT, HupperLimit[i_HupperLimit], index_ZS, index_ZS_enhance, Qgen_ZS, QFixed_enhance_ZS, Qcontrol_SJK_low);

double[][] dataout_F_ZS = new dataStatistics().dataout_F(regulationOfFloodControl_ZS, deltaT);

String[] title_ZS = {"皂市时段初水位", "皂市时段初蓄水量", "皂市入库流量过程", "皂市时段初出库流量", "皂市时段平均出库流量", "皂市时段初泄流能力", "皂市调蓄前三江口（石门）流量", "皂市调蓄后三江口（石门）流量", "皂市调蓄后三江口（石门）流量(平均)"};

String[][] dataPrint_ZS = new dataStatistics().data_print(dataout_F_ZS, title_ZS);

String[][] dataPrint_JPH_JY_ZS = new dataStatistics().dataCombin(dataPrint_JPH, dataPrint_JY, dataPrint_ZS);

int num_JPH = 0;

int num_JY = 0;

int num_ZS = 0;

if (new matrix().matrixMax(arry.columns(dataout_F_ZS, dataout_F_ZS[0].length - 2)) > Qcontrol_SJK) {

////江坪河第二级

for (int i = 0; i < dataout_F_ZS.length; i++) {

if (dataout_F_ZS[i][dataout_F_ZS[0].length - 2] > Qcontrol_SJK) {

if (i >= timeLag_JPH_SM) {

index_JPH[num_JPH] = i - timeLag_JPH_SM;

num_JPH = num_JPH + 1;

}

}

}

//江坪河调洪

regulationOfFloodControl_JPH = new regulationOfFloodControl_JPH().JPH(inflow_JPH, preQSeries_JPH_SJK, DM.getH_V_JPH(),DM.getH_qXie_JPH_high(), DM.getH_qXie_JPH_low(), H_initial_JPH, deltaT, Hchange_JPH, index_JPH, index_JPH_enhance, timeLag, QFixed1_JPH, QFixed2_JPH, Qcontrol_SJK_low);

dataout_F_JPH = new dataStatistics().dataout_F(regulationOfFloodControl_JPH, deltaT);

dataPrint_JPH = new dataStatistics().data_print(dataout_F_JPH, title_JPH);

//江垭调洪

inflow_JY_mid = new double[inflow_JY.length * (int) (deltaT * 3600)];

for (int i = 0; i < inflow_JY_mid.length; i++) {

int j = (int) (i / (deltaT * 3600));

inflow_JY_mid[i] = inflow_JY[j];

preQSeries_JPH_SJK_mid[i] = preQSeries_JPH_SJK[j];

}

for (int i = (int) (timeLag_JPH_JY * deltaT * 3600); i < inflow_JY_mid.length; i++) {

inflow_JY_mid[i] = inflow_JY_mid[i] - (regulationOfFloodControl_JPH[i - (int) (timeLag_JPH_JY * deltaT * 3600)][2] - regulationOfFloodControl_JPH[i - (int) (timeLag_JPH_JY * deltaT * 3600)][3]);

}

preQSeries_JY_SJK = arry.columns(regulationOfFloodControl_JPH, regulationOfFloodControl_JPH[0].length - 2);

regulationOfFloodControl_JY = new regulationOfFloodControl_second().JY(inflow_JY_mid, preQSeries_JY_SJK, DM.getH_V_JY(),DM.getH_qXie_JY(), H_initial_JY, QFixed2_JY, deltaT, Hchange_JY, index_JY, index_JY_enhance, QFixed1_JY, QFixed_enhace_JY, Qcontrol_SJK_low);

dataout_F_JY = new dataStatistics().dataout_F(regulationOfFloodControl_JY, deltaT);

//皂市调洪

preQSeries_ZS_SJK = arry.columns(regulationOfFloodControl_JY, regulationOfFloodControl_JY[0].length - 2);                     //皂市调蓄前三江口（石门）洪水

regulationOfFloodControl_ZS = new regulationOfFloodControl_second().ZS_buChangAndbuChang(inflow_ZS, preQSeries_ZS_SJK, DM.getH_V_ZS(), DM.getH_qXie_ZS(), H_initial_ZS, Hchange_ZS, deltaT, HupperLimit[i_HupperLimit], index_ZS, index_ZS_enhance, Qgen_ZS, QFixed_enhance_ZS, Qcontrol_SJK_low);

dataout_F_ZS = new dataStatistics().dataout_F(regulationOfFloodControl_ZS, deltaT);

dataPrint_ZS = new dataStatistics().data_print(dataout_F_ZS, title_ZS);

////江垭第二级

for (int i = 0; i < dataout_F_ZS.length; i++) {

if (dataout_F_ZS[i][dataout_F_ZS[0].length - 2] > Qcontrol_SJK) {

if (i >= 4) {

index_JY[num_JY] = i - 4;

num_JY = num_JY + 1;

}

}

}

//江垭调洪

inflow_JY_mid = new double[inflow_JY.length * (int) (deltaT * 3600)];

for (int i = 0; i < inflow_JY_mid.length; i++) {

int j = (int) (i / (deltaT * 3600));

inflow_JY_mid[i] = inflow_JY[j];

preQSeries_JPH_SJK_mid[i] = preQSeries_JPH_SJK[j];

}

for (int i = (int) (timeLag_JPH_JY * deltaT * 3600); i < inflow_JY_mid.length; i++) {

inflow_JY_mid[i] = inflow_JY_mid[i] - (regulationOfFloodControl_JPH[i - (int) (timeLag_JPH_JY * deltaT * 3600)][2] - regulationOfFloodControl_JPH[i - (int) (timeLag_JPH_JY * deltaT * 3600)][3]);

}

preQSeries_JY_SJK = arry.columns(regulationOfFloodControl_JPH, regulationOfFloodControl_JPH[0].length - 2);

regulationOfFloodControl_JY = new regulationOfFloodControl_second().JY(inflow_JY_mid, preQSeries_JY_SJK, DM.getH_V_JY(), DM.getH_qXie_JY(), H_initial_JY, QFixed2_JY, deltaT, Hchange_JY, index_JY, index_JY_enhance, QFixed1_JY, QFixed_enhace_JY, Qcontrol_SJK_low);

dataout_F_JY = new dataStatistics().dataout_F(regulationOfFloodControl_JY, deltaT);

dataPrint_JY = new dataStatistics().data_print(dataout_F_JY, title_JY);

//皂市调洪

preQSeries_ZS_SJK = arry.columns(regulationOfFloodControl_JY, regulationOfFloodControl_JY[0].length - 2);                     //皂市调蓄前三江口（石门）洪水

regulationOfFloodControl_ZS = new regulationOfFloodControl_second().ZS_buChangAndbuChang(inflow_ZS, preQSeries_ZS_SJK,  DM.getH_V_ZS(), DM.getH_qXie_ZS(), H_initial_ZS, Hchange_ZS, deltaT, HupperLimit[i_HupperLimit], index_ZS, index_ZS_enhance, Qgen_ZS, QFixed_enhance_ZS, Qcontrol_SJK_low);

dataout_F_ZS = new dataStatistics().dataout_F(regulationOfFloodControl_ZS, deltaT);

dataPrint_ZS = new dataStatistics().data_print(dataout_F_ZS, title_ZS);

////皂市第二级

for (int i = 0; i < dataout_F_ZS.length; i++) {

if (dataout_F_ZS[i][dataout_F_ZS[0].length - 2] > Qcontrol_SJK) {

if (i >= 1) {

index_ZS[num_ZS] = i - 1;

num_ZS = num_ZS + 1;

}

}

}

//皂市调洪

preQSeries_ZS_SJK = arry.columns(regulationOfFloodControl_JY, regulationOfFloodControl_JY[0].length - 2);                     //皂市调蓄前三江口（石门）洪水

regulationOfFloodControl_ZS = new regulationOfFloodControl_second().ZS_buChangAndbuChang(inflow_ZS, preQSeries_ZS_SJK,  DM.getH_V_ZS(), DM.getH_qXie_ZS(), H_initial_ZS, Hchange_ZS, deltaT, HupperLimit[i_HupperLimit], index_ZS, index_ZS_enhance, Qgen_ZS, QFixed_enhance_ZS, Qcontrol_SJK_low);

dataout_F_ZS = new dataStatistics().dataout_F(regulationOfFloodControl_ZS, deltaT);

dataPrint_ZS = new dataStatistics().data_print(dataout_F_ZS, title_ZS);

}

//enhance计算

int num_enhance_JPH = 0;

int num_enhance_JY = 0;

int num_enhance_ZS = 0;

if (new matrix().matrixMax(arry.columns(dataout_F_ZS, dataout_F_ZS[0].length - 2)) > Qcontrol_SJK) {

////江坪河第三级

for (int i = 0; i < dataout_F_ZS.length; i++) {

if (dataout_F_ZS[i][dataout_F_ZS[0].length - 2] > Qcontrol_SJK) {

if (i >= timeLag_JPH_SM) {

index_JPH_enhance[num_enhance_JPH] = i - timeLag_JPH_SM;

num_enhance_JPH = num_enhance_JPH + 1;

}

}

}

//江坪河调洪

regulationOfFloodControl_JPH = new regulationOfFloodControl_JPH().JPH(inflow_JPH, preQSeries_JPH_SJK, DM.getH_V_JPH(),DM.getH_qXie_JPH_high(), DM.getH_qXie_JPH_low(), H_initial_JPH, deltaT, Hchange_JPH, index_JPH, index_JPH_enhance, timeLag, QFixed1_JPH, QFixed2_JPH, Qcontrol_SJK_low);

dataout_F_JPH = new dataStatistics().dataout_F(regulationOfFloodControl_JPH, deltaT);

dataPrint_JPH = new dataStatistics().data_print(dataout_F_JPH, title_JPH);

//江垭调洪

inflow_JY_mid = new double[inflow_JY.length * (int) (deltaT * 3600)];

for (int i = 0; i < inflow_JY_mid.length; i++) {

int j = (int) (i / (deltaT * 3600));

inflow_JY_mid[i] = inflow_JY[j];

preQSeries_JPH_SJK_mid[i] = preQSeries_JPH_SJK[j];

}

for (int i = (int) (timeLag_JPH_JY * deltaT * 3600); i < inflow_JY_mid.length; i++) {

inflow_JY_mid[i] = inflow_JY_mid[i] - (regulationOfFloodControl_JPH[i - (int) (timeLag_JPH_JY * deltaT * 3600)][2] - regulationOfFloodControl_JPH[i - (int) (timeLag_JPH_JY * deltaT * 3600)][3]);

}

preQSeries_JY_SJK = arry.columns(regulationOfFloodControl_JPH, regulationOfFloodControl_JPH[0].length - 2);

regulationOfFloodControl_JY = new regulationOfFloodControl_second().JY(inflow_JY_mid, preQSeries_JY_SJK,DM.getH_V_JY(), DM.getH_qXie_JY(),H_initial_JY, QFixed2_JY, deltaT, Hchange_JY, index_JY, index_JY_enhance, QFixed1_JY, QFixed_enhace_JY, Qcontrol_SJK_low);

dataout_F_JY = new dataStatistics().dataout_F(regulationOfFloodControl_JY, deltaT);

//皂市调洪

preQSeries_ZS_SJK = arry.columns(regulationOfFloodControl_JY, regulationOfFloodControl_JY[0].length - 2);                     //皂市调蓄前三江口（石门）洪水

regulationOfFloodControl_ZS = new regulationOfFloodControl_second().ZS_buChangAndbuChang(inflow_ZS, preQSeries_ZS_SJK,  DM.getH_V_ZS(), DM.getH_qXie_ZS(), H_initial_ZS, Hchange_ZS, deltaT, HupperLimit[i_HupperLimit], index_ZS, index_ZS_enhance, Qgen_ZS, QFixed_enhance_ZS, Qcontrol_SJK_low);

dataout_F_ZS = new dataStatistics().dataout_F(regulationOfFloodControl_ZS, deltaT);

////江垭第三级

for (int i = 0; i < dataout_F_ZS.length; i++) {

if (dataout_F_ZS[i][dataout_F_ZS[0].length - 2] > Qcontrol_SJK) {

if (i >= 4) {

index_JY_enhance[num_enhance_JY] = i - 4;

num_enhance_JY = num_enhance_JY + 1;

}

}

}

//江垭调洪

inflow_JY_mid = new double[inflow_JY.length * (int) (deltaT * 3600)];

for (int i = 0; i < inflow_JY_mid.length; i++) {

int j = (int) (i / (deltaT * 3600));

inflow_JY_mid[i] = inflow_JY[j];

preQSeries_JPH_SJK_mid[i] = preQSeries_JPH_SJK[j];

}

for (int i = (int) (timeLag_JPH_JY * deltaT * 3600); i < inflow_JY_mid.length; i++) {

inflow_JY_mid[i] = inflow_JY_mid[i] - (regulationOfFloodControl_JPH[i - (int) (timeLag_JPH_JY * deltaT * 3600)][2] - regulationOfFloodControl_JPH[i - (int) (timeLag_JPH_JY * deltaT * 3600)][3]);

}

preQSeries_JY_SJK = arry.columns(regulationOfFloodControl_JPH, regulationOfFloodControl_JPH[0].length - 2);

regulationOfFloodControl_JY = new regulationOfFloodControl_second().JY(inflow_JY_mid, preQSeries_JY_SJK, DM.getH_V_JY(), DM.getH_qXie_JY(), H_initial_JY, QFixed2_JY, deltaT, Hchange_JY, index_JY, index_JY_enhance, QFixed1_JY, QFixed_enhace_JY, Qcontrol_SJK_low);

dataout_F_JY = new dataStatistics().dataout_F(regulationOfFloodControl_JY, deltaT);

//皂市调洪

preQSeries_ZS_SJK = arry.columns(regulationOfFloodControl_JY, regulationOfFloodControl_JY[0].length - 2);                     //皂市调蓄前三江口（石门）洪水

regulationOfFloodControl_ZS = new regulationOfFloodControl_second().ZS_buChangAndbuChang(inflow_ZS, preQSeries_ZS_SJK,  DM.getH_V_ZS(), DM.getH_qXie_ZS(), H_initial_ZS, Hchange_ZS, deltaT, HupperLimit[i_HupperLimit], index_ZS, index_ZS_enhance, Qgen_ZS, QFixed_enhance_ZS, Qcontrol_SJK_low);

dataout_F_ZS = new dataStatistics().dataout_F(regulationOfFloodControl_ZS, deltaT);

////皂市第三级

for (int i = 0; i < dataout_F_ZS.length; i++) {

if (dataout_F_ZS[i][dataout_F_ZS[0].length - 2] > Qcontrol_SJK) {

if (i >= 1) {

index_ZS_enhance[num_enhance_ZS] = i - 1;

num_enhance_ZS = num_enhance_ZS + 1;

}

}

}

//皂市调洪

preQSeries_ZS_SJK = arry.columns(regulationOfFloodControl_JY, regulationOfFloodControl_JY[0].length - 2);                     //皂市调蓄前三江口（石门）洪水

regulationOfFloodControl_ZS = new regulationOfFloodControl_second().ZS_buChangAndbuChang(inflow_ZS, preQSeries_ZS_SJK,  DM.getH_V_ZS(), DM.getH_qXie_ZS(), H_initial_ZS, Hchange_ZS, deltaT, HupperLimit[i_HupperLimit], index_ZS, index_ZS_enhance, Qgen_ZS, QFixed_enhance_ZS, Qcontrol_SJK_low);

dataout_F_ZS = new dataStatistics().dataout_F(regulationOfFloodControl_ZS, deltaT);

//                       补个bug:江垭三级调蓄后，可能出现新的需要二级调蓄的时段

for (int i = 0; i < dataout_F_ZS.length; i++) {

if (dataout_F_ZS[i][dataout_F_ZS[0].length - 2] > Qcontrol_SJK) {

if (i >= 4) {

index_JY[num_JY] = i - 4;

num_JY = num_JY + 1;

}

}

}

//江垭调洪

inflow_JY_mid = new double[inflow_JY.length * (int) (deltaT * 3600)];

for (int i = 0; i < inflow_JY_mid.length; i++) {

int j = (int) (i / (deltaT * 3600));

inflow_JY_mid[i] = inflow_JY[j];

preQSeries_JPH_SJK_mid[i] = preQSeries_JPH_SJK[j];

}

for (int i = (int) (timeLag_JPH_JY * deltaT * 3600); i < inflow_JY_mid.length; i++) {

inflow_JY_mid[i] = inflow_JY_mid[i] - (regulationOfFloodControl_JPH[i - (int) (timeLag_JPH_JY * deltaT * 3600)][2] - regulationOfFloodControl_JPH[i - (int) (timeLag_JPH_JY * deltaT * 3600)][3]);

}

preQSeries_JY_SJK = arry.columns(regulationOfFloodControl_JPH, regulationOfFloodControl_JPH[0].length - 2);

regulationOfFloodControl_JY = new regulationOfFloodControl_second().JY(inflow_JY_mid, preQSeries_JY_SJK, DM.getH_V_JY(), DM.getH_qXie_JY(), H_initial_JY, QFixed2_JY, deltaT, Hchange_JY, index_JY, index_JY_enhance, QFixed1_JY, QFixed_enhace_JY, Qcontrol_SJK_low);

dataout_F_JY = new dataStatistics().dataout_F(regulationOfFloodControl_JY, deltaT);

dataPrint_JY = new dataStatistics().data_print(dataout_F_JY, title_JY);

//皂市调洪

preQSeries_ZS_SJK = arry.columns(regulationOfFloodControl_JY, regulationOfFloodControl_JY[0].length - 2);                     //皂市调蓄前三江口（石门）洪水

regulationOfFloodControl_ZS = new regulationOfFloodControl_second().ZS_buChangAndbuChang(inflow_ZS, preQSeries_ZS_SJK,  DM.getH_V_ZS(), DM.getH_qXie_ZS(), H_initial_ZS, Hchange_ZS, deltaT, HupperLimit[i_HupperLimit], index_ZS, index_ZS_enhance, Qgen_ZS, QFixed_enhance_ZS, Qcontrol_SJK_low);

dataout_F_ZS = new dataStatistics().dataout_F(regulationOfFloodControl_ZS, deltaT);

dataPrint_ZS = new dataStatistics().data_print(dataout_F_ZS, title_ZS);

////皂市第三级

for (int i = 0; i < dataout_F_ZS.length; i++) {

if (dataout_F_ZS[i][dataout_F_ZS[0].length - 2] > Qcontrol_SJK) {

if (i >= 1) {

index_ZS_enhance[num_enhance_ZS] = i - 1;

num_enhance_ZS = num_enhance_ZS + 1;

}

}

}

//皂市调洪

preQSeries_ZS_SJK = arry.columns(regulationOfFloodControl_JY, regulationOfFloodControl_JY[0].length - 2);                     //皂市调蓄前三江口（石门）洪水

regulationOfFloodControl_ZS = new regulationOfFloodControl_second().ZS_buChangAndbuChang(inflow_ZS, preQSeries_ZS_SJK,  DM.getH_V_ZS(), DM.getH_qXie_ZS(), H_initial_ZS, Hchange_ZS, deltaT, HupperLimit[i_HupperLimit], index_ZS, index_ZS_enhance, Qgen_ZS, QFixed_enhance_ZS, Qcontrol_SJK_low);

dataout_F_ZS = new dataStatistics().dataout_F(regulationOfFloodControl_ZS, deltaT);

dataPrint_ZS = new dataStatistics().data_print(dataout_F_ZS, title_ZS);

}

dataPrint_JPH_JY_ZS = new dataStatistics().dataCombin(dataPrint_JPH, dataPrint_JY, dataPrint_ZS);

File writePath_JPH_JY_ZS = new File("./output/regulationProcess/" + "江坪河" + String.valueOf(H_initial_JPH) + "起调_江坪河转换水位" + Hchange_JPH + "_江垭" + String.valueOf(H_initial_JY) + "起调_江垭转换水位" + Hchange_JY + "_皂市" + String.valueOf(H_initial_ZS) + "起调_皂市转换水位" + Hchange_ZS + "_皂市上限水位" + HupperLimit[i_HupperLimit] + "_" + floodYear_temp + "年_" + returnPreio_temp + "年一遇.txt");

//System.out.println(writePath_JPH_JY_ZS);

new writeTxt().writeFile3(writePath_JPH_JY_ZS, dataPrint_JPH_JY_ZS);        //输出txt

data_statistical[count][0] = floodYear_temp; //年份

data_statistical[count][1] = returnPreio_temp;//重现期

data_statistical[count][2] = String.valueOf(dataout_F_JPH[dataout_F_JPH.length - 1][0]);//江坪河末水位

data_statistical[count][3] = String.valueOf(dataout_F_JY[dataout_F_JY.length - 1][0]);//江垭末水位

data_statistical[count][4] = String.valueOf(dataout_F_ZS[dataout_F_ZS.length - 1][0]);//皂市末水位

data_statistical[count][5] = String.valueOf(new matrix().matrixMax(arry.columns(dataout_F_ZS, dataout_F_ZS[0].length - 2)));//石门最大流量

data_statistical[count][6] = String.valueOf(H_initial_JPH);//江坪河起调水位；

data_statistical[count][7] = String.valueOf(Hchange_JPH);//江坪河转换水位；

data_statistical[count][8] = String.valueOf(H_initial_JY);//江垭起调水位；

data_statistical[count][9] = String.valueOf(Hchange_JY);//江垭转换水位；

data_statistical[count][10] = String.valueOf(H_initial_ZS);//皂市起调水位；

data_statistical[count][11] = String.valueOf(Hchange_ZS);//皂市转换水位；

data_statistical[count][12] = String.valueOf(regulationOfFloodControl_JPH[regulationOfFloodControl_JPH.length - 1][1] - regulationOfFloodControl_JPH[0][1]);//江坪河拦洪量

data_statistical[count][13] = String.valueOf(regulationOfFloodControl_JY[regulationOfFloodControl_ZS.length - 1][1] - regulationOfFloodControl_JY[0][1]);//江垭拦洪量；

data_statistical[count][14] = String.valueOf(regulationOfFloodControl_ZS[regulationOfFloodControl_JY.length - 1][1] - regulationOfFloodControl_ZS[0][1]);//皂市拦洪量；

double minus_Q = 0;

for (int i = 0; i < regulationOfFloodControl_ZS.length; i++) {

minus_Q = minus_Q + preQSeries_JPH_SJK_mid[i] - regulationOfFloodControl_ZS[i][regulationOfFloodControl_ZS[0].length - 1];

}

minus_Q = minus_Q / Math.pow(10, 8);

data_statistical[count][15] = String.valueOf(minus_Q);//石门洪水削减量；

data_statistical[count][16] = String.valueOf(Double.valueOf(data_statistical[count][12]) + Double.valueOf(data_statistical[count][13]) + Double.valueOf(data_statistical[count][14]) - Double.valueOf(data_statistical[count][15]));//洪量复核；

data_statistical[count][17] = String.valueOf(HupperLimit[i_HupperLimit]);//皂市上限水位；

data_statistical[count][18] = String.valueOf(new matrix().matrixMin(arry.columns(dataout_F_ZS, dataout_F_ZS[0].length - 2)));//调洪后石门最小流量

data_statistical[count][19] = String.valueOf(new matrix().matrixMin(arry.columns(dataout_F_JPH, 3)));//江坪河最小出库流量

data_statistical[count][20] = String.valueOf(new matrix().matrixMin(arry.columns(dataout_F_JY, 3)));//江垭最小出库流量

data_statistical[count][21] = String.valueOf(new matrix().matrixMin(arry.columns(dataout_F_ZS, 3)));//皂市最小出库流量

int count_zeros_JPH = 0;//江垭关停的时段数

for (int i = 0; i < dataout_F_JPH.length; i++) {

if (dataout_F_JPH[i][3] == 0) {

count_zeros_JPH = count_zeros_JPH + 1;

}

}

int count_zeros_JY = 0;//江垭关停的时段数

for (int i = 0; i < dataout_F_JY.length; i++) {

if (dataout_F_JY[i][3] == 0) {

count_zeros_JY = count_zeros_JY + 1;

}

}

int count_zeros_ZS = 0;//江垭关停的时段数

for (int i = 0; i < dataout_F_ZS.length; i++) {

if (dataout_F_ZS[i][3] == 0) {

count_zeros_ZS = count_zeros_ZS + 1;

}

}

data_statistical[count][22] = String.valueOf(count_zeros_JPH);

data_statistical[count][23] = String.valueOf(count_zeros_JY);

data_statistical[count][24] = String.valueOf(count_zeros_ZS);

data_statistical[count][25] = String.valueOf(new matrix().matrixMin(arry.columns(dataout_F_JY, 2)));//调洪后江垭最小入库流量

data_statistical[count][26] = String.valueOf(new matrix().matrixMin(arry.columns(regulationOfFloodControl_JY, 2)));//调洪后江垭最小入库流量（秒级）

data_statistical[count][27] = String.valueOf(new matrix().matrixMin(arry.columns(regulationOfFloodControl_ZS, 8)));//调洪石门流量（秒级）

data_statistical[count][28] = String.valueOf(new matrix().matrixMax(preQSeries_JPH_SJK));//石门天然洪峰

if(count%2==0) {

DecimalFormat df = new DecimalFormat("#.00");

System.out.println("计算进度：" + df.format(count  * 100.0 / (plan.length * 1.5)) + "%");

}

count = count + 1;

//System.out.println("count:"+count);

}

}

}

File writePath_JPH_JY_ZS = new File("./Output/"  + "result.txt");

new writeTxt().writeFile3(writePath_JPH_JY_ZS, data_statistical);        //输出txt

System.out.println("计算完毕,计算结果已导出！");

}

}

package regulationOfFloodControl.function;

import tools.readTxt_F;

import java.util.HashMap;

import java.util.Map;

import java.io.File;

public class floods {

public static Map<String, double[][]> floodData() {

File folder = new File("./input/floods/");

File[] files = folder.listFiles((dir, name) -> name.toLowerCase().endsWith(".txt"));

Map<String, double[][]> floods_Map = new HashMap<>();//洪水数据

for (File file : files) {

double [][]floodData= readTxt_F.file2doubleArray(file, 1);           //洪水过程

floods_Map.put(file.getName(),floodData);

}

return floods_Map;

}

}

package regulationOfFloodControl.function;

import lombok.Data;

import tools.readTxt_F;

import java.io.File;

@Data

public class Param {

//江坪河数据

private String[] reservoirNames = {"zaoShi", "jiangYa", "YiChongQiao", "JiangPingHe"};

private String reservoirName_JPH = reservoirNames[3];

private String currentDir = System.getProperty("user.dir");;

private File fileOfH_V_JPH = new File("./input/ReservoirParameters/"+"H-V-" + reservoirName_JPH + ".txt");

private double[][] H_V_JPH = readTxt_F.file2doubleArray(fileOfH_V_JPH, 1);           //水位库容曲线

private File fileOfH_qXie_JPH_high = new File("./input/ReservoirParameters/"+ "H-X1-" + reservoirName_JPH + "-high.txt");

private double[][] H_qXie_JPH_high = readTxt_F.file2doubleArray(fileOfH_qXie_JPH_high, 1);      //水库水位~下泄流量曲线

private File fileOfH_qXie_JPH_low = new File("./input/ReservoirParameters/"+ "H-X1-" + reservoirName_JPH + "-low.txt");

private double[][] H_qXie_JPH_low = readTxt_F.file2doubleArray(fileOfH_qXie_JPH_low, 1);      //水库水位~下泄流量曲线

//江垭数据

private String reservoirName_JY = reservoirNames[1];

private File fileOfH_V_JY = new File("./input/ReservoirParameters/" +"H-V-" + reservoirName_JY + ".txt");

private double[][] H_V_JY = readTxt_F.file2doubleArray(fileOfH_V_JY, 1);           //水位库容曲线

private File fileOfH_qXie_JY = new File("./input/ReservoirParameters/"+ "H-X1-" + reservoirName_JY + ".txt");

private double[][] H_qXie_JY = readTxt_F.file2doubleArray(fileOfH_qXie_JY, 1);      //水库水位~下泄流量曲线

//皂市数据

private String reservoirName_ZS = reservoirNames[0];

private File fileOfH_V_ZS = new File("./input/ReservoirParameters/"+ "H-V-" + reservoirName_ZS + ".txt");

private double[][] H_V_ZS = readTxt_F.file2doubleArray(fileOfH_V_ZS, 1);           //水位库容曲线

private  File fileOfH_qXie_ZS = new File("./input/ReservoirParameters/"+ "H-X1-" + reservoirName_ZS + ".txt");

private double[][] H_qXie_ZS = readTxt_F.file2doubleArray(fileOfH_qXie_ZS, 1);      //水库水位~下泄流量曲线

}

package regulationOfFloodControl.function;

public class plans {

public static double[][] plan() {

double[] years = {1954, 1964, 1980, 1983, 1991, 1998, 2016, 2020};

double[] returnyear = {20, 50};

double[] H_JPH_initial = {459.7};

double[] H_JY_initial = {210.6};

double[] H_ZS_initial = {125};

double[] H_JPH_change = {462};

double[] H_JY_change = {217};

double[] H_ZS_change = {129};

double[][] plan = new double[years.length * returnyear.length * H_JPH_initial.length * H_JPH_change.length][8];

int count = 0;

for (int i1 = 0; i1 < years.length; i1++) {

for (int i2 = 0; i2 < returnyear.length; i2++) {

for (int i3 = 0; i3 < H_JPH_initial.length; i3++) {

for (int i4 = 0; i4 < H_JPH_change.length; i4++) {

plan[count][0] = years[i1];

plan[count][1] = returnyear[i2];

plan[count][2] = H_JPH_initial[i3];

plan[count][3] = H_JY_initial[i3];

plan[count][4] = H_ZS_initial[i3];

plan[count][5] = H_JPH_change[i4];

plan[count][6] = H_JY_change[i4];

plan[count][7] = H_ZS_change[i4];

count=count+1;

}

}

}

}

return plan;

}

}

package regulationOfFloodControl;

import tools.LinearInterpolation;

import tools.arry;

import java.io.File;

public class regulationOfFloodControl_JPH {

/*

江坪河防洪调度--参照江垭

inflow_JPH            //****江坪河入库流量(时间尺度：2小时)

preQSeries_JPH_SJK    //****江坪河调蓄前三江口流量(时间尺度：2小时)

H_V_JPH               //****江坪河水位库容曲线

H_qXie_JPH            //****江坪河泄流能力曲线

H_initial_JPH         //****江坪河初始水位（即江垭起调水位）

deltaT               //洪水资料的精度，2小时

Hchange_JPH           //江垭由控泄420转换为控泄1700的转换水位     */

public static double[][] JPH(double[] inflow_JPH, double[] preQSeries_JPH_SJK, double[][] H_V_JPH, double[][] H_qXie_JPH_high, double[][] H_qXie_JPH_low, double H_initial_JPH, double deltaT, double Hchange_JPH, int[] index_JPH, int[] index_JPH_enhance, double timeLag, double QFixed1_JPH, double QFixed2_JPH, double Qcontrol_SJK_low) {

//输出数据

double[] postFlow_JPH_SJK = new double[(int) (preQSeries_JPH_SJK.length * deltaT * 3600)];            //****江垭调蓄后的三江口流量过程

double[] outFlow_JPH = new double[postFlow_JPH_SJK.length];                 //****江垭出库流量过程,时段下泄流量

double[] H_JPH = new double[postFlow_JPH_SJK.length];                       //****江垭水位过程

double[] storageWater_JPH = new double[postFlow_JPH_SJK.length];            //****江垭蓄水量变化过程

double[] qxie_JPH = new double[postFlow_JPH_SJK.length];                    //****江垭泄流能力变化过程

double[] preQSeries_JPH_SJK_mid = new double[postFlow_JPH_SJK.length];

double[] inflow_JPH_mid = new double[postFlow_JPH_SJK.length];

double[][] dataOut = new double[postFlow_JPH_SJK.length][9];               //****中间输出数据

//参数

double Qcontrol_SJK = 12000;              //****三江口安全泄量

//System.out.println("江坪河起调水位" + H_initial_JPH);

//double QFixed1_JPH = 1400; //1400                    //****江坪河满发流量332立方米/s

double floodPeaksOf1000years = 9370;          //江坪河大坝设计标准1000年一遇洪水

double H0_normalWaterLevel_JPH = 470;      //****江坪河正常蓄水位470m,当水库水库达到470m,水库不在承担下游防洪任务，按照入库流量下泄

double V_normalWaterLevel_JPH = new LinearInterpolation().LinearInterpolation(H0_normalWaterLevel_JPH, arry.columns(H_V_JPH, 0), arry.columns(H_V_JPH, 1), 2);//****江坪河正常蓄水位470m对应的蓄水量

double V_change_JPH = new LinearInterpolation().LinearInterpolation(Hchange_JPH, arry.columns(H_V_JPH, 0), arry.columns(H_V_JPH, 1), 2);//****江坪河转换水位对应的蓄水量

double floodPeaksOf5years = 3470;             //江坪河大坝5年一遇洪水

double floodPeaksOf20years = 5100;             //江坪河大坝20年一遇洪水

//double QFixed2_JPH = 1400;                      //江坪河转换水位以上的固定下泄流量

//double timeLag = 14;                          //江坪河到三江口（石门）滞时

int numberOfLagTimePeriods = (int) (timeLag / deltaT); //江坪河到三江口（石门）滞时时段数

double QFixed_enhacne = 1400;//江坪河不放水？放332江坪河最低是否关闸**********************************************

//double deltaT = 2;                       //****2小时洪水资料

double Q_manfa = 332;//江坪河转换水位以下的控泄流量

//计算过程

H_JPH[0] = H_initial_JPH;

for (int i = 0; i < deltaT * numberOfLagTimePeriods * 3600; i++) {

int j = (int) (i / (deltaT * 3600));

postFlow_JPH_SJK[i] = preQSeries_JPH_SJK[j];

}

for (int i = 0; i < postFlow_JPH_SJK.length; i++) {

int j = (int) (i / (deltaT * 3600));

preQSeries_JPH_SJK_mid[i] = preQSeries_JPH_SJK[j];

inflow_JPH_mid[i] = inflow_JPH[j];

}

boolean flag = false;

boolean falg_Hchange = false;

for (int i = 0; i < postFlow_JPH_SJK.length - deltaT * 3600 * numberOfLagTimePeriods; i++) {

int j = (int) (i / (deltaT * 3600));

if (inflow_JPH_mid[i] <= floodPeaksOf1000years) {

qxie_JPH[i] = new LinearInterpolation().LinearInterpolation(H_JPH[i], arry.columns(H_qXie_JPH_low, 0), arry.columns(H_qXie_JPH_low, 1), 2);

} else {

qxie_JPH[i] = new LinearInterpolation().LinearInterpolation(H_JPH[i], arry.columns(H_qXie_JPH_high, 0), arry.columns(H_qXie_JPH_high, 1), 2);

}

storageWater_JPH[i] = new LinearInterpolation().LinearInterpolation(H_JPH[i], arry.columns(H_V_JPH, 0), arry.columns(H_V_JPH, 1), 2);

if ((preQSeries_JPH_SJK[j + numberOfLagTimePeriods] <= Qcontrol_SJK) || storageWater_JPH[i] >= V_normalWaterLevel_JPH || flag) {

if (inflow_JPH[j] <= qxie_JPH[i]) {

outFlow_JPH[i] = inflow_JPH[j];

} else {

outFlow_JPH[i] = qxie_JPH[i];

}

if (H_JPH[i] < Hchange_JPH && (preQSeries_JPH_SJK[j + numberOfLagTimePeriods] > Qcontrol_SJK_low) && !falg_Hchange) {

if (Q_manfa > inflow_JPH[j]) {

outFlow_JPH[i] = inflow_JPH[j];

} else {

outFlow_JPH[i] = Q_manfa;

}

if (outFlow_JPH[i] > qxie_JPH[i]) {

outFlow_JPH[i] = qxie_JPH[i];

}

storageWater_JPH[i + 1] = (inflow_JPH_mid[i] - outFlow_JPH[i]) * 1 / (Math.pow(10, 8)) + storageWater_JPH[i];

if (storageWater_JPH[i + 1] > V_change_JPH) {//

if (inflow_JPH[j] <= qxie_JPH[i]) {

outFlow_JPH[i] = inflow_JPH[j];

} else {

outFlow_JPH[i] = qxie_JPH[i];

}

falg_Hchange = true;

}

}

} else {

if (H_JPH[i] < Hchange_JPH && !falg_Hchange) {

if (preQSeries_JPH_SJK[j + numberOfLagTimePeriods] > Qcontrol_SJK_low) {

if (Q_manfa > inflow_JPH[j]) {

outFlow_JPH[i] = inflow_JPH[j];

} else {

outFlow_JPH[i] = Q_manfa;

}

} else {

outFlow_JPH[i] = inflow_JPH[j];

}

if (outFlow_JPH[i] > qxie_JPH[i]) {

outFlow_JPH[i] = qxie_JPH[i];

}

storageWater_JPH[i + 1] = (inflow_JPH_mid[i] - outFlow_JPH[i]) * 1 / (Math.pow(10, 8)) + storageWater_JPH[i];

if (storageWater_JPH[i + 1] > V_change_JPH) {//

if (inflow_JPH[j] <= qxie_JPH[i]) {

outFlow_JPH[i] = inflow_JPH[j];

} else {

outFlow_JPH[i] = qxie_JPH[i];

}

falg_Hchange = true;

}

} else {

if (floodPeaksOf5years < inflow_JPH_mid[i]) {

boolean flag_index_JPH = false;

double QFixedTemp_JPH = 0;

for (int i1 = 0; i1 < index_JPH.length; i1++) {

if (index_JPH[i1] * deltaT * 3600 <= i && i < (index_JPH[i1] + 1) * deltaT * 3600) {

flag_index_JPH = true;

}

}

if (flag_index_JPH) {

QFixedTemp_JPH = QFixed1_JPH;

} else {

QFixedTemp_JPH = QFixed2_JPH;

}

boolean flag_index_enhance_JPH = false;

for (int i1 = 0; i1 < index_JPH_enhance.length; i1++) {

if (index_JPH_enhance[i1] * deltaT * 3600 <= i && i < (index_JPH_enhance[i1] + 1) * deltaT * 3600) {

flag_index_enhance_JPH = true;

}

}

if (flag_index_enhance_JPH) {

QFixedTemp_JPH = QFixed_enhacne;//在转换水位以上且石门流量超过12000，此时江坪河是否

}

outFlow_JPH[i] = QFixedTemp_JPH;

if (QFixedTemp_JPH > inflow_JPH_mid[i]) {

outFlow_JPH[i] = inflow_JPH_mid[i];

}

} else {

outFlow_JPH[i] = inflow_JPH_mid[i];

}

if (outFlow_JPH[i] > qxie_JPH[i]) {

outFlow_JPH[i] = qxie_JPH[i];

}

storageWater_JPH[i + 1] = (inflow_JPH_mid[i] - outFlow_JPH[i]) * 1 / (Math.pow(10, 8)) + storageWater_JPH[i];

if (storageWater_JPH[i + 1] > V_normalWaterLevel_JPH) {//

//System.out.println("第" + i + "时刻后,江坪河达到正常蓄水位470m");

if (inflow_JPH[j] <= qxie_JPH[i]) {

outFlow_JPH[i] = inflow_JPH[j];

} else {

outFlow_JPH[i] = qxie_JPH[i];

}

flag = true;

}

}

}

storageWater_JPH[i + 1] = (inflow_JPH_mid[i] - outFlow_JPH[i]) * 1 / (Math.pow(10, 8)) + storageWater_JPH[i];

H_JPH[i + 1] = new LinearInterpolation().LinearInterpolation(storageWater_JPH[i + 1], arry.columns(H_V_JPH, 0), arry.columns(H_V_JPH, 1), 1);

postFlow_JPH_SJK[i + (int) (numberOfLagTimePeriods * deltaT * 3600)] = preQSeries_JPH_SJK_mid[i + (int) (numberOfLagTimePeriods * deltaT * 3600)] - (inflow_JPH_mid[i] - outFlow_JPH[i]);

}

for (int i = (int) (postFlow_JPH_SJK.length - deltaT * 3600 * numberOfLagTimePeriods); i < postFlow_JPH_SJK.length; i++) {

if (inflow_JPH_mid[i] <= floodPeaksOf1000years) {

qxie_JPH[i] = new LinearInterpolation().LinearInterpolation(H_JPH[i], arry.columns(H_qXie_JPH_low, 0), arry.columns(H_qXie_JPH_low, 1), 2);

} else {

qxie_JPH[i] = new LinearInterpolation().LinearInterpolation(H_JPH[i], arry.columns(H_qXie_JPH_high, 0), arry.columns(H_qXie_JPH_high, 1), 2);

}

outFlow_JPH[i] = Math.min(inflow_JPH_mid[i], qxie_JPH[i]);

if (i + 1 < postFlow_JPH_SJK.length) {

storageWater_JPH[i + 1] = (inflow_JPH_mid[i] - outFlow_JPH[i]) * 1 / (Math.pow(10, 8)) + storageWater_JPH[i];

H_JPH[i + 1] = new LinearInterpolation().LinearInterpolation(storageWater_JPH[i + 1], arry.columns(H_V_JPH, 0), arry.columns(H_V_JPH, 1), 1);

}

}

for (int i = 0; i < dataOut.length; i++) {

dataOut[i][0] = H_JPH[i];                     //****江坪河时段初水位

dataOut[i][1] = storageWater_JPH[i];          //****江坪河时段初蓄水量

dataOut[i][2] = inflow_JPH_mid[i];           //****江坪河入库流量过程

dataOut[i][3] = outFlow_JPH[i];              //****江坪河时段初出库流量 (瞬时值)

dataOut[i][4] = outFlow_JPH[i];              //****江坪河出库流量过程(平均值)

dataOut[i][5] = qxie_JPH[i];                 //****江坪河泄流能力变化过程

dataOut[i][6] = preQSeries_JPH_SJK_mid[i];   //****江坪河调蓄前三江口（石门）流量过程

dataOut[i][7] = postFlow_JPH_SJK[i];         //****江坪河调蓄后三江口（石门）流量过程

dataOut[i][8] = postFlow_JPH_SJK[i];         //****江坪河调蓄后三江口（石门）流量过程（平均）

}

String[] title = {"江坪河时段初水位", "江坪河时段初蓄水量", "江坪河入库流量过程", "江坪河时段初出库流量", "江坪河时段平均出库流量", "江坪河时段初泄流能力", "江江坪河调蓄前三江口（石门）流量", "江坪河调蓄后三江口（石门）流量", "江坪河调蓄后三江口（石门）流量(平均)"};

String[][] dataOut_JPH_mid = new String[dataOut.length + 1][dataOut[0].length];

for (int i = 0; i < dataOut_JPH_mid.length; i++) {

for (int i4 = 0; i4 < dataOut_JPH_mid[0].length; i4++) {

if (i == 0) {

dataOut_JPH_mid[i][i4] = title[i4];

} else {

dataOut_JPH_mid[i][i4] = String.valueOf(dataOut[i - 1][i4]);

}

}

}

return dataOut;

}

}

package regulationOfFloodControl;

import tools.arry;

import tools.LinearInterpolation;

import java.io.File;

public class regulationOfFloodControl_second {

//皂市转换水位上下都按照补偿

/*

double[]inflow_ZS：         ****皂市入库流量过程

double[] SJK_tianRan：      ****三江口（石门）天然流量过程

double []preQSeries_ZS_SJK_mid：****皂市调蓄前三江口（石门）流量过程(时间尺度：秒)

double[][] H_V_ZS：         ****皂市水位库容曲线

double[][] H_qXie_ZS：      ****皂市水库水位~下泄流量曲线

double[] initialQSeries_SJK ****三江口（石门）天然流量过程

double H_initial_ZS：       ****皂市初始水库水位

double Hchange_ZS           ****皂市补偿转换水位(复核)*?130

double Q_changchang1        *****皂市转换水位以下按石门多少流量进行补偿，例如9000,7000,10000

*/

public static double[][] ZS_buChangAndbuChang(double[] inflow_ZS, double[] preQSeries_ZS_SJK_mid, double[][] H_V_ZS, double[][] H_qXie_ZS, double H_initial_ZS, double Hchange_ZS, double deltaT, double HupperLimit, int[] index_ZS, int[] index_ZS_enhance, double Qgen_ZS, double QFixed_enhance, double Qcontrol_SJK_low) {

//输出数据

double[] postFlow_ZS_SJK = new double[(int) (inflow_ZS.length * deltaT * 3600)];            //****皂市调蓄后的三江口流量过程

double[] inFlow_ZS_mid = new double[postFlow_ZS_SJK.length];           //****皂市时段平均入库流量

double[] outFlow_ZS = new double[postFlow_ZS_SJK.length];                 //****皂市出库流量过程,时段下泄流量

double[] H_ZS = new double[postFlow_ZS_SJK.length];                       //****皂市水位过程

double[] storageWater_ZS = new double[postFlow_ZS_SJK.length];            //****皂市蓄水量变化过程

double[] qxie_ZS = new double[postFlow_ZS_SJK.length];                    //****皂市泄流能力变化过程

double[][] dataOut = new double[postFlow_ZS_SJK.length][9];               //****中间输出数据

double Qmanfa_ZS = 273.6;//皂市满发流量

//参数

double Qcontrol_SJK = 12000;              //****三江口安全泄量

//double floodPeakFlowOf20year_SJK = 19700; //****三江口二十年一遇洪峰流量19700立方米/s,2023年批复的是19200立方米/s

//double Qgen_ZS = 273.6;                   //****皂市发电能力,石门补偿控制不住，则皂市满发或不下泄流量

//double QFixed_enhance= 273.6;               //****皂市最低按照满发控制273.6，或最低按照关停控制

//double H0_fc_ZS = 132;                    //****防洪限制水位或起调水位(复核)*?130

//double Hchange_ZS = 132;                  //****补偿转换水位(复核)*?130

//double Hturn_ZS = 143.5;                  //****防洪调度转换水位，防洪高水位143.5m/

//double Qcontrol_ZS = 3200;                //****固定泄量防洪调度时下游安全控制泄量，当起调水位等于转换水位时改为Qchange_ZS****(复核)/*? 2820

//double Qchange_ZS = 999999;                 //****补偿防洪调度时下游安全控制泄量****(复核)/* ? 3200

//double H0_normalWaterLevel_ZS = 140;      //****皂市正常蓄水位140m,防洪调度转换水位

double QFixed_ZS = 1300;                  //****皂市固定下泄流量1300立方米/s

//double deltaT = 2;                          //****2小时洪水资料

//计算过程

H_ZS[0] = H_initial_ZS;

double V_HupperLimit_ZS = new LinearInterpolation().LinearInterpolation(HupperLimit, arry.columns(H_V_ZS, 0), arry.columns(H_V_ZS, 1), 2);

double V_change_ZS = new LinearInterpolation().LinearInterpolation(Hchange_ZS, arry.columns(H_V_ZS, 0), arry.columns(H_V_ZS, 1), 2);

for (int i = 0; i < postFlow_ZS_SJK.length; i++) {

int j = (int) (i / (deltaT * 3600));

inFlow_ZS_mid[i] = inflow_ZS[j];

}

boolean flag = false;             //皂市水位未超过上限水位

boolean flag_Hchange = false;             //皂市水位未超过转换水位

for (int i = 0; i < preQSeries_ZS_SJK_mid.length - (int) (1 * deltaT * 3600); i++) {//preQSeries_ZS_SJK.length

//int j = (int) (i / (deltaT * 3600));

storageWater_ZS[i] = new LinearInterpolation().LinearInterpolation(H_ZS[i], arry.columns(H_V_ZS, 0), arry.columns(H_V_ZS, 1), 2);

qxie_ZS[i] = new LinearInterpolation().LinearInterpolation(H_ZS[i], arry.columns(H_qXie_ZS, 0), arry.columns(H_qXie_ZS, 1), 2);      //****时段初水库泄流能力

if (preQSeries_ZS_SJK_mid[i + (int) (1 * deltaT * 3600)] <= Qcontrol_SJK || storageWater_ZS[i] > V_HupperLimit_ZS || flag) {

if (inFlow_ZS_mid[i] <= qxie_ZS[i]) {//来多少放多少

outFlow_ZS[i] = inFlow_ZS_mid[i];

} else {

outFlow_ZS[i] = qxie_ZS[i];

}

if (H_ZS[i] < Hchange_ZS && preQSeries_ZS_SJK_mid[i + (int) (1 * deltaT * 3600)] > Qcontrol_SJK_low && !flag_Hchange) {//转换水位以下,且,三江口流量大于各水库调度转换水位以下三江口流量控制值

//补偿调度

double Qtemp = inFlow_ZS_mid[i] - (preQSeries_ZS_SJK_mid[i + (int) (1 * deltaT * 3600)] - Qcontrol_SJK_low);

if (Qtemp < 0) {

outFlow_ZS[i] = Math.min(Qmanfa_ZS, inFlow_ZS_mid[i]);

} else {

if (Qtemp > inFlow_ZS_mid[i]) {

outFlow_ZS[i] = inFlow_ZS_mid[i];

} else {

outFlow_ZS[i] = Qtemp;

if (outFlow_ZS[i] < Qmanfa_ZS) {

outFlow_ZS[i] = Qmanfa_ZS;

}

}

}

boolean flag_index_ZS_enhance = false;

for (int i1 = 0; i1 < index_ZS_enhance.length; i1++) {

if (index_ZS_enhance[i1] * deltaT * 3600 <= i && i < (index_ZS_enhance[i1] + 1) * deltaT * 3600) {

flag_index_ZS_enhance = true;

}

}

if (flag_index_ZS_enhance) {

if (Qtemp < 0) {

outFlow_ZS[i] = Math.min(QFixed_enhance, inFlow_ZS_mid[i]);

} else {

if (Qtemp > QFixed_enhance) {

outFlow_ZS[i] = Math.min(Qtemp, inFlow_ZS_mid[i]);

} else {

outFlow_ZS[i] = Math.min(QFixed_enhance, inFlow_ZS_mid[i]);

}

}

}

if (outFlow_ZS[i] > qxie_ZS[i]) {

outFlow_ZS[i] = qxie_ZS[i];

}

storageWater_ZS[i + 1] = storageWater_ZS[i] + (inFlow_ZS_mid[i] - outFlow_ZS[i]) * 1 / Math.pow(10, 8);

if (storageWater_ZS[i + 1] > V_change_ZS) {

if (inFlow_ZS_mid[i] <= qxie_ZS[i]) {

outFlow_ZS[i] = inFlow_ZS_mid[i];

} else {

outFlow_ZS[i] = qxie_ZS[i];

}

flag_Hchange = true;

}

}

} else {

if (H_ZS[i] < Hchange_ZS && !flag_Hchange) {

//补偿调度

double Qtemp = inFlow_ZS_mid[i] - (preQSeries_ZS_SJK_mid[i + (int) (1 * deltaT * 3600)] - Qcontrol_SJK_low);

if (Qtemp < 0) {

outFlow_ZS[i] = Math.min(Qmanfa_ZS, inFlow_ZS_mid[i]);

} else {

if (Qtemp > inFlow_ZS_mid[i]) {

outFlow_ZS[i] = inFlow_ZS_mid[i];

} else {

outFlow_ZS[i] = Qtemp;

if (outFlow_ZS[i] < Qmanfa_ZS) {

outFlow_ZS[i] = Qmanfa_ZS;

}

}

}

boolean flag_index_ZS_enhance = false;

for (int i1 = 0; i1 < index_ZS_enhance.length; i1++) {

if (index_ZS_enhance[i1] * deltaT * 3600 <= i && i < (index_ZS_enhance[i1] + 1) * deltaT * 3600) {

flag_index_ZS_enhance = true;

}

}

if (flag_index_ZS_enhance) {

if (Qtemp < 0) {

outFlow_ZS[i] = Math.min(QFixed_enhance, inFlow_ZS_mid[i]);

} else {

if (Qtemp > QFixed_enhance) {

outFlow_ZS[i] = Math.min(Qtemp, inFlow_ZS_mid[i]);

} else {

outFlow_ZS[i] = Math.min(QFixed_enhance, inFlow_ZS_mid[i]);

}

}

}

if (outFlow_ZS[i] > qxie_ZS[i]) {

outFlow_ZS[i] = qxie_ZS[i];

}

storageWater_ZS[i + 1] = storageWater_ZS[i] + (inFlow_ZS_mid[i] - outFlow_ZS[i]) * 1 / Math.pow(10, 8);

if (storageWater_ZS[i + 1] > V_HupperLimit_ZS) {

if (inFlow_ZS_mid[i] <= qxie_ZS[i]) {

outFlow_ZS[i] = inFlow_ZS_mid[i];

} else {

outFlow_ZS[i] = qxie_ZS[i];

}

flag = true;

}

} else {

//补偿调度

double Qtemp = inFlow_ZS_mid[i] - (preQSeries_ZS_SJK_mid[i + (int) (1 * deltaT * 3600)] - Qcontrol_SJK);

if (Qtemp < 0) {

outFlow_ZS[i] = Math.min(Qgen_ZS, inFlow_ZS_mid[i]);

} else {

if (Qtemp > inFlow_ZS_mid[i]) {

outFlow_ZS[i] = inFlow_ZS_mid[i];

} else {

outFlow_ZS[i] = Qtemp;

if (outFlow_ZS[i] < Qgen_ZS) {

outFlow_ZS[i] = Qgen_ZS;

}

}

}

boolean flag_index_ZS_enhance = false;

for (int i1 = 0; i1 < index_ZS_enhance.length; i1++) {

if (index_ZS_enhance[i1] * deltaT * 3600 <= i && i < (index_ZS_enhance[i1] + 1) * deltaT * 3600) {

flag_index_ZS_enhance = true;

}

}

if (flag_index_ZS_enhance) {

if (Qtemp < 0) {

outFlow_ZS[i] = Math.min(QFixed_enhance, inFlow_ZS_mid[i]);

} else {

if (Qtemp > QFixed_enhance) {

outFlow_ZS[i] = Math.min(Qtemp, inFlow_ZS_mid[i]);

} else {

outFlow_ZS[i] = Math.min(QFixed_enhance, inFlow_ZS_mid[i]);

}

}

}

if (outFlow_ZS[i] > qxie_ZS[i]) {

outFlow_ZS[i] = qxie_ZS[i];

}

storageWater_ZS[i + 1] = storageWater_ZS[i] + (inFlow_ZS_mid[i] - outFlow_ZS[i]) * 1 / Math.pow(10, 8);

if (storageWater_ZS[i + 1] > V_HupperLimit_ZS) {

if (inFlow_ZS_mid[i] <= qxie_ZS[i]) {

outFlow_ZS[i] = inFlow_ZS_mid[i];

} else {

outFlow_ZS[i] = qxie_ZS[i];

}

flag = true;

}

}

}

storageWater_ZS[i + 1] = storageWater_ZS[i] + (inFlow_ZS_mid[i] - outFlow_ZS[i]) * 1 / (Math.pow(10, 8) * 1.0);

H_ZS[i + 1] = new LinearInterpolation().LinearInterpolation(storageWater_ZS[i + 1], arry.columns(H_V_ZS, 0), arry.columns(H_V_ZS, 1), 1);

postFlow_ZS_SJK[i + (int) (1 * deltaT * 3600)] = preQSeries_ZS_SJK_mid[i + (int) (1 * deltaT * 3600)] - (inFlow_ZS_mid[i] - outFlow_ZS[i]);

}

for (int i = 0; i < (int) (1 * deltaT * 3600); i++) {

postFlow_ZS_SJK[i] = preQSeries_ZS_SJK_mid[i];

}

for (int i = (int) (postFlow_ZS_SJK.length - deltaT * 3600 * 1); i < postFlow_ZS_SJK.length; i++) {

qxie_ZS[i] = new LinearInterpolation().LinearInterpolation(H_ZS[i], arry.columns(H_qXie_ZS, 0), arry.columns(H_qXie_ZS, 1), 2);      //****时段初水库泄流能力

outFlow_ZS[i] = Math.min(inFlow_ZS_mid[i], qxie_ZS[i]);

if (i + 1 < postFlow_ZS_SJK.length) {

storageWater_ZS[i + 1] = storageWater_ZS[i] + (inFlow_ZS_mid[i] - outFlow_ZS[i]) * 1 / (Math.pow(10, 8) * 1.0);

H_ZS[i + 1] = new LinearInterpolation().LinearInterpolation(storageWater_ZS[i + 1], arry.columns(H_V_ZS, 0), arry.columns(H_V_ZS, 1), 1);

}

}

for (int i = 0; i < dataOut.length; i++) {

dataOut[i][0] = H_ZS[i];                     //****皂市时段初水位

dataOut[i][1] = storageWater_ZS[i];          //****皂市时段初蓄水量

dataOut[i][2] = inFlow_ZS_mid[i];           //****皂市入库流量过程

dataOut[i][3] = outFlow_ZS[i];              //****皂市时段初出库流量 (瞬时值)

dataOut[i][4] = outFlow_ZS[i];              //****皂市出库流量过程(平均值)

dataOut[i][5] = qxie_ZS[i];                 //****皂市泄流能力变化过程

dataOut[i][6] = preQSeries_ZS_SJK_mid[i];   //****皂市调蓄前三江口（石门）流量过程

dataOut[i][7] = postFlow_ZS_SJK[i];         //****皂市调蓄后三江口（石门）流量过程

dataOut[i][8] = postFlow_ZS_SJK[i];         //****皂市调蓄后三江口（石门）流量过程(平均值)

}

String[] title = {"皂市时段初水位", "皂市时段初蓄水量", "皂市入库流量过程", "皂市时段初出库流量", "皂市时段平均出库流量", "皂市时段初泄流能力", "皂市调蓄前三江口（石门）流量", "皂市调蓄后三江口（石门）流量", "皂市调蓄后三江口（石门）流量(平均值)"};

String[][] dataOut_ZS_mid = new String[dataOut.length + 1][dataOut[0].length];

for (int i = 0; i < dataOut_ZS_mid.length; i++) {

for (int i4 = 0; i4 < dataOut_ZS_mid[0].length; i4++) {

if (i == 0) {

dataOut_ZS_mid[i][i4] = title[i4];

} else {

dataOut_ZS_mid[i][i4] = String.valueOf(dataOut[i - 1][i4]);

}

}

}

File writePath_ZS_temp = new File("F:\\Java\\结果\\LiShui2\\结果\\皂市调洪结果\\临时\\皂市" + String.valueOf(H_initial_ZS) + "起调_皂市调洪结果" + "年一遇.txt");

//System.out.println(writePath_ZS_temp);

//new writeTxt().writeFile3(writePath_ZS_temp, dataOut_ZS_mid);        //输出txt

return dataOut;

}

/*

江垭防洪调度

inflow_JY_mid           //****江垭入库流量（时间尺度：秒）

preQSeries_JY_SJK    //****江垭调蓄前三江口流量（时间尺度：秒）

H_V_JY               //****江垭水位库容曲线

H_qXie_JY            //****江垭泄流能力曲线

H_initial_JY         //****江垭初始水位（即江垭起调水位）

QFixed2_JY           //****江垭在转换水位到防洪控制水位间的控制下泄流量1700

deltaT               //洪水资料的精度，2小时

Hchange_JY           //江垭由控泄420转换为控泄1700的转换水位     */

public static double[][] JY(double[] inflow_JY_mid, double[] preQSeries_JY_SJK_mid, double[][] H_V_JY, double[][] H_qXie_JY, double H_initial_JY, double QFixed2_JY, double deltaT, double Hchange_JY, int[] index_JY, int[] index_JY_enhance, double QFixed1_JY, double QFixed_enhance, double Qcontrol_SJK_low) {

//输出数据

double[] postFlow_JY_SJK = new double[inflow_JY_mid.length];            //****江垭调蓄后的三江口流量过程

double[] outFlow_JY = new double[postFlow_JY_SJK.length];                 //****江垭出库流量过程,时段下泄流量

double[] H_JY = new double[postFlow_JY_SJK.length];                       //****江垭水位过程

double[] storageWater_JY = new double[postFlow_JY_SJK.length];            //****江垭蓄水量变化过程

double[] qxie_JY = new double[postFlow_JY_SJK.length];                    //****江垭泄流能力变化过程

double[][] dataOut = new double[postFlow_JY_SJK.length][9];               //****中间输出数据

//double[][] dataout_F_noTitle = new double[preQSeries_JY_SJK.length][9];             //****最后输出数据

double Q_manfa = 425;//江垭满发流量

//参数

double Qcontrol_SJK = 12000;              //****三江口安全泄量

//double QFixed1_JY = 425;                //****江垭满发流量425立方米/s,423.6立方米/s

//double QFixed_enhancec = 425;                //****江垭最低按照满发控制，或最低按照关停控制

double H0_normalWaterLevel_JY = 236;      //****江垭正常蓄水位236m,当水库水库达到236m,水库不在承担下游防洪任务，按照入库流量下泄

double V_normalWaterLevel_JY = new LinearInterpolation().LinearInterpolation(H0_normalWaterLevel_JY, arry.columns(H_V_JY, 0), arry.columns(H_V_JY, 1), 2);//****江垭正常蓄水位236m对应的蓄水量

double V_change_JY = new LinearInterpolation().LinearInterpolation(Hchange_JY, arry.columns(H_V_JY, 0), arry.columns(H_V_JY, 1), 2);//****江垭正常蓄水位236m对应的蓄水量

//计算过程

H_JY[0] = H_initial_JY;

for (int i = 0; i < deltaT * 4 * 3600; i++) {

postFlow_JY_SJK[i] = preQSeries_JY_SJK_mid[i];

}

boolean flag = false;

boolean flag_Hchange = false; //江垭水位未达到转换水位

for (int i = 0; i < postFlow_JY_SJK.length - deltaT * 3600 * 4; i++) {

qxie_JY[i] = new LinearInterpolation().LinearInterpolation(H_JY[i], arry.columns(H_qXie_JY, 0), arry.columns(H_qXie_JY, 1), 2);

storageWater_JY[i] = new LinearInterpolation().LinearInterpolation(H_JY[i], arry.columns(H_V_JY, 0), arry.columns(H_V_JY, 1), 2);

if ((preQSeries_JY_SJK_mid[i + (int) (4 * deltaT * 3600)] <= Qcontrol_SJK) || storageWater_JY[i] >= V_normalWaterLevel_JY || flag) {

if (inflow_JY_mid[i] <= qxie_JY[i]) {

outFlow_JY[i] = inflow_JY_mid[i];

} else {

outFlow_JY[i] = qxie_JY[i];

}

if (H_JY[i] < Hchange_JY && (preQSeries_JY_SJK_mid[i + (int) (4 * deltaT * 3600)] > Qcontrol_SJK_low) && !flag_Hchange) {

if (Q_manfa > inflow_JY_mid[i]) {

outFlow_JY[i] = inflow_JY_mid[i];

} else {

outFlow_JY[i] = Q_manfa;

}//System.out.println("QFixed_enhance:"+QFixed_enhance);

if(QFixed_enhance!=1700) {

boolean flag_index_enhance_JY = false;

for (int i1 = 0; i1 < index_JY_enhance.length; i1++) {

if (index_JY_enhance[i1] * deltaT * 3600 <= i && i < (index_JY_enhance[i1] + 1) * deltaT * 3600) {

flag_index_enhance_JY = true;

}

}

if (flag_index_enhance_JY) {

outFlow_JY[i] = Math.min(QFixed_enhance, inflow_JY_mid[i]);//江垭转换水位以上，且石门有防洪需求时，第三级控泄流量

}

}

if (outFlow_JY[i] > qxie_JY[i]) {

outFlow_JY[i] = qxie_JY[i];

}

storageWater_JY[i + 1] = (inflow_JY_mid[i] - outFlow_JY[i]) * 1 / (Math.pow(10, 8)) + storageWater_JY[i];

if (storageWater_JY[i + 1] > V_change_JY) {

//                        if (inflow_JY_mid[i] <= qxie_JY[i]) {

//                            outFlow_JY[i] = inflow_JY_mid[i];

//                        } else {

//                            outFlow_JY[i] = qxie_JY[i];

//                        }

flag_Hchange = true;

}

}

} else {

if (H_JY[i] < Hchange_JY && !flag_Hchange) {//转换水位以下且石门有防洪需求，按照满发控泄

if (Q_manfa > inflow_JY_mid[i]) {

outFlow_JY[i] = inflow_JY_mid[i];

} else {

outFlow_JY[i] = Q_manfa;

}//System.out.println("QFixed_enhance:"+QFixed_enhance);

//

if(QFixed_enhance!=1700) {

boolean flag_index_enhance_JY = false;

for (int i1 = 0; i1 < index_JY_enhance.length; i1++) {

if (index_JY_enhance[i1] * deltaT * 3600 <= i && i < (index_JY_enhance[i1] + 1) * deltaT * 3600) {

flag_index_enhance_JY = true;

}

}

if (flag_index_enhance_JY) {

outFlow_JY[i] = Math.min(QFixed_enhance, inflow_JY_mid[i]);//江垭转换水位以上，且石门有防洪需求时，第三级控泄流量

}

}

if (outFlow_JY[i] > qxie_JY[i]) {

outFlow_JY[i] = qxie_JY[i];

}

storageWater_JY[i + 1] = (inflow_JY_mid[i] - outFlow_JY[i]) * 1 / (Math.pow(10, 8)) + storageWater_JY[i];

if (storageWater_JY[i + 1] > V_change_JY) {

//System.out.println("第" + (i - 1) + "秒时刻后,江垭固定下泄420转为固定下泄1700");

//                        if (inflow_JY_mid[i] <= qxie_JY[i]) {

//                            outFlow_JY[i] = inflow_JY_mid[i];

//                        } else {

//                            outFlow_JY[i] = qxie_JY[i];

//                        }

flag_Hchange = true;

}

if (storageWater_JY[i + 1] > V_normalWaterLevel_JY) {//

//System.out.println("第" + i + "时刻后,江垭达到正常蓄水位236");

if (inflow_JY_mid[i] <= qxie_JY[i]) {

outFlow_JY[i] = inflow_JY_mid[i];

} else {

outFlow_JY[i] = qxie_JY[i];

}

flag = true;

}

} else {

double QFixedTemp_JY = 0;

boolean flag_index_JY = false;

for (int i1 = 0; i1 < index_JY.length; i1++) {

if (index_JY[i1] * deltaT * 3600 <= i && i < (index_JY[i1] + 1) * deltaT * 3600) {

flag_index_JY = true;

}

}

if (flag_index_JY) {

QFixedTemp_JY = QFixed1_JY;//江垭转换水位以上，且石门有防洪需求时，第二级控泄流量

} else {

QFixedTemp_JY = QFixed2_JY;//江垭转换水位以上，且石门有防洪需求时，第一级控泄流量1700

}

boolean flag_index_enhance_JY = false;

for (int i1 = 0; i1 < index_JY_enhance.length; i1++) {

if (index_JY_enhance[i1] * deltaT * 3600 <= i && i < (index_JY_enhance[i1] + 1) * deltaT * 3600) {

flag_index_enhance_JY = true;

}

}

if (flag_index_enhance_JY) {

QFixedTemp_JY = QFixed_enhance;//江垭转换水位以上，且石门有防洪需求时，第三级控泄流量

}

if (QFixedTemp_JY < inflow_JY_mid[i]) {

outFlow_JY[i] = QFixedTemp_JY;

} else {

outFlow_JY[i] = inflow_JY_mid[i];

}

if (outFlow_JY[i] > qxie_JY[i]) {

outFlow_JY[i] = qxie_JY[i];

}

storageWater_JY[i + 1] = (inflow_JY_mid[i] - outFlow_JY[i]) * 1 / (Math.pow(10, 8)) + storageWater_JY[i];

if (storageWater_JY[i + 1] > V_normalWaterLevel_JY) {//

//System.out.println("第" + i + "时刻后,江垭达到正常蓄水位236");

if (inflow_JY_mid[i] <= qxie_JY[i]) {

outFlow_JY[i] = inflow_JY_mid[i];

} else {

outFlow_JY[i] = qxie_JY[i];

}

flag = true;

}

}

}

storageWater_JY[i + 1] = (inflow_JY_mid[i] - outFlow_JY[i]) * 1 / (Math.pow(10, 8)) + storageWater_JY[i];

H_JY[i + 1] = new LinearInterpolation().LinearInterpolation(storageWater_JY[i + 1], arry.columns(H_V_JY, 0), arry.columns(H_V_JY, 1), 1);

postFlow_JY_SJK[i + (int) (4 * deltaT * 3600)] = preQSeries_JY_SJK_mid[i + (int) (4 * deltaT * 3600)] - (inflow_JY_mid[i] - outFlow_JY[i]);

}

for (int i = (int) (postFlow_JY_SJK.length - deltaT * 3600 * 4); i < postFlow_JY_SJK.length; i++) {

qxie_JY[i] = new LinearInterpolation().LinearInterpolation(H_JY[i], arry.columns(H_qXie_JY, 0), arry.columns(H_qXie_JY, 1), 2);

outFlow_JY[i] = Math.min(qxie_JY[i], inflow_JY_mid[i]);

if (i + 1 < postFlow_JY_SJK.length) {

storageWater_JY[i + 1] = (inflow_JY_mid[i] - outFlow_JY[i]) * 1 / (Math.pow(10, 8)) + storageWater_JY[i];

H_JY[i + 1] = new LinearInterpolation().LinearInterpolation(storageWater_JY[i + 1], arry.columns(H_V_JY, 0), arry.columns(H_V_JY, 1), 1);

}

}

for (int i = 0; i < dataOut.length; i++) {

dataOut[i][0] = H_JY[i];                     //****江垭时段初水位

dataOut[i][1] = storageWater_JY[i];          //****江垭时段初蓄水量

dataOut[i][2] = inflow_JY_mid[i];           //****江垭入库流量过程

dataOut[i][3] = outFlow_JY[i];              //****江垭时段初出库流量 (瞬时值)

dataOut[i][4] = outFlow_JY[i];              //****江垭出库流量过程(平均值)

dataOut[i][5] = qxie_JY[i];                 //****江垭泄流能力变化过程

dataOut[i][6] = preQSeries_JY_SJK_mid[i];   //****江垭调蓄前三江口（石门）流量过程

dataOut[i][7] = postFlow_JY_SJK[i];         //****江垭调蓄后三江口（石门）流量过程

dataOut[i][8] = postFlow_JY_SJK[i];         //****江垭调蓄后三江口（石门）流量过程（平均）

}

String[] title = {"江垭时段初水位", "江垭时段初蓄水量", "江垭入库流量过程", "江垭时段初出库流量", "江垭时段平均出库流量", "江垭时段初泄流能力", "江垭调蓄前三江口（石门）流量", "江垭调蓄后三江口（石门）流量", "江垭调蓄后三江口（石门）流量(平均)"};

String[][] dataOut_JY_mid = new String[dataOut.length + 1][dataOut[0].length];

for (int i = 0; i < dataOut_JY_mid.length; i++) {

for (int i4 = 0; i4 < dataOut_JY_mid[0].length; i4++) {

if (i == 0) {

dataOut_JY_mid[i][i4] = title[i4];

} else {

dataOut_JY_mid[i][i4] = String.valueOf(dataOut[i - 1][i4]);

}

}

}

return dataOut;

}

}

package tools;

public class LinearInterpolation {

public static double LinearInterpolation(double referenceData, double[] curve_data_A, double[] curve_data_B, int index) {

double interpolationResult = 0;

if (curve_data_A.length == curve_data_B.length) {

if (index == 1) {

for (int i = 0; i < curve_data_A.length - 1; i++)

if (((referenceData >= curve_data_B[i]) && (referenceData <= curve_data_B[i + 1])) || ((referenceData > curve_data_B[i]) && (referenceData <= curve_data_B[i + 1])))

interpolationResult = (referenceData - curve_data_B[i]) / (curve_data_B[i + 1] - curve_data_B[i]) * (curve_data_A[i + 1] - curve_data_A[i]) + curve_data_A[i];

} else if (index == 2) {

for (int i = 0; i < curve_data_B.length - 1; i++)

if (((referenceData >= curve_data_A[i]) && (referenceData <= curve_data_A[i + 1])) || ((referenceData > curve_data_A[i]) && (referenceData <= curve_data_A[i + 1])))

interpolationResult = (referenceData - curve_data_A[i]) / (curve_data_A[i + 1] - curve_data_A[i]) * (curve_data_B[i + 1] - curve_data_B[i]) + curve_data_B[i];

} else System.out.println("请输入 1 或 2 来指定要插的列,获取数据的那一列");

} else System.out.println("数据长度不一致，无法插值");

return interpolationResult;

}

}

package tools;

import java.io.BufferedWriter;

import java.io.File;

import java.io.FileWriter;

import java.io.IOException;

/*

将数据写入txt文件

*/

public class writeTxt {

/*

writeFile3可以输出带表头的String[][] dataIn

*/

public static void writeFile3(File writeName, String[][] dataIn) {

try {

writeName.createNewFile(); // 创建新文件,有同名的文件的话直接覆盖

try (FileWriter writer = new FileWriter(writeName);

BufferedWriter out = new BufferedWriter(writer)

) {

for (int i = 0; i < dataIn.length; i++) {

StringBuilder sb = new StringBuilder();

for (int j = 0; j < dataIn[0].length; j++) {

if (j == 0) {

sb = sb.append(dataIn[i][j]);

} else {

sb = sb.append("\t" + dataIn[i][j]);

}

}

out.write(sb + "\r\n"); // \r\n即为换行

}

//out.write("我会写入文件啦1\r\n"); // \r\n即为换行

//out.write("我会写入文件啦2\r\n"); // \r\n即为换行

out.flush(); // 把缓存区内容压入文件

//System.out.println("数据已成功输出");

}

} catch (IOException e) {

e.printStackTrace();

}

}

}

package tools;

import java.io.BufferedReader;

import java.io.File;

import java.io.FileReader;

import java.io.IOException;

import java.util.ArrayList;

import java.util.List;

//数据直接读取为double类型

public class readTxt_F {

public static double[][] file2doubleArray(File file, int title) {

BufferedReader br = null;

List<String> list = new ArrayList<String>();

try {

br = new BufferedReader(new FileReader(file));

String str = null;

int count = 0;

while ((str = br.readLine()) != null) {

if ((count == 0) && (title == 1)) {

} else {

list.add(str);

}

count++;

}

br.close();

} catch (IOException e) {

e.printStackTrace();

}

String[] strs = list.toArray(new String[0]);

/*

读入的String矩阵转换为double数据类型

*/

String[] str;

int width = strs[0].split("\\\t").length;//根据空格截取字符串，根据逗号截取字符串str= strs[i].split("\\, "),width表示矩阵宽度

double[][] data = new double[strs.length][width];

for (int i = 0; i < strs.length; i++) {

for (int j = 0; j < width; j++) {

str = strs[i].split("\\\t");//根据空格、逗号截取字符串，str= strs[i].split("\\	")、str= strs[i].split("\\, ")，容易出错，打开txt粘贴空格或逗号

data[i][j] = Double.valueOf(str[j]);

}

}

return data;

}

}