NAME

dr3m - Distributed Routing Rainfall-Runoff Model--version II

ABSTRACT

DR3M is a watershed model for routing storm runoff through a

Branched system of pipes and (or) natural channels using rainfall as

input.  DR3M provides detailed simulation of storm-runoff periods

selected by the user.  There is daily soil-moisture accounting

between storms.  A drainage basin is represented as a set of

overland-flow, channel, and reservoir segments, which jointly

describe the drainage features of the basin.  This model is usually

used to simulate small urban basins.  Interflow and base flow are

not simulated.  Snow accumulation and snowmelt are not simulated.

METHOD

The rainfall-excess components include soil-moisture accounting,

pervious-area rainfall excess, impervious-area rainfall excess, and

parameter optimization.  The Green-Ampt equation is used in the

calculations of infiltration and pervious area rainfall excess.  A

Rosenbrock optimization procedure may be used to aid in calibrating

several of the infiltration and soil-moisture accounting parameters.

Kinematic wave theory is used for both overland-flow and channel

routing.  There are three solution techniques available:  method of

characteristics, implicit finite difference method, and explicit

finite difference method.  Two soil types may be defined.  Overland

flow may be defined as turbulent or laminar.  Detention reservoirs

may be simulated as linear storage or using a modified-Puls method.

Channel segments may be defined as gutter, pipe, triangular cross

section, or by explicitly specifying the kinematic channel

parameters alpha and m.

HISTORY

1991 - DR3M-version II, added option to output simulated time-series

data to Watershed Data Management (WDM) file.  Output file

modified to reduce width from 132 characters to 80 characters or

less.

1984 - DR3M-version II, WDM file replaces "card" input of time-

series data.

1982 - DR3M-version II, added two solution techniques for kinematic

wave routing.  Improved general output.

1978 - Original DR3M version, incorporated the routing component

from a version of the Massachusetts Institute of Technology

catchment model into the lumped parameter rainfall-runoff model.

1972 - A lumped parameter rainfall-runoff model for small rural

watersheds

DATA REQUIREMENTS

Daily precipitation, daily evapotranspiration, and short-interval

precipitation are required.  Short-interval discharge is required

for the optimization option and to calibrate the model.  These time

series are read from a WDM file.  Roughness and hydraulics

parameters and sub-catchment areas are required to define the basin.

Six parameters are required to calculate infiltration and soil-

moisture accounting.  Up to three rainfall stations may be used.

Two soil types may be defined.  A total of 99 flow planes, channels,

pipes, reservoirs, and junctions may be used to define the basin.

OUTPUT OPTIONS

The computed outflow from any flow plane, pipe, or channel segment

for each storm period may be written to the output file or to the

WDM file.  A summary of the measured and simulated rainfall, runoff,

and peak flows is written to the output file.  A flat file

containing the storm rainfall, measured flow (if available), and

simulated flow at user selected sites can be generated.  A flat file

for each storm containing the total rainfall, the measured peak flow

(if available), and the simulated peak flow for user-selected sites

can be generated.

SYSTEM REQUIREMENTS

DR3M is written in Fortran 77 with the following extension: use of

include files. The UTIL, ADWDM, and WDM libraries from LIB are used.

A subset of these libraries is provided with the code and may be

used instead of the libraries, this subset uses INTEGER*4 and mixed

type equivalence. For more information, see System Requirements in

LIB.

DOCUMENTATION

Alley, W.M., and Smith, P.E., 1982, Distributed routing rainfall-

runoff model--version II:  U.S. Geological Survey Open-File

Report 82-344, 201 p.

RELATED DOCUMENTATION

Flynn, K.M., Hummel, P.R., Lumb, A.M., Kittle, J.L., Jr., 1995,

User's manual for ANNIE, version 2, a computer program for

interactive hydrologic data management:  U.S. Geological Survey

Water-Resources Investigations 95-4085, 211 p.

REFERENCES

Dawdy, D.R., Lichty, R.W., and Bergmann, J.M., 1972, A rainfall-

runoff simulation model for estimation of flood peaks for small

drainage basins: U.S. Geological Survey Professional Paper 506-B,

28 p.

Dawdy, D.R., Schaake, J.C., Jr., and Alley, W.M., 1978, User's guide

for distributed routing rainfall-runoff model:  U.S. Geological

Survey Water-Resources Investigations Report 78-90, 146 p.

Doyle, H.W., Jr., and Miller, J.E., 1980, Calibration of a

distributed routing rainfall-runoff model at four urban sites

near Miami, Florida: U.S. Geological Survey Water-Resources

Investigations Report 80-1, 87 p.

Guay, J.R., and Smith, P.E., 1988, Simulation of quantity and

quality of storm runoff for urban catchments in Fresno,

California:  U.S. Geological Survey Water-Resources

Investigations Report 88-4125, 76 p.

Leclerc, Guy, and Schaake, J.C., Jr., 1973, Methodology for

assessing the potential impact of urban development on urban

runoff and the relative efficiency of runoff control

alternatives:  Ralph M. Parsons Laboratory Report no. 167,

Massachusetts Institute of Technology, 257 p.

TRAINING

Watershed Systems Modeling I (SW2008TC), offered annually at the

USGS National Training Center.

Watershed Systems Modeling II (SW3018TC), offered upon request at

the USGS National Training Center.

CONTACTS

Operation and Distribution:

U.S. Geological Survey

Hydrologic Analysis Software Support Program

437 National Center

Reston, VA 20192

h2osoft@usgs.gov

Official versions of U.S. Geological Survey water-resources analysis

software are available for electronic retrieval via the World Wide

Web (WWW) at:

http://water.usgs.gov/software/

and via anonymous File Transfer Protocol (FTP) from:

water.usgs.gov (path: /pub/software).

The WWW page and anonymous FTP directory from which the DR3M

software can be retrieved are, respectively:

http://water.usgs.gov/software/dr3m.html

--and--

/pub/software/surface_water/dr3m