._  ._. _. .  .     ._  ._. _. .  .     ._  ._. _. .  .

| | |_| _| |\/|     | | |_| _| |\/|     | | |_| _| |\/|

|_| | \ _| |  |     |_| | \ _| |  |     |_| | \ _| |  |

Distributed Routing Rainfall-Runoff Model--Version II

April, 1994, Release 2.0

While the basic algorithms in DR3M have not been modified, there have been

major changes in the code since the Open-File report 82-344 was printed.

Most of these changes are related to the files that are read and written by

DR3M.  There were a number of reasons for these changes.  Most notably:

- WDM file interface added.  Model now reads time series data from a WDM

file.  Simulated flows can be output to the same WDM file.  This allows

the use of the data management capabilities of ANNIE for pre and post

processing of the time series data.

- output file is now 80 characters wide instead of 132.  Easier to use in

interactive, PC, and workstation environments.

- opening of input/output files has been moved from the system level to a

program function.  The user is now asked to enter the name of a master

file that contains a list of the files needed for a particular DR3M model

run.  The format of this file is documented below.

- input file has a number of changes.  These changes included simplifying

the way storm periods were specified, additions required to define the

model link with the WDM file, and improved compatibility with the model

DR3M-QUAL.  The revised format of this file is documented below.

There is a brief discussion below of the test data sets distributed with DR3M.

_______________________________________________________________________________

REFERENCES:

Alley, W. M., and Smith, P. E., 1982, Distributed Routing

Rainfall-Runoff Model--Version II: U.S. Geological

Survey Open-file Report 82-344

Lumb, A. M., Kittle, J. L., and Flynn, K. M., Users Manual

for ANNIE; a Computer Program for Interactive Hydrologic

Analyses and Data Management U.S. Geological Survey

Open-file Report 89-4080.

_______________________________________________________________________________

EXAMPLE/TEST DATA SETS

Example/Test data sets are distributed with DR3M.  These include all of the

input time series, the input files, and the output files for 5 test runs.

These tests are described in the table below.  Test is the test number.

Program is the program that is run for the test.  Status indicates how a file

is used; i for input, o for output, and i/o for both input and output.

See the installation instructions distributed with the program for specific

instructions on running the tests on the system you are using.

test program description of test   status file name   file description

____ _______ _____________________ ______ __________  __________________________

01   annie  create and fill WDM      i   test1.log   annie command file

i   sand.xpt    Sand Creek data

o   sand.wdm    data management file

02    DR3M  optimization             i   test2.mtr   master file

i   test2.inp   model control file

i   sand.wdm    data management file

o   test2.out   output/print file

o   test2.dmp   summary of data

03    DR3M  routing-flows saved      i   test3.mtr   master file

to WDM                 i   test3.inp   model control file

i/o  sand.wdm    data management file

o   test3.out   output/print file

o   test3.dmp   summary of data

o   test3.pks   pltgen file of peaks

o   test3.unt   pltgen file of hydrographs

04    DR3M  routing-flows saved      i   test4.mtr   master file

to old format          i   test4.inp   model control file

i   sand.wdm    data management file

o   test4.out   output/print file

o   test4.dmp   summary of data

o   test4.sgs   segment flow file

o   test4.pks   pltgen file of peaks

o   test4.unt   pltgen file of hydrographs

05    DR3M  routing-flows are not    i   test5.mtr   master file

saved                  i   test5.inp   model control file

i   sand.wdm    data management file

o   test5.out   output/print file

o   test5.dmp   summary of data

o   test5.pks   pltgen file of peaks

o   test5.unt   pltgen file of hydrographs

_______________________________________________________________________________

MASTER CONTROL FILE:

RCRD  COLUMNS  FORMAT  VARIABLE  DESCRIPTION

----  -------  ------  --------  ---------------------------------------------

1                              FILE CONTROL RECORD--Min of 3, Max of 7

File names may be entered in any order.

1-3    A3      CODES     Identifier for file type, see below for

required values

11-74   A64     NAME      Name of file.  May be any name that is

valid on the computer system being used.

May include the complete path name if

necessary.  The length of the file name

may be restricted on some machines.

CODES  Required  Description

-----  --------  ----------------------------

***      opt    Comment record

WDM      yes    WDM file containing observed

data.  Simulated data may be

output to this file.

INP      yes    Model control file, input.

OUT      yes    Model output (print) file.

DMP      opt    Listing of input time series

data.

UNT      opt    PLTGEN file of simulated

hydrographs

PKS      opt    PLTGEN file of simulated

peaks.

SGS      opt    Original format segment file

_______________________________________________________________________________

Example:

1         2         3         4         5         6         7         8

12345678901234567890123456789012345678901234567890123456789012345678901234567890

WDM       sand.wdm

INP       test04.inp

OUT       test04.out

DMP       test04.dmp

UNT       test04.unt

PKS       test04.pks

SGS       test04.sgs

1         2         3         4         5         6         7         8

12345678901234567890123456789012345678901234567890123456789012345678901234567890

_______________________________________________________________________________

MODEL CONTROL FILE (note variables flagged by "qual>>" are for DR3M-QUAL,

and do not need to be input to DR3M.)

RCRD  COLUMNS  FORMAT  VARIABLE  DESCRIPTION

----  -------  ------  --------  ---------------------------------------------

1                              PROGRAM CONTROL RECORD--REQUIRED

1-4    I1      OPTION    Option to list time series data

('    '-no,'LIST'-yes)

If LIST, all input time series are

output to Fortran unit 11.

5    I1      OPT       Unit discharge input (0-yes,1-no)

0 - read from WDM file

1 - not input

6    I1      NOPT1     Irrigation data input

0 - no

1 - record type 2 IRREG

7-8    I2      JPERM     Save segment outflows

0 - no

1 - old format file

2 - WDM file

9-10   I2      JPUN      Output simulated outflow hydrograph

0 - no

1 - WDM file, record type 3 OTHYD required

11-12   I2      JINH      Input inflow hydrograph

0 - no

1 - WDM file, record type 3 INHYD required

13-14   I2      JPTG      Output to pltgen file

0 - no

1 - hydrographs

2 - peaks

3 - both hydrographs and peaks

if not 0, set IPR = 2 for at least

one segment in group 9

21-28   F8.0    DA        Drainage area of basin, square miles

29-32   F4.0    PTIME     Time interval of unit data, minutes,

restricted to : 1, 2, 3, 4, 5, 10, 15,

30, 45, or 60

33-36   I4      NRG       Number of rain gages, max of 3

(one record type 3 UNPRn required for

each)

76-80   CNTRL   ---       Card identifier

RCRD  COLUMNS  FORMAT  VARIABLE  DESCRIPTION

----  -------  ------  --------  ---------------------------------------------

2                              IRRIGATION DATA--REQUIRED IF NOPT1=1

1-60   12F5.3  IRR(1-12) Monthly irrigation rates, inches/week

76-80   IRREG   ---       Card identifier

----  -------  ------  --------  ---------------------------------------------

3                              STATION RECORDS -- MIN OF 4, MAX OF 8

( in order listed below )

1-16   A16     STA(1:16) Station number

17-66   A50     STA(17:66)Station name and/or description

67-71   I5      DSN       Time series data set index number

76-80   xxxxx   ---       Card identifier, see below for values

-------------------------------

STATION TYPE      xxxxx REQUIRED

----------------  ----- --------

Unit discharge    DISCH    yes

Evaporation       EVAPR    yes

Daily rain        PRECP    yes

Unit rain 1       UNPR1    yes

Unit rain 2       UNPR2     no

Unit rain 3       UNPR3     no

Outflow station   OTHYD     no

Inflow station    INHYD     no

----  -------  ------  --------  ---------------------------------------------

4                              PERIOD OF DAILY RECORD--REQUIRED

1-15   I4,2I3  JBEG      Begin date (yr,mo,dy)

17-26   I4,2I3  JEND      End date (yr,mo,dy)

32   I1      CODE      Flag for last 4 record (0-no,9-yes)

33-37   I5      NOFE      Number of storm events, max of 60,

entered on first 4 record only.

76-80   PRD n             Card identifier, n starts at zero and

is incremented by one for each 4 record

RCRD  COLUMNS  FORMAT  VARIABLE  DESCRIPTION

----  -------  ------  --------  ---------------------------------------------

4                              continued

First record 4 defines the entire period

of simulation.  If the simulation

period contains a broken record,

additional 4 records are required to

define each period.  Example :

76 01 01       80 12 31

76 01 01       78 12 31

80 01 01       80 12 31       9

----  -------  ------  --------  ---------------------------------------------

5                              STORM RECORDS--NOFE RECORDS REQUIRED

1-15  I4,3I3,I2BEG       Begin date (yr,mo,dy,hr,mn)

17-31  I4,3I3,I2END       End date (yr,mo,dy,hr,mn)

33    I1      KOUT      Routing performed for storm (1-yes,0-no)

34    I1      TESTNO    Storm used in computation of objective

function (0-no,1-yes)

35    I1      IPL       Plot outlet discharge (0-no,1-yes)

36    I1      IHYD      Input hydrograph (0-no,1-yes if yes,

record type 3 INHYD required)

qual>>    38    I1      KOUT      List simulated concentrations (1-yes,0-no)

qual>>    39    I1      IPL       Plot water quality data

0 - no

1 - instantaneous concentrations

2 - instantaneous concentrations and load

characteristics curves

3 - load characteristics curves

qual>>    40    I1      NWQPF     Flag for observed water quality data

(0-no,1-yes)

44-50   F7.2    VOL1      Runoff volume, inches, not required

51-55   F5.2    DISCH     Constant discharge input to flow

computations, required if an input-

discharge segment (type 11) is used

qual>> 56-75    4F5.2   WFALL     Precipitation concentration for each constituent

76-80   STMnn   ---       Card identifier, nn = storm number

RCRD  COLUMNS  FORMAT  VARIABLE  DESCRIPTION

----  -------  ------  --------  ---------------------------------------------

6                              OPTIMIZATION RECORDS--REQUIRED

1-4    I4      NPAR      Number of soil types in the basin

1 - basin is to be treated as 1 soil type

2 - basin is divided into 2 parts with

different infiltration and soil moisture

parameters.  PTIME must be 5 minutes or

greater for NPAR = 2.

5-8    I4      FO        Number of parameters to be adjusted

9-12   I4      K         Number of trial adjustments per

parameter

13-20   F8.0    EPSLN     Initial step size multiplier for

parameter adjustment (usually .05)

76-80   OPTIM             Card identifier

----  -------  ------  --------  ---------------------------------------------

7                              PARAMETER RECORDS--7 RECORDS REQUIRED

1-10   F10.0   X(n)      Initial magnitude

11-20   F10.0   G(n)      Lower limit (see guidelines below)

21-30   F10.0   H(n)      Upper limit (see guidelines below)

31-34   I4      OPTNO(n)  Optimization flag (0-no, 1-yes)

include the following 4 if NPAR = 2

36-45   F10.0   X(n+7)    Initial magnitude

46-55   F10.0   G(n+7)    Lower limit (see guidelines below)

56-65   F10.0   H(n+7)    Upper limit (see guidelines below)

66-69   I4      OPTNO(n+7)Optimization flag (0-no, 1-yes)

76-80   yyyy    ---       Card identifier, see below

--------------------------------

ORDER  PARAMETER   LOWER   UPPER

n       yyyy       G       H

-----  ---------   -----   -----

1,8      PSP        .5     8.

2,9      KSAT       .05    1.2

3,10     RGF       5.     20.

4,11     BMSN      2.      6.

5,12     EVC        .5     1.

6,13     RR         .7      .95

7        EAC        .85    1.15

RCRD  COLUMNS  FORMAT  VARIABLE  DESCRIPTION

----  -------  ------  --------  ---------------------------------------------

8                              MODEL CONTROL--REQUIRED

1-5    I5      NSEG      Number of different segments used to

describe basin (2 to 99)

6-10   F5.0    IMP       Maximum inpervious retention, inches

11-15   F5.0    DT        Time interval used in flow routing,

minutes, allowable values :

PTIME  DT

1-4   .1, .2, .5, 1.

5,10  .1, .2, .5, 1., 2.5, 5.

15-60  .1, .2, .5, 1., 2.5, 5., 7.5, 15.

16-20   F5.0    RAT       Ratio of the sum of the pervious and

noneffective impervious areas to the

pervious area  ( greater than or equal

to 1.0 )

21-25   F5.0    ALPADJ    Alpha adjustment--segment alpha's

computed from segment data are

multiplied by ALPADJ (initially = 1.0)

26-30   F5.2    WX        Model parameter WX corresponds to

weighting factor (W) on space derivative

in implicit finite difference method

( 0.5 < WX <= 1.0 )

qual>>  31-35   F5.2    DAREA     Effective impervious area, in acres

qual>>  36-40   F5.2    SSAREA    Fraction of effective impervious area that

is swept by street sweepers.  (Required

for lumped parameter simulation only.)

76-80   SEGMT   ---       Card identifier

RCRD  COLUMNS  FORMAT  VARIABLE  DESCRIPTION

----  -------  ------  --------  ---------------------------------------------

9A                             SEGMENT CHARACTERISTICS--REQUIRED

see DR3M manual page 53, group 16

1-4    A4      ISEG      Alphanumeric identification for segment

6-17   3A4     IUP(1-3)  Alphanumeric identification for up to 3

segments which contribute inflow to the

upstream end of this segment (leave blank

where upstream segments are not present).

18-33   4A4     ILAT(1-4) Alphanumeric identification for up to 4

segments which contribute uniform lateral

inflow into this segment (leave blank

where lateral inflow segments are not

presend).

qual>>  34-42   A9      IP1       Alphanumeric designation for land use of

lateral inflow setments.  Leave blank if

ITYPE is not 1.

43-44   I2      KPSET     Soil type for overland flow segment.

If NPAR = 1, enter a 1.

If NPAR = 2, then use 1 for soil parameters

1-6, use 2 for soil parameters 8-13.

Leave blank for segment types 1-4 and 7-11.

45-49   F5.0    FLGTH     Length of segment, in feet

50-54   F5.0    SLOPE     Slope of segment, in feet per feet

55-59   F5.0    FRN       Roughness coefficient for segment.

for ITYPE:

4, 7 - blank

1, 2, 3, 5 - Manning's n

6 - emperical coefficient for laminar

overland flow

61      I1      METH      Kinematic routing solution method

0 - explicit finite difference method

1 - method of characteristics

2 - implicit finite difference method

63-64   I2      NDX       for ITYPE:

1-6 and finite difference routing it is

the number of ntervals into which total

length is divided

8 it is the number of points in the storage-

outflow relationship

all others, leave blank

maximum value of NDX for any segment is 10.

RCRD  COLUMNS  FORMAT  VARIABLE  DESCRIPTION

----  -------  ------  --------  ---------------------------------------------

9A                             continued

66-67   I2      ITYPE     Type of segment

1 - gutter

2 - pipe

3 - triangular cross section

4 - to specify explicitly the kinematic

channel parameters alpha and m

5 - overland flow (turbulent)

6 - overland flow (laminar)

7 - junction

8 - detention reservoir, modified-Puls

9 - detention reservoir, linear storage

10 - input hydrograph point (only one allowed)

11 - input discharge point (only one allowed)

qual>>  68-69   I2      ITYPE     Type of segment

1 - channel segment

2 - overland-flow segment

3 - reservoir segment

4 - nodal segment

76-80  SEG 1              Record identifier

----  -------  ------  --------  ---------------------------------------------

9B                             SEGMENT CHARACTERISTICS--REQUIRED

6-15   2F5.0   PARAM(1-2) Pair of parameters which depend on type of

segment

16-30   3F5.0   RCOEF(1-3) for ITYPE

4 - channel segments whic have 2 alpha and m

see manual

5,6 - thiessen coefficients for overland

flow for each rain gage.

qual>>  31-50   2F5.0   PARAM(1-4) Parameters which depend on type of

segment

76-80   SEG 2             Record identifier

RCRD  COLUMNS  FORMAT  VARIABLE  DESCRIPTION

----  -------  ------  --------  ---------------------------------------------

9C                             SEGMENT CHARACTERISTICS--REQUIRED

6-7    I2      IPR       Outflow output indicator

0 - not output

1 - outflow hydrograph is printed

2 - outflow hydrograph to PLTGEN

qual>>   8-9    I2      IPR       Outflow output indicator

0 - not output

1 - outflow conentration is printed

2 - outflow conentration to PLTGEN

11-15   I5      DSNS      data set number for segment flow

qual>>  16-35   I5      DSNS      data set number for concentrations

76-80   SEG 3             Record identifier

----  -------  ------  --------  ---------------------------------------------

10                              OUTFLOW-STORAGE--REQUIRED FOR TYPE 8

SEGMENTS

1-10   F10.0   O2        Outflow, in cfs

11-20   F10.0   S2        Storage, in cfs-hours

_______________________________________________________________________________

Example:

1         2         3         4         5         6         7         8

12345678901234567890123456789012345678901234567890123456789012345678901234567890

LIST10 0 0 0 0          .377  5.   1                                       CNTRL

06711600        Sanderson Gulch Tributary at Lakewood, CO                  DISCH

06712990        Cherry Creek Lake, CO                                 6    EVAPR

06711600        Sanderson Gulch Tributary at Lakewood, CO             5    PRECP

06711600        Sanderson Gulch Tributary at Lakewood, CO             7    UNPR1

77  4 01        79  8 27          6                                      PRD 0

77  4 01        77  7 29                                                 PRD 1

79  5 02        79  8 27     9                                           PRD 2

77  6  5 2215   77  6  6 0240 0100          .116                         STM 1

77  7 20 1950   77  7 21 0045 0100          .160                         STM 2

77  7 25 1540   77  7 25 2100 0100          .159                         STM 3

79  7  4 1745   79  7  4 2020 0100          .215                         STM 4

79  8 10 0405   79  8 10 0900 0100          .127                         STM 5

79  8 19 1700   79  8 19 2000 0100          .214                         STM 6

1   1  10     .06                                                       OPTIM

6.0       0.5       8.0   0                                         PSP 1

.25        .1        .6   0                                         KSAT1

10.        5.       20.   0                                         RGF 1

4.        2.        6.   0                                         BMSN1

.7        .5        1.   0                                         EVC 1

.8        .7       .95   0                                         RR  1

1.       .85      1.15   1                                         EAC 1

2  .05   .5 1.29 1.00                                                  SEGMT

FP01                                       13240.  .20 .016 0  1  5        SEG 1

1.00  .25  1.0                                                       SEG 2

0                                                                    SEG 3

CH01             FP01                       3240.           0  1  4        SEG 1

3.2  1.3                                                            SEG 2

0                                                                    SEG 3

1         2         3         4         5         6         7         8

12345678901234567890123456789012345678901234567890123456789012345678901234567890