Session 1: Introduction

Download the tutorial.tar.gz file from the dhsvm ftp site

ftp://ftp.hydro.washington.edu/pub/dhsvm/

To access through netscape, type:

unix% netscape &

Select tutorial.tar.gz and save it to your working directory.

## Unzip and untar the file

unix% gunzip tutorial.tar.gz

unix% tar -xvf tutorial.tar

This should expand into the following directory structure:

tutorial/

arcinfo.tar.gz

configfiles/

metfiles/

output/

GMTscripts/

programs/

source/

mwm/

dhsvm_2.0.1.tar.gz

Unzip/untar the files in the arcinfo database

unix% cd tutorial

unix% gunzip arcinfo.tar.gz

unix% tar -xvf arcinfo.tar

This should expand into the following directory structure:

arcinfo/

rainysoil/

rainyveg/

rainydem/

rainymask/

roads/

rainy10m/

amlscripts/

info/

Run arc/info and view database files

Arc/Info only runs on one of our computers (plane), so here are our system-specific commands for running arc/info.

unix% xterm & <switch to new window>

unix% ssh plane

plane% cd tutorial/arcinfo/

plane% arc

View database files: rainysoil, rainyveg, rainydem & rainymask

Convert NODATA from -9999 to 0 in the mask file:

Arc: grid

Grid: rainymasks = con (isnull (rainymask), 0, rainymask)

Grid: quit

Export soil, vegetation, mask and dem files as ascii grids

Arc: gridascii rainysoil rainysoil.txt, etc.

Arc: quit

## Convert grids to binary

Since Arc/Info runs on a SUN and we are running on pc's (different byteorder), you must convert the grids to binary on your local machine (i.e. go back to your home window).

Remove the headers from the ascii files:

unix% cd tutorial/arcinfo

If necessary, make the following script executable:

unix% chmod 755 ../programs/rmhdr.scr

unix% ../programs/rmhdr.scr

Only use this script once. Multiple uses will cause errors.

Compile the utility program myconvert.c:

unix% cd tutorial/programs/

unix% gcc myconvert.c -lm -o myconvert

Use myconvert to create dhsvm binary input grids:

usage: myconvert source_format target_format source_file target_file number_of_rows number_of_column

unix% myconvert ascii char rainymask.txt rainymask.bin 52 67

Repeat for each of the map files, changing the target format as follows:

mask	= char

dem	= float

soil	= char

veg	= char

Create model states

Compile the MakeModelStateBin.c:

unix% make -f Makefile.ModelState

Usage: MakeModelState <infofile>

The info file MUST contain the following information:

- path for output file

- date for the model state, in mm/dd/yyyy-hh

- number of rows (ny) and number of columns (nx)

- maximum number of vegetation layers

- rain interception in m for each vegetation layer

- snow interception in m for top vegetation layer

- snow cover mask

- number of days since last snow fall

- snow water equivalent in m

- liquid water content in m of bottom layer of snowpack

- temperature in C of bottom layer of snow pack

- liquid water content in m of top layer of snowpack

- temperature in C of top layer of snow pack

- cold content of snow pack

- maximum number of root zone layers

- volumetric soil moisture content for each layer

(including the layer below the lowest root zone layer)

- temperature in C at soil surface

- soil temperature in C for each root zone layer

- ground heat storage

- runoff

InititalState.txt provides a sample input file set up for Rainy Creek, assuming no interception storage, no snow for 210 days and 35% volumetric soil moisture in all layers.

Run the program

unix% MakeModelStateBin InitialState.txt

This will create the Interception, Snow and Soil state files for the date specified in InitialState.txt

unix% mv *State* ../modelstate

# Session 2: Stream, Soil and Road Inputs

Go back to your plane window:

Compile required programs (fixroads.c and AddAat2.java)

plane% cd /tutorial/programs/

plane% gcc fixroads.c -lm -o fixroads

plane% javac AddAat2.java Aat.java AatError.java

plane% cd ../arcinfo/

plane% arc

Set the path for aml scripts:

Arc: &amlpath amlscripts/

Create stream and soil inputs

usage: STREAMNETWORK <dem> <wshed> <soildepth> <stream network> <MOUTH|MASK> {source area} {min depth} {max depth}

Arc: &run createstreamnetwork rainydem rainymask soild streams MASK 100000 .76 1.5

Note: all grids should have same domain and resolution

dem: a grid of basin elevations

wshed: EITHER a basin mask file, or a grid designating the location of the basin mouth, as indicated by the keyword MOUTH | MASK, if MOUTH is specified, the mask file will be created.

soild: grid of soil depth, if it doesn't exist it will be created as a function of cumulative drainage area and slope, min and max depth must then be specified.

streams: an arc coverage of stream locations, if it doesn't exist the coverage will be created using the specified constant threshold area (source area)

You will be prompted to see if you want to continue. Type y.

This step creates the files stream.network.dat and stream.map.dat that are needed by DHSVM.

Create road inputs

This step creates the files road.network.dat and road.map.dat that are needed by DHSVM.

usage: ROADNETWORK <dem> <soildepth> <road network>

Arc: &run createroadnetwork rainydem soild roads

You will be prompted to see if you want to continue. Type y.

Designate stream save indicator

Routing results for this stream segment will be placed in the stream output file if the keyword SAVE appears in the last column.  Add SAVE "RAINY CREEK" to the last column of Rainy Creek, stream segment 1.

unix% xemacs &

Export soildepth grid and convert to binary

Arc: gridascii soild soildepth.txt

Remove the header

On freebsd machine:

unix% cd ../arcinfo

unix% tail +7 soildepth.txt >! temp1

unix% rm soildepth.txt

unix% mv temp1 soildepth.txt

Convert to binary

unix% ../programs/myconvert ascii float soildepth.txt soildepth.bin 52 67

Create road and stream class files

These are ascii files that contain a look-up table of channel hydraulic properties  for each road or stream class.  The following columns are necessary for each channel class:

<Channel Class> <Hydraulic Width (m)> <Hydraulic Depth (m)> <Manning's friction coefficient> <Max Infiltration (m/s)>

The channel hydraulic width and depth are currently hard-wired in the createroadnetwork and createstreamnetwork scripts, as follows:

Roads:

#Class Width 	Depth

1	 0.5 	0.5

2	 0.5 	0.5

3	 0.25 	0.25

4	 0.25 	0.25

5	 0.25 	0.25

6	 0.25 	0.25

7	 0.25 	0.25

8	 0.25 	0.25

9	 0.25 	0.25

Streams:

#Class Width 	Depth

1	 1.5 	1.0

2	 2.0 	1.5

3	 3.0 	5.0

4	 5.0 	7.5

5	 2.0 	1.5

6	 5.0 	4.0

7	 7.5 	7.0

8	 10.0 	10.0

9	 15.0 	13.0

Edit these tables (road.class.dat and stream.class.dat) in emacs to add the friction and infiltration columns.

unix% xemacs &

The maximum infiltration rate is not used for the stream class file, but it must be specified as a place holder.  Manning's roughness can vary from 0.025 to 0.15 for natural channels.  Infiltration into the roads may be assumed to be 0 m/s for a starting point (i.e. 100% impervious).

Create initial channel state files

If necessary, make the script MakeChannelState.scr executable:

unix% chmod 755 MakeChannelState.scr

usage: MakeChannelState.scr <StreamNetworkFile> <InitialDepth> <Output Date String: MM.DD.YYYY.hh.mm.ss>

To run the script (calculates volume of water in each channel segment, given an assumed uniform initial depth in meters):

MakeChannelState.scr ../arcinfo/stream.network.dat 0.25 10.01.1990.03.00.00

unix% mv *State* ../modelstate

# Session 3: Running the model

Untar and compile the source code

unix% cd source/

unix% gunzip dhsvm_2.0.1.tar.gz

unix% tar -xvf dhsvm_2.0.1.tar

unix% cd dhsvm_2.0.1/

unix% make

Complete the configuration file

unix% xemacs &

Look through the file and fill out:

path names

Model Start  = 10/01/1990-03

Model End = 10/01/1994-03

Number of Model States: 1

Model State Date 1 = 10/01/1994-03

Run the model

unix% DHSVM ../../configfiles/INPUT.rainycr

# Session 4: Model Output and Calibration

Copy the model state files and backup previous output

unix% cd tutorial/

unix% cp output/*1994* modelstate/

unix% mv output output1

unix% mkdir output

unix% mv output1/GMTscripts/* output

Edit the configuration file

unix% xemacs &

Change:

Model Start  = 10/01/1994-03

Model End = 10/01/1996-03

Model State Date 1 = 10/01/1995-03

Fill out the Model Maps Section.  Refer to the handouts for the Map Variables. (Map 512 is not working.)

## Run the model

unix% cd source/dhsvm_2.0.1

unix% DHSVM ../../configfiles/INPUT.rainycr

Use GMT to view output hydrographs/basin average time series

unix% cd ../../output/GMTscripts

unix% plot_flow.scr	# Produces a graph of the Rainy Creek flow  (plot_flow.ps)

unix% ctrl-c <to close ghostview window>

unix% plot_precip.scr	# Produces a graph of precipitation over the basin (plot_precip.ps)

unix% plot_SWE.scr	# Produces a graph of snow water equivalent (plot_swe.ps)

Each of these can also be viewed in ghostscript, e.g.:

unix% gs plot_flow.ps

Use GMT or ArcInfo to view output spatial images

The script plot_modelmap.scr allows you to plot three types of maps, by changing the script slighty. It works if you designated only one date per map number. For each map you need to designate the map name, the name of the output file, the name of the input file and the name of the color plot table (cpt).  Create the Soil Moisture Map first.

unix% xemacs &

Edit plot_modelmap.scr so that the Soil Moisture map name, output file, input file and cpt file do not have a “#” in front of it.  Add a “#” in front of all the other map file names.  Save the file, but leave it open.

unix% plot_modelmap.scr

unix% ctrl-c <to close ghostview window>

Repeat these steps for the other 2 types of maps.

Each of these can also be viewed in ghostscript, e.g.:

unix% gs Map.1.Soil.Moist.ps

Experiment with adjusting calibration parameters and re-run

Open the configuration file in emacs and edit the parameters

unix% xemacs &

Save the previous output and rerun DHSVM

unix% mv output output2

unix% mkdir output

unix% cd ../../source/dhsvm_2.0.1

unix% DHSVM ../../configfiles/INPUT.rainycr

Run alternate road or vegetation scenario

No Roads:

Open configuration file in emacs and edit Road Network Section by commenting out the three path and file names (place a # before each Road).

unix% xemacs &

New Vegetation Scenario:

Open the rainyveg.txt in emacs and change all the vegetation to type 7 (Woodland Grass) or type 8 (Closed Shrub).

unix% xemacs &

Remove the header:

unix% cd /arcinfo

unix% tail +7 rainyveg.txt >! newrainyveg.txt

Convert to binary:

unix% /programs/myconvert ascii char newrainyveg.txt rainyveg.bin 52 67

Open the configuration file in emacs and edit the file names

unix% xemacs &

Save previous output and rerun DHSVM.

Give colleen locations of your final hydrographs

# Session 5: Sediment Model

Rerun DHSVM with sediment output turned on:

unix% xemacs configfiles/INPUT.rainycr &

Adjust:

Sediment = TRUE

Model Start  = 10/01/1995-03

Model End = 06/01/1996-03

Rerun DHSVM.  Saturated fraction and runoff files will now be output to your output directory.  Meanwhile:

Export 10 m dem  grid and convert to binary

Arc: gridascii rainy10m rainy10.txt

Remove the header

On freebsd machine:

unix% cd ../arcinfo

unix% tail +7 rainy10.txt >! temp1

unix% rm rainy10.txt

unix% mv temp1 rainy10.txt

Convert to binary

unix% ../programs/myconvert ascii float rainy10.txt rainy10.bin 52 67

Adjust the time period, input and output paths in tutorial/configfiles/INPUT.mwm in emacs

Model Start	=  11/27/1995-00     # Model start time (MM/DD/YYYY-HH)

Model End	=  11/30/1995-21     # Model end time (MM/DD/YYYY-HH)

Maximum Iterations	=  1              # Max iterations of 1 means deterministic mode

Dem File	= ../../arcinfo/rainy10.bin

Basin Mask File = ../../arcinfo/rainymask.bin

Soil Map File = ../../arcinfo/rainysoil.bin

Soil Depth File = ../../arcinfo/soildepth.bin

Soil Saturation Path  = ../../output/

Vegetation Map File = ../../arcinfo/rainyveg.bin

Output Directory     =  ../../output/

Untar and compile the source code

unix% cd source/

unix% gunzip mwm_0.99.tar.gz

unix% tar -xvf mwm_0.99.tar

unix% cd mwm/

unix% make

Run the model in deterministic mode

unix%: MWM ../../configfiles/INPUT.mwm

View output sediment map & failures

Sample plotting scripts similar to the ones used above are available in tutorial/output/GMTscripts.  The date to be plotted is currently set to 11/28/1995 in each of these scripts.  If desired, change the date by editing the script in xemacs to look for a different file.

unix% cd ../../output/GMTscripts

unix% Plot.SatFrac.scr	# Produces a map of the saturated soil fraction as output from DHSVM (Map.SatFrac.ps)

unix% Plot.FailureMap.scr	# Produces a map showing failure locations (Map.Failure.ps)

unix% Plot.FailureTrack.scr	# Produces a map showing change in soil depth due to failure (Map.Soildepth.ps)

Each of these can be viewed in ghostscript, e.g.:

unix% gs Map.Failure.ps	# Stochastic mode

Adjust mode in tutorial/configfiles/INPUT.mwm

Maximum Iterations	       =  100

## Run the model in stochastic mode

unix% MWM ../../configfiles/INPUT.mwm

View failure risk maps

More GMT scripts, same as above:

unix% Plot.Probability.scr	# Produces a map of the probability of landslide occurrence (Map.probability.ps)

unix% Plot.CumDepth.scr	# Produces a map showing track of all possible failures, as reflected in the change in soil depth (Map.cumdepth,ps)

unix% gs Map.probability.ps