U.S. Geological Survey                                modflow(1)

NAME

modflow - Modular three-dimensional finite-difference

ground-water model

ABSTRACT

MODFLOW is a three-dimensional finite-difference ground-

water model that was first published in 1984.  It has a modular

structure that allows it to be easily modified to adapt the

code for a particular application.  Many new capabilities have

been added to the original model.  OFR 00-92 (complete reference

below) documents a general update to MODFLOW, which is called

MODFLOW-2000 in order to distinguish it from earlier versions.

MODFLOW-2000 simulates steady and nonsteady flow in an

irregularly shaped flow system in which aquifer layers can

be confined, unconfined, or a combination of confined and

unconfined.  Flow from external stresses, such as flow to

wells, areal recharge, evapotranspiration, flow to drains,

and flow through river beds, can be simulated.  Hydraulic

conductivities or transmissivities for any layer may differ

spatially and be anisotropic (restricted to having the

principal directions aligned with the grid axes), and the

storage coefficient may be heterogeneous.  Specified head

and specified flux boundaries can be simulated as can a head

dependent flux across the model's outer boundary that allows

water to be supplied to a boundary block in the modeled area

at a rate proportional to the current head difference

between a "source" of water outside the modeled area and the

boundary block.  MODFLOW is currently the most used

numerical model in the U.S. Geological Survey for ground-

water flow problems.

In addition to simulating ground-water flow, the scope of

MODFLOW-2000 has been expanded to incorporate related

capabilities such as solute transport and parameter estimation.

METHOD

The ground-water flow equation is solved using the finite-

difference approximation.  The flow region is subdivided

into blocks in which the medium properties are assumed

to be uniform.  In plan view the blocks are made from

a grid of mutually perpendicular lines that may be

variably spaced.  Model layers can have varying thickness.

A flow equation is written for each block, called a cell.

Several solvers are provided for solving the resulting matrix

problem; the user can choose the best solver for the particular

problem.  Flow-rate and cumulative-volume balances from each

type of inflow and outflow are computed for each time step.

HISTORY

MODFLOW-2000 Version 1.19.01 2010/03/25 - This release corrects a

minor bug in the GMG Package, and the Unix distribution includes

a revised openspec.inc file to correct a problem introduced at

version 1.19.00.

MODFLOW-2000 Version 1.19.00 2010/03/15 - This release includes the

revised Multi-Node Well (MNW2) Package, and other changes and bug

fixes, as described in release.txt.

MODFLOW-2000 Version 1.18.01 2008/06/20 - This release includes bug

fixes and minor changes.  Bug fixes and other changes are described

in release.txt.

MODFLOW-2000 Version 1.18.00 2007/08/23 - Added the SWT Package to

support simulation of subsidence in water-table aquifers.  It also

includes bug fixes.

MODFLOW-2000 Version 1.17.04 2007/08/03 - This release includes bug

fixes and minor changes.  Bug fixes and other changes are described

in release.txt.

MODFLOW-2000 Version 1.17.02 2007/02/14 - This release includes bug

fixes and minor changes.  Bug fixes and other changes are described

in release.txt.

MODFLOW-2000 Version 1.17.01 2006/09/22 - This is a bug-fix release.

Bug fixes are described in release.txt.

MODFLOW-2000 Version 1.17.00 2006/07/25 - The LAK3, SFR2, and GAG5

Packages have been updated to support simulation of new

capabilities.  The GMG Package has been updated to support a new

adaptive-damping option.  This version also includes a change in the

SUB Package and fixes for bugs in the ADV, MNW, and DRT Packages.

Please see release.txt and files referenced therein for details on

the new options and changes.

MODFLOW-2000 Version 1.16.00 2006/03/28 - The SFR2 Package replaces

the SFR1 Package of version 1.14.00, adding support for simulation

of unsaturated flow beneath streams.  The ADV2 Package has been

made more compatible with MODPATH.  Other changes and bug fixes

are described in release.txt.

MODFLOW-2000 Version 1.15.01 2005/04/05 - This is a bug-fix release.

Bug fixes are described in release.txt.

MODFLOW-2000 Version 1.15.00 2004/08/06 - Added the GMG Solver Package.

Modifications were made to the SFR Package.

MODFLOW-2000 Version 1.14.00 2004/07/01 - Added the SFR Package to

allow enhanced capabilities for simulation of streams.  The maximum

number of layers is increased from 200 to 999.

MODFLOW-2000 Version 1.13.00 2004/01/22 - Includes revisions to the

LAK3 Package and bug fixes for the DRT, SUB, HUF2, and PCG2

Packages.

MODFLOW-2000 Version 1.12.01 2003/10/03 - This is a bug-fix release.

Bug fixes are described in release.txt.

MODFLOW-2000 Version 1.12 2003/09/08 - Added the SUB Package to allow

simulation of subsidence and aquifer-system compaction.  The HUF

Package was upgraded from version 1 to version 2, providing support

for the new Layer Variable-Direction Horizontal Anisotropy (LVDA)

capability, an alternative specification of storage properties for

the uppermost active cells, calculation of flow in  hydrogeologic

units, and the new Hydraulic-Conductivity Depth-Decay (KDEP)

capability.  The ADV Package was updated from version 2.3 to version

2.4, which includes support for porosity parameters.  Additional

enhancements and bug fixes are described in release.txt.

MODFLOW-2000 Version 1.11 2003/04/10 - Added the DAF Package to allow

coupling of MODFLOW with the Diffusion Analogy Surface-Water Flow

Model, and the MNW Package to allow simulation of well completed in

intervals represented by multiple model cells.  New options for

showing progress of the simulation and controlling the calculation

of drawdowns have been added. Modifications to the PCG2 Package

affect the way DAMP works.  Please see release.txt for information

on using these new packages, new options, and DAMP.  This version

also includes a number of bug fixes and minor output changes, also

described in release.txt.

MODFLOW-2000 Version 1.10 2002/07/26 - Fixes bugs encountered when

compiling the source code distributed with version 1.9 and a

problem in a seldom-used part of the LMG Package.

MODFLOW-2000 Version 1.9 2002/07/15 - Updates were made to the LAK3,

GAGE5, and HUF1 Packages: (1) The LAK Package was modified to add

compatibility with the LPF and HUF packages and MODPATH, to clarify

output, and report cumulative lake budget; (2) Modifications to the

GAGE Package allow optional additional output related to lake gages;

and (3) A problem related to the calculation of horizontal-

conductance sensitivities was fixed in the HUF Package.  Some

problems related to omission of observations (from the sum of

squared, weighted residuals objective function and other statistics

and the underscore ("_") output files) were corrected.  Please see

the Release Notes in the file release.txt for more information

related to these issues.

MODFLOW-2000 Version 1.8 2002/05/01 - Added support for an option

that can be used to suppress printing of cell lists in any or all

of the RIV, GHB, DRN, DRT, HFB, WEL, and CHD Packages.  This

version also includes fixes to (1) problems in the ADV Package

related to stagnant particles and incorrect sensitivities for

observation locations in confining units; (2) a bug that caused an

invalid value to be put into a hydraulic-property array when the

OBS Process was active and the PES Process was inactive; (3)

problems in LPF and HUF related to the sensitivity of vertical

conductance to head in the calculation of sensitivities for

unconfined cells; (4) a problem in HUF related to the rewetting

capability; (5) a problem in LMT related to use of the STR Package

and and the Extended-header option of LMT; and (6) problems in in

the RESAN2K post-processing program that affected ordering of

observation names and plot symbols.

MODFLOW-2000 Version 1.7 2001/12/04 - Added support for time-varying

parameters.  Time-varying parameters are useful when the user

desires to estimate one parameter but vary properties included in

the parameter definition from one stress period to another.

Please see files Time-varying-parameters.pdf and str6.pdf in the

doc directory for details.

MODFLOW-2000 Version 1.6 2001/10/19 - Fixed two problems.  The first

caused simulation time to be truncated, for runs that involve

calculation of sensitivities, to a simulation period one time step

too short when the last flow observation in a transient simulation

falls in a later time step than the final head observation.  The

second problem caused observation sensitivities for constant-head

flow observations to be incorrect for log-transformed parameters.

In addition, support for new options for the name file was added to

allow users to designate files of type DATA, DATA(BINARY), DATAGLO,

or DATAGLO(BINARY) as either input or output files.  Please see the

Release Notes in the file release.txt for a description of these

options.

MODFLOW-2000 Version 1.5 2001/08/16 - Added the LMT Package to

support generation of the link file required by the contaminant-

transport program MT3DMS.

MODFLOW-2000 Version 1.4 2001/07/10 - Added the LMG Package for

solving matrix equations using an algebraic multigrid solver.

MODFLOW-2000 Version 1.3 2001/06/11 - Fixed problem in LPF where if

a lower cell goes dry while the cell above is still wet, there

is a non-zero vertical conductance to a dry cell, causing

calculated heads and flows to be in error.

MODFLOW-2000 Version 1.2 2001/04/12 - Added support for using

vendor-specific options for opening unformatted (binary)

files.  On personal computers, this can make it possible

for programs compiled by different compilers to write files

that have the same structure.  The formula used to calculate

dimensionless, scaled sensitivities of simulated equivalents

to observations for log-transformed parameters is changed.

Other changes include bug fixes and clarification of output.

The MODFLOW-2000 distribution now includes Release Notes in

the file release.txt in the doc directory -- this file

describes in detail changes to MODFLOW-2000 for each version.

MODFLOW-2000 Version 1.1 2001/01/17 - Added IBS, HUF, LAK, ETS,

and DRT Packages.  Added HYDMOD option.  The EVT Package was

modified as described in the LAK documentation to include

the option (NEVTOP=3) to apply ET to the uppermost wet cell

in a vertical column of cells as done in the RCH Package.

MODFLOW-2000 Version 1.0 2000/07/20 - Enhanced modular structure

that facilitates the addition of broader capabilities such as

parameter estimation and solute transport.  New data input

methods are included to support parameter estimation.

Vertical cell dimensions are specified in a separate

discretization file.  A new package called the LPF Package

has been added as an alternative to the BCF Package.

The MODFLOW-2000 documentation report (OFR 00-92) does not

include the STR Package; however, this code has been upgraded

to use parameters and is included in this version of

MODFLOW-2000.  Revised STR input instructions can be found

in file str6.pdf, which is included in the doc directory that

is part of the distribution for MODFLOW-2000.

The ADV Package has been modified for use with MODFLOW-2000.

Changes in input instructions are contained in file adv.txt,

which is also in the doc distribution directory.

The RES and FHB Packages are included in MODFLOW-2000 without

modification. These will work as before, but they do not

incorporate parameters or observations.

The IBS, TLK, and GFD Packages are not included in this

release. They require modification in order to work within

MODFLOW-2000.

Note that MODFLOW-2000 sends some messages to the computer

display.  Some Fortran compilers make use of a reserved file

unit, for example unit 6, for output to the display.  If such

compilers are used, it is important to avoid using a reserved

unit for any files in the MODFLOW name file.

MODFLOW-96 Version 3.3 2000/05/02 - Error fixed in IBS code that

caused incorrect values for critical head to be written when

the option is invoked to save these values in an external

file (variable IHCSV>0).  The error did not affect the

actual interbed storage calculations, and none of the

other model results, such as computed head and budget

flows, are affected by the correction. There are no

changes to any other packages.

MODFLOW-96 Version 3.2 1998/01/09 - The Flow and Head Boundary Package,

Version 1, (FHB1) has been added. This package allows for

assignment of transient specified-flow and specified-head

boundaries in applications of MODFLOW-96.  The FHB1

package is documented in U.S. Geological Survey Open-File

Report 97-571.

MODFLOW-96 Version 3.1 1997/03/11 - Calls to the Horizontal Flow

Barrier (HFB) Package have been corrected in the main

program.  The main program for Version 3.0 incorrectly

calls the HFB1RP module where it should call the HFB1FM

module.

MODFLOW-96 Version 3.0 1996/12/03 - Updated version of overall model,

which is called MODFLOW-96.  A number of changes were

made to make MODFLOW easier to use and easier to enhance.

MODFLOW-96 can use existing input data sets, and has the

same computational methods.  Any package added to the

original model (now referred to as MODFLOW-88) will also

work with MODFLOW-96.

The Preconditioned Conjugate Gradient Package, Version 2

(PCG2) has been updated.  This requires a minor change in

input data compared to the documentation in U.S.

Geological Survey Water-Resources Investigations Report

(WRIR) 90-4048 (full reference below).  WRIR 90-4048

specifies two lines of input data.  All of the data are

the same except the seventh value on the 2nd line.  This

value is named IPCGCD in WRIR-4048, but it has been

replaced by a value named DAMP.  That is, IPCGCD is no

longer part of the input data.  DAMP can be used to

reduce oscillation when the solver is having difficulty

converging due to excessive oscillation.  The value of

DAMP is multiplied times the head change calculated each

iteration at all cells.  Thus, if DAMP is 0.5, the head

change is cut in half.  If DAMP is 1.0, then PCG2 behaves

as it did prior to the addition of this capability.  DAMP

should be set equal to 1.0 except when there is

indication of excessive oscillation.  If the value of

DAMP is specified as 0.0 or less, it is automatically

changed to 1.0.

Also the sign of the C.B. STORAGE cell-by-cell budget

data in the TLK1 Package was changed to match the

standard sign convention in MODFLOW.  The change to TLK1

does not impact computed heads or the overall volumetric

budget; it only affects data written to a cell-by-cell

budget file.

MODFLOW-88 Version 2.6 1996/09/20 - Added Reservoir package (RES1) as

documented in U.S. Geological Survey Open-File Report

96-364.  Problem fixed for IBS package.  Although

subsidence is only meant to be active for layers in which

IBQ>0, sometimes MODFLOW performed subsidence

calculations when IBQ<0.  Note that this was a problem

only if negative IBQ values were specified.  That is, the

code has always worked correctly for IBQ=0 and IBQ>0.

MODFLOW-88 Version 2.5 1995/06/23 - Added direct solution package

(DE45).

MODFLOW-88 Version 2.4 1995/06/15 - Added transient leakage package

(TLK1).

MODFLOW-88 Version 93/08/30 - Release with PCG2, BCF3, STR1, HFB1,

ISB1, CHD1, and GFD1 additions.

MODFLOW-88 Version 87/07/24 - Fortran 77 version published in U.S.

Geological Survey Techniques of Water-Resources

Investigations 6-A1.

MODFLOW Version 83/12/28 - Fortran 66 version published in U.S.

Geological Survey Open-File Report 83-875.

DATA REQUIREMENTS

In order to use MODFLOW, initial conditions, hydraulic

properties, and stresses must be specified for every model

cell in the finite-difference grid.

OUTPUT OPTIONS

Primary output is head, which can be written to the listing

file or into a separate file.  Other output includes the

complete listing of all input data, drawdown, and budget

data.  Budget data are printed as a summary in the listing

file, and detailed budget data for all model cells can be

written into a separate file.

SYSTEM REQUIREMENTS

MODFLOW-2000 generally is written in Fortran 77 with the

following extensions:  use of variable names longer than 6

characters, Fortran 90 statements for dynamic memory

allocation, a call to SUBROUTINE GETARG to retrieve command-

line arguments, and two calls to DATE_AND_TIME, which is a

Fortran 90 intrinsic subroutine. Generally, the program is

easily installed on most computer systems.  The code has

been used on UNIX-based computers and DOS-based 386 or

greater computers having a math coprocessor and 4 mb of

memory.

Starting with version 1.12, a Fortran-90 or later compiler

is required to compile one or more of the source-code files,

including gwf1sub1.f.

PROCESSES

This version of MODFLOW includes the following processes:

GWF1 -- Ground-Water Flow Process

SEN1 -- Sensitivity Process

OBS1 -- Observation Process

PES1 -- Parameter-Estimation Process

PACKAGES

This version of MODFLOW includes the following packages:

BAS6 -- Basic Package

BCF6 -- Block-Centered Flow Package

LPF1 -- Layer-Property Flow Package

RIV6 -- River Package

DRN6 -- Drain Package

WEL6 -- Well Package

GHB6 -- General Head Boundary Package

RCH6 -- Recharge Package

EVT6 -- Evapotranspiration Package

CHD6 -- Time-Variant Specified-Head Package

HFB6 -- Horizontal Flow Barrier Package

SIP5 -- Strongly Implicit Procedure Package

SOR5 -- Slice Successive Over-Relaxation Package

PCG2 -- Version 2 of Preconditioned Conjugate Gradient

Package

DE45 -- Direct solver

LMG1 -- Multigrid solver (for USGS use only)

STR6 -- Streamflow-Routing Package

ADV2 -- Advective-Transport Observation Package

RES1 -- Reservoir Package (RES is the file type in the

MODFLOW name file)

FHB1 -- Flow and Head Boundary Package (FHB is the file

type in the MODFLOW name file)

IBS6 -- Interbed Storage (subsidence) Package (IBS is the

file type in the name file)

HUF2 -- Hydrogeologic-Unit Flow Package

LAK3 -- Lake Package

ETS1 -- Evapotranspiration with a Segmented Function Package

DRT1 -- Drains with Return Flow Package

LMT6 -- Link to MT3DMS contaminant-transport model

MNW1 -- Multi-Node Well Package

DAF1 -- Diffusion Analogy Surface-Water Flow Package

SUB1 -- Subsidence and Aquifer-System Compaction Package

SFR2 -- Stream-Flow Routing Package, version 2

GMG1 -- Geometric MultiGrid Solver Package

SWT1 -- Subsidence and Aquifer-System Compaction Package

for Water-Table Aquifers

MNW2 -- Revised Multi-Node Well Package

ADDITIONAL CAPABILITIES:

HYDMOD -- Hydrograph option

GAGE -- Hydrograph option for lakes (LAK3 Package) and streams

(SFR Package)

DOCUMENTATION

The basic documentation is contained in the following five

reports:

Harbaugh, A.W., Banta, E.R., Hill, M.C., and McDonald, M.G., 2000,

MODFLOW-2000, the U.S. Geological Survey modular ground-water model --

User guide to modularization concepts and the Ground-Water Flow

Process: U.S. Geological Survey Open-File Report 00-92, 121 p.

Hill, M.C., Banta, E.R., Harbaugh, A.W., and Anderman, E.R., 2000,

MODFLOW-2000, the U.S. Geological Survey modular ground-water model --

User guide to the Observation, Sensitivity, and Parameter-Estimation

Processes and three post-processing programs: U.S. Geological Survey

Open-File Report 00-184, 210 p.

Harbaugh, A.W., and McDonald, M.G., 1996, User's

documentation for MODFLOW-96, an update to the U.S.

Geological Survey modular finite-difference ground-water

flow model: U.S. Geological Survey Open-File Report

96-485, 56 p.

Harbaugh, A.W., and McDonald, M.G., 1996, Programmer's

documentation for MODFLOW-96, an update to the U.S.

Geological Survey modular finite-difference ground-water

flow model: U.S. Geological Survey Open-File Report

96-486, 220 p.

McDonald, M.G., and Harbaugh, A.W., 1988, A modular three-

dimensional finite-difference ground-water flow model:

U.S. Geological Survey Techniques of Water-Resources

Investigations, book 6, chap. A1, 586 p.

The BCF6 code is documented in several places.  It includes

the capabilities of BCF1, BCF2, and BCF3 Packages.  The

primary documentation is in the basic model documentation

(TWRI 6-A1, OFR 96-485, and OFR 96-486).

BCF2 documentation describes the addition of the capability

to convert dry cells to wet:

McDonald, M.G., Harbaugh, A.W., Orr, B.R., and Ackerman,

D.J., 1992, A method of converting no-flow cells to

variable-head cells for the U.S. Geological Survey

modular finite-difference ground-water flow model: U.S.

Geological Survey Open-File Report 91-536, 99 p.

BCF3 documentation describes the addition of alternate

interblock transmissivities.

Goode, D.J., and Appel, C.E., 1992, Finite-difference

interblock transmissivity for unconfined aquifers and for

aquifers having smoothly varying transmissivity: U.S.

Geological Survey Water-Resources Investigations Report

92-4124, 79 p.

Version 2 of Preconditioned Conjugate Gradient Package is

documented in:

Hill, M.C., 1990, Preconditioned conjugate-gradient 2

(PCG2), a computer program for solving ground-water flow

equations: U.S. Geological Survey Water-Resources

Investigations Report 90-4048, 43 p.

The Streamflow-Routing (STR) Package is documented in:

Prudic, D.E., 1989, Documentation of a computer program to

simulate stream-aquifer relations using a modular,

finite-difference, ground-water flow model: U.S.

Geological Survey Open-File Report 88-729, 113 p.

The Time-Variant Specified-Head Package is documented in:

Leake, S.A., and Prudic, D.E., 1991, Documentation of a

computer program to simulate aquifer-system compaction

using the modular finite-difference ground-water flow

model:  U.S. Geological Survey Techniques of Water-

Resources Investigations, book 6, chap. A2, 68 p.

The Horizontal-Flow Barrier Package is documented in:

Hsieh, P.A., and Freckleton, J.R., 1993, Documentation of a

computer program to simulate horizontal-flow barriers

using the U.S. Geological Survey modular three-

dimensional finite-difference ground-water flow model:

U.S. Geological Survey Open-File Report 92-477, 32 p.

The DE45 Package is documented in:

Harbaugh, A.W., 1995, Direct solution package based on

alternating diagonal ordering for the U.S. Geological

Survey modular finite-difference ground-water flow model:

U.S. Geological Survey Open-File Report 95-288, 46 p.

The RES1 Package is documented in:

Fenske, J.P., Leake, S.A., and Prudic, D.E., 1996,

Documentation of a computer program (RES1) to simulate

leakage from reservoirs using the modular finite-

difference ground-water flow model (MODFLOW): U.S.

Geological Survey Open-File Report 96-364, 51 p.

The FHB1 Package is documented in:

Leake, S.A., and Lilly, M.R., 1997, Documentation of a

computer program (FHB1) for assignment of transient

specified-flow and specified-head boundaries in

applications of the modular finite- difference ground-

water flow model (MODFLOW): U.S. Geological Survey Open-

File Report 97-571, 50 p.

The ADV Package is documented in:

Anderman, E.R. and Hill, M.C., 1997, Advective-transport

observation (ADV) package, a computer program for adding

advective-transport observations of steady-state flow

fields to the three-dimensional ground-water flow

parameter-estimation model MODFLOWP: U.S. Geological

Survey Open-File Report 97-14, 67 p.

Anderman, E.R. and Hill, M.C., 2001, MODFLOW-2000, the U.S.

Geological Survey modular ground-water model -- Documentation

of the  Advective-Transport Observation (ADV2) Package,

version 2: U.S. Geological Survey Open-File Report 01-54,

69 p.

File MODPATH-ADV_compatibility_3-2006.pdf, which is included

in the doc directory of the MODFLOW-2000 distribution.

HYDMOD is documented in:

Hanson, R.T. and Leake, S.A., 1999, Documentation of HYDMOD,

a program for extracting and processing time-series data

from the U.S. Geological Survey's modular three-dimensional

finite-difference ground-water flow model: U.S. Geological

Survey Open-File Report 98-564, 57 p.

The IBS Package is documented in:

Leake, S.A. and Prudic, D.E., 1991, Documentation of a

computer program to simulate aquifer-system compaction using

the modular finite-difference ground-water flow model: U.S.

Geological Survey Techniques of Water-Resources

Investigations, Book 6, Chapter A2, 68 p.

The HUF Package is documented in:

Anderman, E.R., and Hill, M.C., 2000, MODFLOW-2000, the U.S.

Geological Survey modular ground-water model -- Documentation

of the Hydrogeologic-Unit Flow (HUF) Package: U.S. Geological

Survey Open-File Report 00-342, 89 p.

Anderman, E.R., Kipp, K.L., Hill, M.C., Valstar, Johan, and

Neupauer, R.M., 2002, MODFLOW-2000, the U.S. Geological Survey

modular ground-water model -- Documentation of the Model-Layer

Variable-Direction Horizontal Anisotropy (LVDA) capability of

the Hydrogeologic-Unit Flow (HUF) Package: U.S. Geological

Survey Open-File Report 02-409, 60 p.

Anderman, E.R., and Hill, M.C., 2003, MODFLOW-2000, the U.S.

Geological Survey modular ground-water model -- Three additions

to the Hydrogeologic-Unit Flow (HUF) Package: Alternative storage

for the uppermost active cells, Flows in hydrogeologic units, and

the Hydraulic-coductivity depth-dependence (KDEP) capability:

U.S. Geological Survey Open-File Report 03-347, 36 p.

The LAK Package is documented in:

Merritt, L.M. and Konikow, L.F., 2000, Documentation of a

computer program to simulate lake-aquifer interaction using

the MODFLOW ground-water flow model and the MOC3D

solute-transport model:  U.S. Geological Survey Water-Resources

Investigations Report 00-4167, 146 p.

The ETS and DRT Packages are documented in:

Banta, E.R., 2000, MODFLOW-2000, the U.S. Geological Survey

modular ground-water model -- Documentation of packages for

simulating evapotranspiration with a segmented function (ETS1)

and drains with return flow (DRT1): U.S. Geological Survey

Open-File Report 00-466, 127 p.

The LMG Package is documented in:

Mehl, S.E. and Hill, M.C., MODFLOW-2000, the U.S. Geological

Survey modular ground-water model -- User guide to the LINK-AMG

(LMG) Package for solving matrix equations using an algebraic

multigrid solver: U.S. Geological Survey Open-File Report 01-177,

33 p.

The LMT Package is documented in:

Zheng, Chunmiao, Hill, M.C., and Hsieh, P.A., 2001, MODFLOW-2000,

the U.S. Geological Survey modular ground-water model - User

guide to the LMT6 package, the linkage with MT3DMS for multi-

species mass transport modeling: U.S. Geological Survey Open-File

Report 01-82, 43 p.

The MNW1 Package is documented in:

Halford, K.J. and Hanson, R.T., 2002, User guide for the drawdown-

limited, multi-node well (MNW) package for the U.S. Geological

Survey's modular three-dimensional finite-difference ground-water

flow model, versions MODFLOW-96 and MODFLOW-2000: U.S. Geological

Survey Open-File Report 02-293, 33 p.

The MNW2 Package is documented in:

Konikow, L.F., Hornberger, G.Z., Halford, K.J., and Hanson, R.T.,

2009, Revised multi-node well (MNW2) package for MODFLOW ground-

water flow model: U.S. Geological Survey Techniques and Methods

6-A30, 67 p., available at: http://pubs.usgs.gov/tm/tm6a30/

The DAF Package is documented in:

Jobson, H.E. and Harbaugh, A.W., 1999, Modifications to the

diffusion analogy surface-water flow model (DAFLOW) for coupling

to the moddular finite-difference ground-water flow model (MODFLOW):

U.S. Geological Survey Open-File Report 99-217, 107 p.

The SUB Package is documented in:

Hoffmann, J., Leake, S.A., Galloway, D.L., and Wilson, A.M., 2003,

MODFLOW-2000 Ground-Water Model--User Guide to the Subsidence and

Aquifer-System Compaction (SUB) Package: U.S. Geological Survey

Open-File Report 03-233, 44 p.

The SFR Package is documented in:

Prudic, D.E., Konikow, L.F., and Banta, E.R., 2004, A new stream-

flow routing (SFR1) package to simulate stream-aquifer interaction

with MODFLOW-2000: U.S. Geological Survey Open-File Report

2004-1042, 95 p.

Niswonger, R.G. and Prudic, D.E., 2005, Documentation of the

Streamflow-Routing (SFR2) Package to include unsaturated flow

beneath streams--A modification to SFR1: U.S. Geological Survey

Techniques and Methods, Book 6, Chap. A13, 47 p.

The GMG Package is documented in:

Wilson, J.D. and Naff, R.L., 2004, The U.S. Geological Survey modular

ground-water model -- GMG linear equation solver package

documentation:  U.S. Geological Survey Open-File Report 2004-1261,

47 p.  <http://pubs.water.usgs.gov/ofr2004-1261/>

The SWT Package is documented in:

Leake, S.A., and Galloway, D.L., 2007, MODFLOW ground-water model --

User guide to the Subsidence and Aquifer-System Compaction Package

(SUB-WT) for Water-Table Aquifers: U.S. Geological Survey Techniques

and Methods, Book 6, Chap. A23, 42 p.

<http://pubs.usgs.gov/tm/2007/06A23/pdf/TM6a23.pdf>

CONTACTS

Operation:

U.S. Geological Survey

Office of Ground Water

Arlen Harbaugh

411 National Center

Reston, VA 20192

harbaugh@usgs.gov

The USGS MODFLOW fact sheet (a short description of the MODFLOW

model) can be retrieved using the World-Wide Web at site:

http://water.usgs.gov/public/pubs/FS/FS-121-97/

See

http://water.usgs.gov/software/ordering_documentation.html

for information on ordering printed copies of USGS publications.

SEE ALSO

modpath(1) - Particle-tracking postprocessor program for the

modular three-dimensional finite-difference

ground-water flow model

zonebudget(1) -- Program for computing subregional water budgets

for MODFLOW ground-water flow models