CNT Tool

(Draft Copy)

April 30th, 2010

The CNT Tool is used for deriving Digital Elevation Model (DEM), Drainage Direction Map (DDM) and Flow Accumulation Map (FAM) from Shuttle Research Topography Mission data for a specified area and grid cell size. The results from this program and then directly usable in the CREST hydrologic model for defining a modeling area.

1.

2.

3.

4.

5.

6.

7.

8.

9.

10.

11.

## Configuration file

You can select an already made configuration file or specify a nonexistent file "something.txt" name here. Something must be specified in this location or the tool will crash! This is also the name that will be given to created DEM, DDM and FAM files. In this example the output will be called "conus.dem.txt", "conus.dire.txt" and "conus.acc.txt". These files should be renamed to "DEM.txt", "DDM.txt" and "FAM.txt" respectively for use in the CREST model.

## Source Data Folder

This is the location of the SRTM files such as "na.dem", "na.dire" and "na.acc".

## Exclusion Points List

To be described in the future.

## Domain North

The location in decimal degrees of the northern edge of the domain. North is positive, south negative.

## Domain East

The location of the eastern edge of the domain in decimal degrees. East is positive, west negative.

## Domain South

The location of the southern edge of the domain in decimal degrees. North is positive, south negative.

## Domain West

The location of the western edge of the domain in decimal degrees. East is positive, west negative.

## Domain Cell Size

The cell size to use for the domain. Specified in decimal degrees. Example values include 0.01 which is about 1 km2 everywhere in the globe (subject to distortion due to longitude convergence near the poles).

## No Data Value

This is the no data value to be used in the resulting Arc ASCII grid files.

## Threshold

To be described in the future. Suggested default value is 6.

## Quit Time

To be described in the future.

## Running the CNT Tool

Once all of the fields listed above have been filled out with appropriate values push the "upgrade" button this will start the process of deriving the needed grids. This process can take a long time (on the order of days) depending on the area and cell size for which grids are being derived.