泥石流运动要素观测说明

泥石流运动要素共包括三张数据表：①泥石流运动要素数据表；②泥石流运动要素统计数据表；③泥石流运动要素总体数据表。泥石流运动要素数据表(编号××××)中的序号、流态、龙头时间、龙尾时间、泥面宽、泥深、测速距离、测速时间、容重等为原始观测数据，其他数据由计算得到。泥石流运动要素统计数据表中的数据、泥石流运动要素总体数据表中的数据，由计算统计得到。数据要素说明如下：

- 泥石流运动要素数据表

（1）编号 Code

每场泥石流以四位数字的惟一编号标识，前四位表示年，后两位表示该年场次。如198913表示1989年第13场泥石流。

（2）序号 No.

一场泥石流中各阵的统计编号。

（3）流态 Type

分为阵性流S、连续流C与缺测I。当阵与阵之间持续流量较大，或一阵持续时间很长则视为连续流C；当阵与阵之间明显断流时则为阵性流S；缺测Ⅰ表示该阵没有观测数据，不能参加计算和统计。

（4）龙头时间 T₁ (h: min: s)

龙头到达测速起始观测断面的时间。如4:27:08 表示凌晨4 点 27 分8 秒。所有时间以24 小时制表示。对于连续流为连续流开始的时间。

（5）龙尾时间 T₂ (h: min: s)

龙尾到达测速终点观测断面的时间。对于连续流为连续流结束的时间。表示方法同龙头时间。

（6）历时 T (s)

阵性流历时T=T2(龙尾时间)-T1(龙头时间)

（7）泥面宽 B (m)

泥石流流动时龙头的表面宽度。根据泥石流暴发前的断面观测资料及泥位高低确定。

（8）泥深H (m)

泥石流流动时龙头的深度。泥石流泥深的测量是利用系在绞车上的测绳，先后读出一阵泥石流经过观测断面前及流经断面时的读数，两读数之差即为实测泥深值。有些阵次系参照实测泥深、沟槽断面测量结果、断面标记及泥位高低人工估计确定。

（9）测速距离 L (m)

事先选取测定长度的顺直沟段并标记用于测量泥石流龙头流速及历时。

（10）测速时间 t (s)

用秒表计录的龙头经过选取的测速距离的时间，即龙头到达测速终点断面与测速起点断面的时间差。

（11）流速 V (m/s)

V=Lt

式中，L为测速距离(m)；t为测速时间(s)。

（12）流量 Q (m³/s)

Q=V×B×H

式中，V为流速(m/s); B为泥面宽(m); H为泥深(m)。

（13）容重 γc（t/m3）

进行了容重取样的阵次，以所取样品容重为该阵的容重，没有取样的阵用该阵附近所取样品的容重或某时段内所取样品容重的平均容重值。

（14）径流量 Wc (m³)

Wc=Q×T2    阵性流

Wc=Q×T  连续流

式中，Q 为流量(m3/s)；T为历时(s)。

（15）输沙量 Ws (m³)

Ws=Wc×Cv   Cv=γc-γγs-γ

式中，Wc为径流量(m³)；Cv为泥石流的体积比含沙量；γ、γc、γs分别为清水、泥石流、固体物质三者的容重(t/m³)，其中γs取 2750kg/m3 [参考泥石流物理力学性质统计表（1966年）中实测值]

（16）备注 Note

该栏凡注有“sam”，均进行了容重取样。

2. 泥石流运动要素统计数据表

本表中数据主要对连续流和阵性流分别统计，缺阵仅统计阵流数。某项特征值的平均值为连续流或阵性流所有阵次的该项特征值之和除以各自的阵次总数。最大值为连续流或阵性流所有阵次的特征值中的最大值。这里仅对含沙量和输沙率作一说明，其余栏目已在前文中说明。

（1）含沙量 Sm (kg/m3)

Sm=γs×Cv

式中，γs为固体物质的比重，取2750 kg/m³；Cv为泥石流的体积比含沙量。

（2）输沙率 Qc (t/s)

Qc=Q×S1000   S=γs×Cv   Cv=γc-γγs-γ

式中， Q为流量(m³/s)； S 为含沙量(kg/m³)。

3. 泥石流运动要素总体数据表

本表数据主要对本场泥石流所有阵次(包括连续流和阵性流，不包括缺测)的统计数据。这里仅对前文中未加说明的栏目加以说明。

（1）开始时间 Start time (Y/M/D/h/m)

该场泥石流的第一阵龙头达到观测断面的时间或开始观测的时间，依次表示：年/月/日/时/分。

（2）终止时间 End time (Y/M/D/h/m)

该场泥石流的最后一阵龙尾达到观测断面的时间或结束观测的时间，依次表示：年/月/日/时/分。

（3）历时 Duration (h: min: s)

历时：终止时间-开始时间。

（4）沟床纵比降 J

用经纬仪或手提水准仪实测的沟床纵坡。

泥石流运动要素观测方法

泥石流观测断面一般位于东川站2号观测楼附近顺直沟道内，由于泥石流沟道会改道，观测断面并不完全固定，近年来泥石流沟道观测断面位于蒋家沟鸡冠山附近，具体位置见下图：

泥石流泥深根据沟岸高度标记人工确定，泥面宽度根据沟道宽度对比得到，具体观测细节见下图。部分泥石流容重为铅鱼采样或人工采样烘干计算得到，部分容重值为基于泥石流流态人工确定。1999至2001年同时采用超声波泥位计进行泥位观测，观测数据见另表。

泥石流观测细节说明

备注：

（1）请在论文发表、专利申请、专著出版等工作中标注数据来源，并在公开发表的中文出版物的致谢部分标明“感谢中国科学院东川泥石流观测研究站为本研究提供了相关数据”，在英文论文致谢部分标明“Dongchuan Debris Flow Observation and Research Station (DDFORS), Chinese Academy of Sciences, which provided the field observation data for this study.”.

（2）更多详细信息见东川站网页（中文网站http://nsl.imde.ac.cn/；英文网站http://nsl.imde.ac.cn/en/）.

（3）数据问题可联系魏丽（weili@imde.ac.cn）和宋东日（drsong@imde.ac.cn）.

Remarks on Data Collection, Calculation and Nomenclatures

The data for each debris flow regularly observed include three tables: ① Data of debris flow kinematic observation; ② Statistical data of debris flow kinematic observation; ③ General data of debris flow kinematic observation. There is also a general data table for each year's debris flow kinematic observation at the end of each year's observation data. The surge number, flow type, front time, rear time, surface breadth, depth, velocity measuring distance, velocity measuring time, and unit weight are original observation data items. Other data items are calculated based on the original observation data. The data items in each table are as follows.

1. Data of debris flow kinematic observation

(1) Code

Each debris flow is denoted by a four-digit number. The former four digits denote year and the latter two denote the debris flow number in the year. For instance, code 198913 denotes the thirteenth debris flow in 1989.

(2) No.

It is the surge number in each debris flow.

(3) Type

It denotes the flow type of each surge, and it is classified to three types, i.e. surge flow (de-noted by S), continuative flow (denoted by C), and data-incomplete surge (denoted by I). When the debris flow discharge between one surge and its succeeding one is quite large or the flow time of one surge is quite long, it is taken as a continuative flow. When there is an obvious flow break between one surge and its succeeding one, it is a surge flow. For a data-incomplete surge some original data items are missed, and other data items for this surge cannot be calculated. The data-incomplete surges are not counted for statistical data except surge number or for general date.

(4) T₁ (h: min: s)

It is the time for the front of each surge to reach the starting observation cross section and is denoted by a six-digit number in hour: minute: second. The former two denote hour (h), the middle two denote minute (min), and the latter two denote second (s). For continuative flow, it is the the start time of the continuous flow.(5) T₂ (h: min: s).It is the time for the rear of each surge to reach the final observation cross section and is also denoted by a six-digit number in hour: minute: second. For continuative flow, it is the the end of the continuous flow. All time data is provided in China Standard Time (CST, UTC+8)

(6) T (s)

It is the flow time of each surge, and T=T₂-T₁.

(7) B (m)

It denotes the surface width of each surge and is determined according to the debris flow channel cross section observation, cross section marks, and debris flow depth.

(8) H (m)

It denotes the front height or depth of each surge. For part of the surges, the depth is measured by supersonic level meter through computer. For other surges, the depth is determined referring to cross section marks and debris flow level.

(9) L (m)

It is the distance used to measure the velocity of each surge front and is marked along a straight channel. It is called velocity measuring distance.

(10) t (s)

It is the time recorded by stopwatch for the surge front to pass the velocity measuring distance L.

(11) V (m/s)

It is the front velocity.

V=Lt

where, L is distance, t is the record time

(12) Q (m³/s)

It denotes the front discharge,

Q=V×B×H

where, V is velocity, B is surface width, H is surge depth.

(13) γc (t/m³)

It denotes the unit weight of each surge. It is determined according to sampling or referring to sampling. When the duration between two samplings is not too long, it is determined by the former sampling. If there is a debris flow sampling for one surge, it is marked in the column of “Note”.

(14) Wc (m³)

It denotes the runoff of each surge.

Wc=Q×T2  for surge flow;

Wc=Q×T  for continuative flow.

where, Q is discharge, T is the record time.

(15) Ws (m³): It denotes the sediment runoff of each surge.

Ws=Wc×Cv   Cv=γc-γγs-γ

where, Wc is runoff; Cv is the volume concentration of debris flow; γ, γc , γs (t/m³) are unit weight of water, debris flow, and sediment respectively; γs is usually taken as 2650 (kg/m³).

(16) Note

If there is a debris flow sampling, it is marked in this column as “Sam”. Then the unit weight is determined by the sampling.

2. Statistical data of debris flow kinematic observation

The data in this table are mainly statistics for surge flow and continuative flow respectively. For data-incomplete surges only surge number is counted.

(1) Qm (m³/s): mean discharge.

(2) Sm (kg/m3): mean sediment concentration. S denotes sediment concentration.

S=rs×Cv

in which, rs is the unit weight of sediment, usually taken as 2650 kg/m³; Cv is the volume concentration of debris flow.

(3) Cvm: mean volume concentration

(4) Qcm (t/s): mean sediment transport rate. Qc denotes sediment transport rate.

Qc=Q×S/1000

in which, Q (m3/s) is debris flow discharge; S (kg/m3) is sediment concentration.

(5) Wc (m³): total debris flow runoff.

(6) Ws (m³): total sediment flow runoff.

(7) Vmax (m/s): maximum front velocity.

(8) Hmax (m): maximum front depth.

(9) Qmax (m³/s): maximum front discharge.

(10) Qmax No.: surge number for maximum front discharge.

(11) Total No.: the total surge number for different type of surges.

3. General data of debris flow kinematic observation

The data in this table are statistics for all surges except data-incomplete ones. Here some data items which are not described in pretext are discussed.

(1) Start time (Y/M/D/h/m)

It is the time for the first debris flow front to reach the observation cross section or the time to start observation, and it is expressed as year/month/day/hour/minute.

(2) End time (Y/M/D/h/m)

It is the time for the last debris flow rear to reach the observation cross section or the time to end observation, and it is also expressed as year/month/day/hour/minute.

(3) Duration (h: min:s)

Duration =End time-Start time

(4) JIt denotes the channel slope measured by theodolite or portable leveling instrument.

Observation method of kinematic parameters of debris flow at Jiangjia Ravine

Measurement

The debris-flow kinematic parameters were monitored in a straight channel adjacent to the DDFORS. The location of two debris-flow observation sections is shown in Fig. 1 Observation distances ranged from 50 m to 200 m. Flow time between the two sections was measured using a second chronograph. Depth, defined as the surge front height, was determined by referencing cross-sectional marks on the channel banks, representing the vertical distance between the top of the debris-flow surge and the channel surface (Fig. 2). Surface width of debris flow was determined by referencing of channel width. Velocity was calculated by dividing the observation distance by the flow time for each surge as it passed through the cross sections. Bulk density of each surge was determined either through sampling weighting or estimation.

Figure 1 The location of two debris-flow observation sections

Figure 4. Field observation of debris-flow kinematics and sampling:(a) (b) surface width, depth and velocity measurement. (c)illustration of flow depth.

Note:

(1) In works such as paper, patent, and monograph, please indicate the data source. In the acknowledgments section of Chinese publications, include the statement “感谢中国科学院东川泥石流观测研究站为本研究提供了相关数据” . In the acknowledgments section of English publications, please state, “We would like to thank the Dongchuan Debris Flow Observation and Research Station (DDFORS), Chinese Academy of Sciences, for providing the field observation data for this study.”

(2) For more detailed information, please visit the website of Dongchuan Debris Flow Observation and Research Station (DDFORS) (Chinese website: http://nsl.imde.ac.cn/; English website: http://nsl.imde.ac.cn/en/).

(3) For any data-related issues, please contact Li Wei (weili@imde.ac.cn) and Dongri Song (drsong@imde.ac.cn).