蒋家沟泥石流超声波泥位数据说明

- 观测仪器

  - 生产厂家：Siemens Milltronics Process Instruments Inc.

  - 型号：Airanger SPL

  - 采样频率：10 Hz

  - 测量范围：0.3m～60m;

  - 精度：量程的±度：量程的或6mm,取其较大者;

  - 分辨率：量程的0.1%或2mm，取其较大者

2. 观测位置及方法

仪器安装位于东川站泥石流观测段沟道正上方

3. 数据处理说明

泥石流阵流龙头流深是通过安装高度与超声波传感器记录值之间的差异确定的，超声波传感器的安装高度设定为10米。在阵性流龙头存在明显的飞溅现象，导致超声波传感器测量的流深异常偏高。因此，基于1999年至2001年蒋家沟泥石流运动要素的数据，对获取到的7场阵性泥石流的龙头流深进行了修正。

泥石流观测场景

使用的超声波采集器及换能器

备注：

（1）请在论文发表、专利申请、专著出版等工作中标注数据来源，并在公开发表的中文出版物的致谢部分标明“感谢中国科学院东川泥石流观测研究站为本研究提供了相关数据”，在英文论文致谢部分标明“Dongchuan Debris Flow Observation and Research Station (DDFORS), Chinese Academy of Sciences, which provided the field observation data for this study.”.

（2）更多详细信息见东川站网页（中文网站http://nsl.imde.ac.cn/；英文网站http://nsl.imde.ac.cn/en/）.

（3）数据问题可联系魏丽（weili@imde.ac.cn）和宋东日（drsong@imde.ac.cn）.

Description of debris-flow depth data measured by ultrasonic level meter at Jiangjia Ravine

- Monitoring sensor:

- Manufacturer: Siemens Milltronics Process Instruments Inc.

- Model: Airanger SPL

- Sampling Frequency: 10 Hz

- Measurement Range: 0.3 m to 60 m

- Accuracy: ±0.25% of the range or 6 mm, whichever is greater

- Resolution: 0.1% of the range or 2 mm, whichever is greater

- Observation Location and Method

The instrument is installed directly above the debris flow gully.

- Data processing

The measured surge-front flow depths are the differences between the installation heights and the measured values of the ultrasonic sensors, where the installation heights of the ultrasonic level meter are taken as 10 m. There are obvious splashes in the surge-flow fronts, which make the flow depths measured by the ultrasonic level meter abnormally high. Therefore, based on the data of debris-flow kinematic elements in Jiangjia Ravine, Dongchuan, Yunnan Province from 1999 to 2001, the surge-front flow depths of these 7 surge-flow events are corrected.

Scenario of debris-flow observation at Jiangjia Ravine

The ultrasonic collector and transducer

Note:

(1) In works such as paper, patent, and monograph, please indicate the data source. In the acknowledgments section of Chinese publications, include the statement “感谢中国科学院东川泥石流观测研究站为本研究提供了相关数据” . In the acknowledgments section of English publications, please state, “We would like to thank the Dongchuan Debris Flow Observation and Research Station (DDFORS), Chinese Academy of Sciences, for providing the field observation data for this study.”

(2) For more detailed information, please visit the website of Dongchuan Debris Flow Observation and Research Station (DDFORS) (Chinese website: http://nsl.imde.ac.cn/; English website: http://nsl.imde.ac.cn/en/).

(3) For any data-related issues, please contact Li Wei (weili@imde.ac.cn) and Dongri Song (drsong@imde.ac.cn).