Land use data set of Altay national key prevention area in 2019

DOI：10.12072/ncdc.hhstbc.db2670.2022

CSTR：11738.11.ncdc.HHSTBC.2021.308

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

The data set of land use statistics of Altay national key prevention area in 2019 includes the land use statistics of Altay City, Buerjin County, Fuyun County, Habahe County, Qinghe County, Jimunai county and Beitun city in Xinjiang Uygur Autonomous Region in 2019, which is based on the satellite remote sensing image with spatial resolution better than 16M and saved in xlsx format, The data is named in the form of "key governance area + administrative region + year + land use statistical table", such as“ ×× Key governance areas ×× county ×× Statistics of land use in 2001 ". The classification system of land use includes 8 first class and 25 second class

Data source description

The land use remote sensing image data sources are ZY-3 and gaofen-1 satellite images, which are obtained from the information center of the Ministry of water resources.

Data processing method

Based on ecognition software platform, the method of combining object-oriented computer automatic classification with manual visual interpretation was used to extract the annual land use data of the study area. Finally, three methods are used to verify the data accuracy: field sample point survey, high resolution image recognition and Google Earth sample point recognition.

Data quality description

The remote sensing images are preprocessed by radiation correction, orthorectification, fusion and mosaic. The actual surface area corresponding to the minimum patch area is not less than 0.1 Hm2, the polygon has no overlap, no gap, and the patch attribute has no vacancy or redundancy. Before remote sensing image interpretation, remote sensing image, typical survey and field comparison were used to establish remote sensing interpretation marks of land use. Based on the remote sensing image, combined with the interpretation marks, the land use types are extracted. Reexamination of interpretation results: take no less than 5% of the total map spots for verification. The number and results of field verification samples meet the requirements of technical specification for remote sensing monitoring of soil and water conservation (sl592-2012). For the verification map spots, 10% of the verification samples are selected for field verification.

Base information

collect time:2019/01/01-2019/12/31

collect place:Altai mountain national key prevention area

data size:238.8 KiB

data format:

Coordinate system: 无

Keywords

Theme:There are 8 first class elements and 25 second class elements in the statistical table of land use
	Subject:
	Time:2018
	Place:Altay Mountain National key prevention area

Space scope

|  | north： |  |
| --- | --- | --- |
| west： |  | east： |
|  | south： |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Monitoring Center for ecological environment of soil and water conservation in the Yellow River Basin

author:Li Suya

manager:Li Suya

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Monitoring Center for ecological environment of soil and water conservation in the Yellow River Basin. Land use data set of Altay national key prevention area in 2019. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.HHSTBC.2021.308.