2023 Rainfall Observation Dataset of Qiaogou Watershed in the First Sub District of Loess Hilly and Gully Region

DOI：10.12072/ncdc.hwsdzhan.db6583.2024

CSTR：11738.11.NCDC.HWSDZHAN.DB6583.2024

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

This dataset contains precipitation observation data from the Qiaogou Basin rainfall station in 2023. The Qiaogou watershed is located in Suide County, Yulin City, Shaanxi Province, and is a first level tributary of Peijiamaogou (a first level tributary of Wuding River). There are four rainfall stations in the Qiaogou watershed, namely Qiaogou 1, Qiaogou 2, Qiaogou 3, and Qiaogou 5. DSJ2 siphon type self recording rain gauges and SM1 ordinary rain gauges with a diameter of 20cm are used for parallel observation. When the self recording rain gauges fail, ordinary rain gauges are used instead. The data includes the layout map of the watershed station network, daily precipitation table, precipitation extraction table, and maximum precipitation table for each period. The basic information of the rainfall station in the Qiaogou Basin can be found in the "Qiaogou Basin 2023 Situation Description"

Data source description

Continuous observation by instrument and regular observation manually

Data processing method

Precipitation observation instruments include DSJ2 siphon self recording rain gauge and SM1 ordinary rain gauge with a diameter of 20cm. The DSJ2 self recording rain gauge adopts the 24 segment system for flood season observation, and the SM1 ordinary rain gauge adopts the 8:00 and 20:00 two segment system for annual observation. In case of failure of the self recording rain gauge, the observation data of the ordinary rain gauge shall be used for supplement. The starting and ending time of precipitation is mainly observed, and the precipitation is counted. When the interval time of precipitation is more than 15min, it is observed as two times of precipitation before and after the interval time. When the interval time is less than 15min, it is recorded as a precipitation observation. If a precipitation exceeds 8 and 20 hours, it should be added at 8 and 20 hours

Data quality description

The recording rain gauge shall be calibrated before the start of observation every year. In the process of data entry, there are four levels of manual verification: tabulation, initial verification, re verification, and audit to ensure data quality

Base information

collect time:2023/01/01-2023/12/31

collect place:Qiaogou watershed, Suide County, Yulin City, Shaanxi Province

data size:354.7 KiB

data format:

Coordinate system: WGS84

Keywords

Theme:Daily precipitation, precipitation intensity, maximum precipitation, period precipitation
	Subject:
	Time:2023
	Place:Qiaogou,  Yulin,  Suide,  Shaanxi Province

Space scope

|  | north： 37.50 |  |
| --- | --- | --- |
| west：110.29 |  | east：110.30 |
|  | south： 37.49 |  |

Service information

contacts:Min Yu Fang

phone:09314967596

mailbox:myf@lzb.ac.cn

Contact information

contributors:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

author: ,

manager:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation) , LEI Xin

Project information

number:126201340005150007，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation). 2023 Rainfall Observation Dataset of Qiaogou Watershed in the First Sub District of Loess Hilly and Gully Region. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2024. https://cstr.cn/CSTR:11738.11.NCDC.HWSDZHAN.DB6583.2024.