Heihe River eco hydrological remote sensing experiment: multi scale observation experiment of surface evapotranspiration over non-uniform underlying surface flux observation matrix dataset (eddy covariance at point 11-2012)

DOI：10.12072/ncdc.NIEER.db0042.2020

CSTR：11738.11.ncdc.nieer.2020.1574

Share type:Apply to Access

Category:Meteorology

Datasets description

The data set contains the eddy related flux observation data of point 11 in the flux observation matrix from May 29 to September 18, 2012. The station is located in the farmland of Yingke Irrigation District, Zhangye City, Gansu Province, with corn as the underlying surface. The longitude and latitude of the observation point are 100.34197e, 38.86991n, and the altitude is 1575.65m. The eddy correlator frame is 3.5m high, the sampling frequency is 10Hz, the ultrasonic direction is due north, and the distance between the ultrasonic anemometer and CO2 / H2O analyzer is 15cm. The original observation data of eddy correlator is 10Hz. The released data is the 30 minute data post processed by edire software. The main processing steps include: field value elimination, delay time correction, coordinate rotation (quadratic coordinate rotation), frequency response correction, ultrasonic virtual temperature correction and density (WPL) correction. At the same time, the quality of each flux value is evaluated, mainly the atmospheric stability（ Δ St) and turbulence similarity characteristics (ITC). The 30 min flux value output by edire software is also screened: (1) eliminate the data when the instrument is wrong（ 2) Eliminate the data 1H before and after precipitation（ 3) Eliminate the data with a loss rate of more than 3% every 30min of 10Hz original data（ 4) Eliminate the observation data of weak turbulence at night (U * less than 0.1M / s). The average period of observation data is 30 minutes, 48 data a day, and the missing data is marked as - 6999. Suspicious data caused by instrument drift and other reasons shall be identified in red font. The published observation data include: date / time, wind direction WDIR (°), horizontal wind speed wnd (M / s), standard deviation of lateral wind speed STD_ Uy (M / s), ultrasonic virtual temperature TV (℃), water vapor density H2O (g / m3), carbon dioxide concentration CO2 (mg / m3), friction velocity ustar (M / s), stability Z / L (dimensionless), sensible heat flux HS (w / m2), latent heat flux Le (w / m2), carbon dioxide flux FC (mg / (M2S)), quality identification QA of sensible heat flux_ HS, quality identification of latent heat flux QA_ Le, quality identification QA of carbon dioxide flux_ Fc。 The quality identification of sensible heat, latent heat and carbon dioxide flux is divided into three levels (quality identification 0:（ Δ st < 30, ITC< 30）; 1: （ Δ st < 100, ITC< 100）; The rest are 2). The meaning of data time, for example, 0:30 represents the average of 0:00-0:30; The data is stored in *. XLS format. For multi-scale observation test or site information, please refer to Liu et al. (2016), and for observation data processing, please refer to Xu et al. (2013)

Data source description

The published observation data include: date / time, wind direction WDIR (°), horizontal wind speed wnd (M / s), standard deviation of lateral wind speed STD_ Uy (M / s), ultrasonic virtual temperature TV (℃), water vapor density H2O (g / m3), carbon dioxide concentration CO2 (mg / m3), friction velocity ustar (M / s), stability Z / L (dimensionless), sensible heat flux HS (w / m2), latent heat flux Le (w / m2), carbon dioxide flux FC (mg / (M2S)), quality identification QA of sensible heat flux_ HS, quality identification of latent heat flux QA_ Le, quality identification QA of carbon dioxide flux_ Fc。 The quality identification of sensible heat, latent heat and carbon dioxide flux is divided into three levels (quality identification 0:（ Δ st < 30, ITC< 30）; 1: （ Δ st < 100, ITC< 100）; The rest are 2). The meaning of data time, for example, 0:30 represents the average of 0:00-0:30; The data is stored in *. XLS format

Data processing method

Automatic observation and manual calibration of the instrument

Data quality description

The average period of observation data is 30 minutes, 48 data a day, and the missing data is marked as - 6999. Suspicious data caused by instrument drift and other reasons shall be identified in red font

Base information

collect time:2012/05/29-2012/09/18

collect place:Zhangye City, Gansu Province

data size:793.5 KiB

data format:

Coordinate system: 无

Keywords

Theme:CO2 flux, Eddy correlator, Sensible heat flux, Latent heat flux, hydrometeor
	Subject:geography 
	Time:2012
	Place:Heihe River Basin,  artificial oasis experimental area in the middle reaches,  flux observation matrix

Space scope

|  | north： 38.87 |  |
| --- | --- | --- |
| west：100.34 |  | east：100.34 |
|  | south： 38.87 |  |

Service information

contacts:

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Liu shaomin

author:Liu shaomin

manager:Xu ziwei

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Liu shaomin. Heihe River eco hydrological remote sensing experiment: multi scale observation experiment of surface evapotranspiration over non-uniform underlying surface flux observation matrix dataset (eddy covariance at point 11-2012). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.nieer.2020.1574.

Article citation:

Liu Shaomin, Xu Ziwei, Song Lisheng, Zhao Qianyi, Ge Yong, Xu Tongren, Ma Yanfei, Zhu Zhongli, Jia Zhenzhen, Zhang Fen. Upscaling evapotranspiration measurements from multi-site to the satellite pixel scale over heterogeneous land surfaces[J]. Agricultural and Forest Meteorology, 2016, 230: 97-113.

Xu Ziwei, Liu Shaomin, Li Xin, Shi Shengjin, Wang Jiemin, Zhu Zhongli, Xu Tongren, Wang Weizhen, Ma Mingguo. Intercomparison of surface energy flux measurement systems used during the HiWATER‐MUSOEXE[J]. Journal of Geophysical Research, 2013, 118, (23).