Multi year Observation of Near Bottom Water Dynamics and Suspended Sediments in the Core Area of the Yangtze River Estuary with Maximum Turbidity (2015-2018)

DOI：10.4121/6d4fcaff-6a30-4990-881a-a5ac8ebb1ca6

Share type:Open Access

Category:Hydrology

Datasets description

This dataset is derived from a comprehensive multi-year field investigation activity called the North Channel Measurement Program (NP ChamM), aimed at deepening the understanding of hydrodynamics and sedimentary dynamics of the North Channel, the main channel of the Yangtze River estuary in China. Covering eight observation points, lasting 50 days, distributed over four years, including two dry seasons and two rainy seasons. By deploying a tripod system equipped with multiple instruments on the seabed, reliable monitoring of near bottom physical processes has been achieved. The final dataset obtained includes the following: (1) fluid motion data, covering pressure, flow velocity and direction (bottom and full water column range), as well as wave morphology; (2) Near bottom environmental conditions, including temperature, salinity, and turbidity (within 1 meter of the bottom layer and near the bottom); (3) Supplementary meteorological data from reliable suppliers; (4) Preliminary results of post-processing demonstrate the practical application value of the data, such as analysis of lateral flow field and turbulent kinetic energy characteristics. This dataset is particularly valuable due to its wide temporal and spatial coverage and high concentration observation characteristics (concentration range from grams per liter to tens of grams). The NP Cham observation program was implemented annually from 2015 to 2018, achieving precise observations of seasonal changes in environmental conditions and related physical processes. Eight observation points distributed on both sides of the deep-water channel can quantitatively analyze the channel shallow water exchange process, flow mechanics along the channel, and saltwater intrusion phenomena. This dataset can deepen our understanding of the dynamics along and across channels in the channel shoal system, and is also suitable for calibrating numerical models. All data undergoes strict quality control to ensure reliability and accuracy.

Data source description

Between 2015 and 2018, the NP Cham observation plan for the North Passage of the Yangtze River estuary was established by measuring the near bottom environmental conditions through eight tripod frames. The observation period covers both the rainy and dry seasons, with each measurement lasting over 12 days, fully capturing the transition period from small tide to large tide. The measurement points are concentrated on both sides of the deep-water channel in the middle of the Xinpu River.

Data processing method

Given the relevant temporal and spatial scales, this observation activity should: (1) prioritize monitoring the bottom layer and vertical structure of the water near the seabed; (2) Covering both the north and south sides of the deep-water channel, comprehensively recording the exchange of water and sediment between the channel and adjacent shallow waters; (3) Multiple observation cycles covering both dry and rainy seasons, with each observation lasting approximately 14 days.     Following the above standards, a tripod frame is used to monitor near bottom physical processes and key variables, including flow velocity and direction, salinity, and suspended solids content. The framework is powered by batteries and can work continuously for more than 10 days. The framework structure and mounted instruments are detailed in Section 4. As part of the NP Cham program, a total of four observation missions were carried out between 2015 and 2018, covering eight monitoring stations strategically located in the central region of NP: five of them are located between the southern shoal and the deep-water channel, and the remaining three are located between the northern shoal and the deep-water channel.

Data quality description

Most of the instruments installed on the framework are running well. However, due to various reasons, operational interruptions occasionally occur, with battery issues caused by installation errors or accidental collisions being particularly frequent. For example, in the NP Cham observation activity, the ADV used in the 2015D observation stopped working from the seventh day, and the ASM in the 2017A observation and the RBR in the 2018A observation failed to collect the expected data. Therefore, these instruments were excluded from the published dataset.

Base information

collect time:2015/01/01-2018/12/31

collect place:Yangtze River estuary

data size:296.4 MiB

data format:.csv、.dat、.png

Coordinate system: 无

Keywords

Theme:Tripod measurement, sediment concentration, close bed observation
	Subject:geography 
	Time:2015 2016 2017 2018
	Place:Yangtze River estuary

Space scope

|  | north： 31.87 |  |
| --- | --- | --- |
| west：121.67 |  | east：122.50 |
|  | south： 31.17 |  |

Service information

contacts:Min Yufang

phone:09314967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Ge Jianzhong

author:Ge Jianzhong

manager:Ge Jianzhong

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Ge Jianzhong. Multi year Observation of Near Bottom Water Dynamics and Suspended Sediments in the Core Area of the Yangtze River Estuary with Maximum Turbidity (2015-2018). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2025. https://www.doi.org/10.4121/6d4fcaff-6a30-4990-881a-a5ac8ebb1ca6.

Article citation:

Zhou Z, Ge J, van Maren DS, Luan H, Guo W, Ma J, Tao Y, Xu P, Dao F, Yang W, Ke K, Shi S, Zhang J, Kuai Y, Li C, Gu J, Ding P. Multi-year observations of near-bed hydrodynamics  and suspended sediment at the core of the estuarine turbidity maximum of the Changjiang Estuary[J/OL]. Earth Syst. Sci. Data, 2025-03-13, 17, (3): 917-935. 
            DOI: 10.5194/essd-17-917-2025.