Monitoring Dataset of Temperature and Humidity Fields on the Slope of Liugou Reservoir Bank in Xinjiang (August 31, 2019- December 3, 2019)

DOI：10.12072/ncdc.nieer.db2829.2024

CSTR：11738.11.NCDC.NIEER.DB2829.2023

Share type:Login to Access

Category:Others

Datasets description

This dataset involves monitoring data of temperature and humidity fields on the bank slope of Liugou Reservoir in Xinjiang. It mainly includes monitoring the temperature change data and humidity change data of the cross-section. The time range of the dataset is from 9:00 on August 31, 2019 to 1:00 on December 3, 2019. The time accuracy of this dataset is 1 hour. The spatial scope involves two monitoring cross-sections, with recorded ranges of approximately 17.3m × 7.5m and 3.5m × 4.5m, respectively. Since it does not involve three-dimensional spatial coordinates, this spatial accuracy is not considered. The temperature probe adopts the "multipoint temperature measuring cable" of State Key Laboratory of Frozen Soil Engineering, with an accuracy of ± 0.05 ℃. The temperature and humidity meter uses the 5TM soil temperature and humidity sensor produced by Decagon Corporation in the United States (Figure 3). The 5TM uses an oscillator with an operating frequency of 70 MHz to determine the volumetric moisture content of the soil by measuring its dielectric constant. The thermistor on the fork shaped probe of the sensor measures the soil temperature. 5TM determines the volumetric water content (VWC) by measuring the dielectric constant of soil (or other media) using capacitance/frequency domain techniques. Signal filtering minimizes salinity and texture effects, making 5TM more accurate in most soils and soilless media.

Data source description

This dataset consists of one set of data per hour, which involves monitoring the temperature and humidity data of the slope along the reservoir bank. The first four lines are system information, lines 1, 3, and 4 have no actual meaning, and the second line is the sensor numbers. The second line is the ground temperature or humidity data obtained by monitoring the actual probes of each borehole at a height above the ground. The real-time temperature or humidity data at each temperature detection point is measured from the 5th to the last row.

Data processing method

Measured results.

Data quality description

The data quality is good.

Base information

collect time:2019/08/31-2019/12/03

collect place:Reservoir bank slope of Liugou Reservoir in Xinjiang

data size:3.8 MiB

Coordinate system: 无

Keywords

Theme:Reservoir bank slope, cold area, humidity, temperature
	Subject:geography 
	Time:2019
	Place:Liugou Reservoir

Space scope

|  | north： 49.17 |  |
| --- | --- | --- |
| west： 73.50 |  | east： 96.38 |
|  | south： 34.33 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:bai ruiqiang

author:bai ruiqiang

manager:bai ruiqiang

Project information

number:2018YFC0809600，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

bai ruiqiang. Monitoring Dataset of Temperature and Humidity Fields on the Slope of Liugou Reservoir Bank in Xinjiang (August 31, 2019- December 3, 2019). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2023. https://cstr.cn/CSTR:11738.11.NCDC.NIEER.DB2829.2023.