Xinjiang Dazimiao Reservoir Thermal Coating/Insulation Board Concrete Dam Monitoring Section Dataset (September 2, 2019- March 7, 2020)

DOI：10.12072/ncdc.nieer.db2826.2023

CSTR：11738.11.NCDC.NIEER.DB2826.2023

Share type:Login to Access

Category:Others

Datasets description

This dataset is mainly aimed at the research and protection needs construction of frozen slopes in the northwest cold and freeze-thaw environment. Based on the monitoring section of the thermal coating/insulation board concrete dam in Dazimiao Reservoir, Xinjiang, it mainly records meteorological data (temperature, wind speed), deformation, heat flux, and temperature monitoring data. The time range of this dataset is: 1) Thermal coating section data: from 13:00 on September 2, 2019 to 23:00 on March 7, 2020, with a data interval of 1 hour. 2) Meteorological data (air temperature and wind speed): Air temperature from 13:30 on September 2, 2019 to 13:00 on September 11, 2021, and wind speed from 12:30 on November 7, 2019 to 13:00 on September 11, 2021; Among them, there is a lack of data on wind speed and air temperature from 10:30 am on June 17, 2020 to 4:00 pm on November 30, 2020, with a recording interval of 0.5 hours. 3) Hot coating heat flow data: from 2:00 pm on September 2, 2019 to 3:00 pm on April 8, 2021, with a recording interval of 1 hour. 4) Hot coating cross-section and comparative cross-section data: from 0:00 am on September 10, 2019 to 2:00 pm on September 1, 2021, with a data interval of 1 hour. 5) Insulation board cross-section and comparative cross-section data: from 0:00 am on September 3, 2019 to 12:30 pm on September 11, 2021, with a data interval of 0.5 hours. The spatial range involves two monitoring cross-sections, with records of approximately 20m × 7m each. Within a range of 15m x 7m. Since it does not involve three-dimensional spatial coordinates, this spatial accuracy is not considered. The temperature probe adopts the "multipoint temperature measuring cable" of State Key Laboratory of Frozen Soil Engineering, with an accuracy of ± 0.05 ℃.

Data source description

Observation and monitoring data.

Data processing method

Observation and monitoring.

Data quality description

The data quality is good.

Base information

collect time:2019/09/02-2020/03/07

collect place:Dazimiao Reservoir, Xinjiang

data size:94.8 MiB

data format:Excel

Coordinate system: 无

Keywords

Theme:Thermal coating/insulation board concrete dam, cold zone, temperature, deformation, heat flux, meteorological elements
	Subject:geography 
	Time:2019 2020
	Place:Dazimiao Reservoir

Space scope

|  | north： 49.17 |  |
| --- | --- | --- |
| west： 73.50 |  | east： 96.38 |
|  | south： 34.33 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:bai ruiqiang

author:bai ruiqiang

manager:bai ruiqiang

Project information

number:2018YFC0809600，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

bai ruiqiang. Xinjiang Dazimiao Reservoir Thermal Coating/Insulation Board Concrete Dam Monitoring Section Dataset (September 2, 2019- March 7, 2020). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2023. https://cstr.cn/CSTR:11738.11.NCDC.NIEER.DB2826.2023.