Heihe River Integrated Remote Sensing joint experiment: Airborne widas ground synchronous observation data set in Linze grassland intensive observation area (June 29, 2008)

DOI：10.12072/ncdc.NIEER.db1879.2022

CSTR：11738.11.ncdc.nieer.2020.1552

Share type:Login to Access

Category:Remote Sensing

Datasets description

On June 29, 2008, the aviation flight test of widas (wide angle infrared dual mode line / area array scanner) was carried out in the intensive observation area of Linze grassland. Widas is composed of 4 CCD cameras, 1 mid infrared thermal imager (AGEMA 550) and 1 thermal infrared thermal imager (S60). It can simultaneously obtain data of 5 angles of visible / near infrared (CCD) band, 7 angles of mid infrared band (MIR) and 7 angles of thermal infrared band (TIR). The main variable of ground synchronous observation is surface temperature. This data can provide a basic data set for surface temperature inversion and verification of airborne satellite remote sensing data. In the intensive observation area of Linze grassland, surface temperature measurement is carried out by hand-held infrared thermometer in quadrat a (reed land), quadrat B (saline alkali land), quadrat C (saline alkali land) and quadrat e (barley land). Synchronous quadrat includes 120m × 120m quadrat and 30m × There are two kinds of 30m densified small quadrats, which are measured continuously along the northeast direction. This data set includes the excel table of surface temperature measurement data of 4 quadrats. See metadata "Heihe integrated remote sensing joint experiment: layout of quadrat transects in Linze grassland densified observation area" for the distribution information of quadrat transects

Data source description

On June 29, 2008, the aviation flight test of widas (wide angle infrared dual mode line / area array scanner) was carried out in the intensive observation area of Linze grassland. Widas is composed of 4 CCD cameras, 1 mid infrared thermal imager (AGEMA 550) and 1 thermal infrared thermal imager (S60). It can simultaneously obtain data of 5 angles of visible / near infrared (CCD) band, 7 angles of mid infrared band (MIR) and 7 angles of thermal infrared band (TIR). The main variable of ground synchronous observation is surface temperature. This data can provide a basic data set for surface temperature inversion and verification of airborne satellite remote sensing data.

Data processing method

In the intensive observation area of Linze grassland, surface temperature measurement is carried out by hand-held infrared thermometer in quadrat a (reed land), quadrat B (saline alkali land), quadrat C (saline alkali land) and quadrat e (barley land). Synchronous quadrat includes 120m × 120m quadrat and 30m × There are two kinds of 30m densified small quadrats, which are measured continuously along the northeast direction.

Data quality description

Good data quality

Base information

collect time:2008/06/29-2008/06/30

collect place:Heihe River Basin, arid area in the middle reaches, hydrological experimental area, Linze grassland intensive observation area

data size:534.6 MiB

data format:excel

Coordinate system: WGS84

Keywords

Theme:Multi angle thermal infrared and CCD camera, handheld infrared thermometer, ground synchronous observation, ground radiation temperature, infrared wide-angle dual-mode imager, widas, ground radiation characteristics, airborne ground remote sensing
	Subject:physical geography 
	Time:2008
	Place:Heihe River Basin,  hydrological experimental area of arid area in the middle reaches,  Linze grassland intensive observation area

Space scope

|  | north： 38.97 |  |
| --- | --- | --- |
| west：100.59 |  | east：100.44 |
|  | south： 38.04 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:HuXiaoli,  GAI Chunmei,  HuangChunling,  Wang shuguo

author:HuXiaoli

manager:HuXiaoli

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

HuXiaoli, GAI Chunmei, HuangChunling, Wang shuguo. Heihe River Integrated Remote Sensing joint experiment: Airborne widas ground synchronous observation data set in Linze grassland intensive observation area (June 29, 2008). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2022. https://cstr.cn/CSTR:11738.11.ncdc.nieer.2020.1552.