Data set of ice control index system in the upper reaches of the Yellow River (1960-2018)

DOI：10.12072/ncdc.iceflood.db2379.2022

CSTR：11738.11.ncdc.iceflood.db2379.2022

Share type:Login to Access

Category:Hydrology

Datasets description

The ice prevention control index system includes ice development index, ice stability control index and ice corresponding control index. The development index of ice jam is obtained from the measured data of hydrological station through statistical analysis; The flood stability control index adopts statistical analysis, empirical formula derivation, model calculation and other methods, combined with the different boundary conditions of the river, puts forward the appropriate river closure flow of the river, the under ice flow during the stable closure period, the key flow of the river opening, and the appropriate increment of tank storage; Based on the above analysis and in combination with the upstream reservoir group and the anti ice regulation scheme in the sub ice area, the ice jam response control index system for the whole process of the ice jam period is formulated.

Data source description

The data of this data set are all the data of the actual observation data of the hydrological station and the reservoir after analysis and calculation, including the river closure time, ice flow and river opening time, the daily average flow of the hydrological station, the year when the ice jam ice dam actually occurred and the disaster situation, the regulation rules of the reservoir ice prevention, etc. The hydrological station and reservoir data are observed in strict accordance with the requirements of the national hydrological station observation specifications. The source data are mainly from the hydrological Yearbook published every year, purchased from the Hydrological Bureau of the Yellow River Water Conservancy Commission, and the ice flood summary officially published by the Yellow River Water Conservancy Commission every year.

Data processing method

The development index of ice jam is obtained from the measured data of hydrological station through statistical analysis; The flood stability control index adopts statistical analysis, empirical formula derivation, model calculation and other methods, combined with the different boundary conditions of the river, puts forward the appropriate river closure flow of the river, the under ice flow during the stable closure period, the key flow of the river opening, and the appropriate increment of tank storage; Based on the above analysis and in combination with the upstream reservoir group and the anti ice regulation scheme in the sub ice area, the ice jam response control index system for the whole process of the ice jam period is formulated.

Data quality description

The basic data are measured by professionals of hydrological stations and reservoirs, which are officially published after verification and compilation. The analysis of ice jam control index system is analyzed and calculated by professional scientific researchers and adopted after verification and review.

Base information

collect time:1960/01/01-2018/12/31

collect place:Ningmeng reach

altitude:990.0m- 1090.0m

data size:43.0 KiB

data format:excel

Coordinate system: 无

Keywords

Theme:River closure flow, river opening flow, flow during stable closure period
	Subject:hydrology 
	Time:1960 2018
	Place:Ningmeng reach

Space scope

|  | north： 40.27 |  |
| --- | --- | --- |
| west：106.78 |  | east：111.07 |
|  | south： 39.03 |  |

Service information

contacts:Li Hongxing

phone:0931-4967592

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:LU Jun,  tong haijie,  ma weiming,  mu hongwei

author:tong haijie

manager:tong haijie

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

LU Jun, tong haijie, ma weiming, mu hongwei. Data set of ice control index system in the upper reaches of the Yellow River (1960-2018). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2022. https://cstr.cn/CSTR:11738.11.ncdc.iceflood.db2379.2022.