Heihe River Ecological hydrological remote sensing test: hydrometeorological observation network data set (vortex correlator-2013 at bajitan Gobi station)

DOI：10.12072/ncdc.Westdc.db0016.2020

CSTR：11738.11.ncdc.Westdc.2020.704

Share type:Apply to Access

Category:Remote Sensing

Datasets description

The data set contains the eddy related observation data of bajitan Gobi station in the middle reaches of Heihe hydrometeorological observation network from September 18, 2012 to December 31, 2013 The station is located in Zhangye City, Gansu Province, with Gobi as its underlying surface. The longitude and latitude of the observation point are 100.30420e, 38.91496n and 1562.00m above sea level. The frame height of eddy correlator is 4.6m, the sampling frequency is 10Hz, the ultrasonic direction is due north, and the distance between ultrasonic anemometer and CO2 / H2O analyzer is 15cm

Data source description

The published observation data include: date / time, wind direction WDIR (°), horizontal wind speed wnd (M / s), standard deviation of lateral wind speed STD_ Uy (M / s), ultrasonic virtual temperature TV (℃), water vapor density H2O (g / m3), carbon dioxide concentration CO2 (mg / m3), friction velocity ustar (M / s), stability Z / L (dimensionless), sensible heat flux HS (w / m2), latent heat flux Le (w / m2), carbon dioxide flux FC (mg / (M2S)), quality identification QA of sensible heat flux_ HS, quality identification of latent heat flux QA_ Le, quality identification QA of carbon dioxide flux_ Fc。 The quality identification of sensible heat, latent heat and carbon dioxide flux is divided into three levels (quality identification 0:（ Δ st < 30, ITC< 30）; 1: （ Δ st < 100, ITC< 100）; The rest are 2)  The meaning of data time, for example, 0:30 represents the average of 0:00-0:30; The data is stored in *. XLS format

Data processing method

The original observation data of eddy correlator is 10Hz, and the released data is the 30 minute data processed by edire software. The main processing steps include:  Field value elimination, delay time correction, coordinate rotation (quadratic coordinate rotation), frequency response correction, ultrasonic virtual temperature correction and density (WPL) correction

Data quality description

At the same time, the quality of each flux value is evaluated, mainly the atmospheric stability（ Δ St) and turbulence similarity characteristics (ITC). The 30 min flux value output by edire software is also screened:  （ 1) Eliminate the data in case of instrument error  （ 2) Eliminate the data 1H before and after precipitation  （ 3) Eliminate the data with deletion rate greater than 3% every 30min in the 10Hz original data  （ 4) Eliminate the observation data of weak turbulence at night (U * less than 0.1M / s). The average period of observation data is 30 minutes, 48 data a day, and the missing data is marked as - 6999   The suspicious data caused by instrument drift and other reasons are marked in red font. From May 22 to June 13, 2013, the internal battery of the collector was out of power, resulting in the loss of data during this period

Base information

collect time:2012/09/18-2013/12/31

collect place:

data size:3.2 MiB

data format:

Coordinate system: WGS84

Keywords

Theme:Hydrometeorological observation network, Eddy correlator, CO2 flux, Latent heat flux, Sensible heat flux
	Subject:Hydrogeography 
	Time:2013
	Place:Heihe River Basin,  Artificial oasis experimental area in the middle reaches,  Gobi station

Space scope

|  | north： 38.92 |  |
| --- | --- | --- |
| west：100.30 |  | east：100.30 |
|  | south： 38.92 |  |

Service information

contacts:

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Liu shaomin

author:Liu shaomin

manager:Heihe project data management center , Xu ziwei

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Liu shaomin. Heihe River Ecological hydrological remote sensing test: hydrometeorological observation network data set (vortex correlator-2013 at bajitan Gobi station). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.Westdc.2020.704.

Article citation:

Liu Shaomin, Xu Ziwei, Wang Weishan, Jia Zhenzhen, Zhu Mingjia, Bai J, Wang Jianjun. A comparison of eddy-covariance and large aperture scintillometer measurements with respect to the energy balance closure problem[J]. Hydrology and Earth System Sciences, 2011, 15, (4): 1291-1306.

Liu Shaomin, Xu Ziwei, Wang Weishan, Jia Zhenzhen, Zhu Mingjia, Bai J, Wang Jianjun. A comparison of eddy-covariance and large aperture scintillometer measurements with respect to the energy balance closure problem[J]. Hydrology and Earth System Sciences, 2011, 15, (4): 1291-1306.