Heihe River Integrated Remote Sensing joint test: L & K band airborne microwave radiometer data set in ARJO biandukou flight area (March 19, 2008)

DOI：10.12072/ncdc.NIEER.db1729.2022

CSTR：11738.11.ncdc.NIEER.2021.12

Share type:Apply to Access

Category:Remote Sensing

Datasets description

This data set was acquired by L & K-band airborne microwave radiometer on March 19, 2008 at the flight area of Ajo biandukou. The L-band frequency is 1.4GHz, and the back view is 35 degrees to obtain the dual polarization (H and V) information; K-band frequency is 18.7ghz, zenith angle observation, no polarization information. The plane took off from Zhangye airport at 9:25bjt and landed at 12:50. 9: At 38-9:39, it flew over shuangshusi reservoir with a relative navigation height of 800m and the water surface was frozen. 9: At 50-11:00, he worked in the arzou photography area, with a flight altitude of about 4100m and a flight speed of about 270km / hr. 11: At 20-12:25, he worked in the Biandu mouth photography area, with a flight altitude of about 4200m and a flight speed of about 290km / hr. The original data is divided into two parts: microwave radiometer data and GPS data. The L and K bands of microwave radiometer are non imaging observations. The digital values obtained from instantaneous observations are recorded by text files, and the longitude and latitude during flight and aircraft attitude parameters are recorded by GPS data. When using microwave radiometer observation data, it is necessary to convert the recorded digital value into bright temperature value according to the calibration coefficient (the calibration coefficient file is archived with the original observation data). At the same time, through the respective clock records of microwave radiometer and GPS, microwave observation can be connected with GPS record to match geographic coordinate information for microwave observation. Due to the coarse observation resolution of microwave radiometer, the yaw, roll and pitch effects of aircraft are generally ignored in data processing. According to the use target and flight relative altitude (H), after calibration and coordinate matching, the observation information can also be rasterized. The resolution (x) of L and K bands can be considered consistent with the observation footprint. The reference resolution is: L band, x = 0.3H; K-band, x = 0.24h. After the above steps, the products that users can use directly can be obtained.

Data source description

The original data is divided into two parts: microwave radiometer data and GPS data. The L and K bands of microwave radiometer are non imaging observations. The digital values obtained from instantaneous observations are recorded by text files, and the longitude and latitude during flight and aircraft attitude parameters are recorded by GPS data.

Data processing method

When using microwave radiometer observation data, it is necessary to convert the recorded digital value into bright temperature value according to the calibration coefficient (the calibration coefficient file is archived with the original observation data). At the same time, through the respective clock records of microwave radiometer and GPS, microwave observation can be connected with GPS record to match geographic coordinate information for microwave observation.

Data quality description

Good data quality

Base information

collect time:2008/03/19-2008/03/19

collect place:Heihe River Basin

data size:1.7 MiB

data format:

Coordinate system: WGS84

Keywords

Theme:Microwave radiometer, airborne, ground remote sensing
	Subject:
	Time:2008
	Place:Heihe River Basin,  upstream cold region,  hydrological test area,  arrou intensive observation area

Space scope

|  | north： 34.92 |  |
| --- | --- | --- |
| west：100.95 |  | east：100.59 |
|  | south： 34.35 |  |

Service information

contacts:

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Wang shuguo,  Wang Xufeng,  Che Tao,  Xiao qing

author:Wang shuguo ,  Che Tao ,  Wang Xufeng ,  Xiao qing

manager:MIN Yufang

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Wang shuguo, Wang Xufeng, Che Tao, Xiao qing. Heihe River Integrated Remote Sensing joint test: L & K band airborne microwave radiometer data set in ARJO biandukou flight area (March 19, 2008). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.NIEER.2021.12.