Meteorological observation data set of yanwachuan basin in Loess Plateau Gully Region from 2001 to 2010

DOI：10.12072/ncdc.HWXFZhan.db0035.2020

CSTR：11738.11.ncdc.HWXFZhan.2020.98

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

This data set is the daily evaporation data of yanwachuan runoff station in yanwachuan watershed of Loess Plateau Gully Region from 2001 to 2010. 20cm evaporating dish is used to measure the daily evaporation. The observation is carried out every December and 31 days every month. The observation elements include monthly evaporation, monthly maximum evaporation, monthly minimum evaporation, annual statistical maximum and minimum evaporation, and winter ice onset time.

Data source description

The model of evaporation observation instrument is HY.AM3 The diameter of the dish is 20cm, and the observation accuracy is 0.1mm. It was observed at 8 o'clock every day and every five days during the freezing period. The evaporation capacity of evaporating dish is observed by rainfall cup of the same specification or special platform scale. The original water in the evaporating dish is 20 mm. At 8:00 a.m. of the previous day, the water in the cup is filled with 20 mm. After 24 hours, the remaining water in the dish is returned to the measuring cup, and the amount of reduction is the evaporation of the day. After observation, change water to keep the water volume of 20 mm in the dish. When there is rain falling into the dish, the actual evaporation is obtained by subtracting the precipitation during the observation and recording. When there is ice in the dish, use the weighing method to observe. Pour 20 mm clear water into the dish, weigh the weight (g) on the platform scale, and set the scale at the weight of 20 mm water. On the observation day, put the evaporation pan (containing ice) on the platform scale, and add water to the pan with a rainfall cup until the scale is flat. The water added is the accumulated evaporation.

Data processing method

Instrument monitoring, manual collection.

Data quality description

In the process of compilation and input of observation data, there are four levels of manual verification, including tabulation, primary calibration, recheck and audit, so as to ensure the data quality.

Base information

collect time:1981/01/01-1990/12/31

collect place:Yanwachuan basin, Ningxian County, Qingyang City, Gansu Province

altitude:980.0m- 1340.0m

data size:228.5 KiB

data format:

Coordinate system: 无

Keywords

Theme:Precipitation excerpt, precipitation, maximum precipitation, period precipitation
	Subject:Hydrometeorology 
	Time:1981 1982 1983 1984 1985 1986 1987 1988 1989 1990
	Place:Ningxian County,  Qingyang,  Yan wa Chuan,  Gansu Province

Space scope

|  | north： 35.74 |  |
| --- | --- | --- |
| west：107.92 |  | east：107.63 |
|  | south： 35.50 |  |

Service information

contacts:

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Yellow River Water Conservancy Commission Xifeng Soil and Water Conservation Scientific Experimental Station

author:YUAN Jing

manager:YUAN Jing

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Yellow River Water Conservancy Commission Xifeng Soil and Water Conservation Scientific Experimental Station. Meteorological observation data set of yanwachuan basin in Loess Plateau Gully Region from 2001 to 2010. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2020. https://cstr.cn/CSTR:11738.11.ncdc.HWXFZhan.2020.98.