Runoff data set of Nanxiaohegou watershed in Loess Plateau Gully Region, 1981-1990

DOI：10.12072/ncdc.HWXFZhan.db0032.2020

CSTR：11738.11.ncdc.HWXFZhan.2020.64

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

This data set is the runoff and flow field data of Nanxiaohegou watershed from 1981 to 1990. Its main purpose is to reduce water and sediment benefits of soil and water conservation and the influence of terrain factors on Soil and water loss on slope. The area of runoff field is 30.5-1640 m2, and the observation elements include runoff, sediment, soil moisture, coverage and rainfall. The runoff and sediment are observed manually in the runoff bucket (pool). The soil moisture content was measured by manual sampling and drying method. The coverage was measured manually and visually.

Data source description

Manual timing observation, instrument timing observation

Data processing method

(1) Runoff observation 1) Runoff pool: directly measure water after the end of runoff generation, and calculate the total runoff according to the water level volume curve. 2) Diversion bucket: the total amount of runoff can be calculated by the water distribution coefficient and the amount of water diversion. Total runoff (M3) = primary diversion bucket (M3) + secondary diversion bucket (M3) × number of primary diversion holes + +Runoff of n-stage diverter barrel (M3) × number of primary diversion holes × number of secondary diversion holes × Number of n-stage shunt holes (2) Sediment Observation 1) Runoff pool: observe immediately after the end of precipitation and runoff. The sludge and water in the collecting channel are swept into the runoff tank (bucket) and stirred evenly. Two to three columnar water samples (1000-3000ml in total) are taken, and 500ml water sample after mixing is taken as the standard sample. The sand content was determined by displacement method (Pycnometer method). 2) Diverter bucket: take samples separately, and the calculation is as follows: Total sediment (kg) = runoff of primary diversion bucket (M3) × sediment concentration (g / L) + runoff of secondary diversion bucket (M3) × sediment concentration (g / L) × number of primary diversion holes + +Runoff of n-stage diversion bucket (M3) × sediment concentration (g / L) × number of primary diversion holes × number of secondary diversion holes × Number of n-stage shunt holes

Data quality description

In the process of compilation and input of observation data, there are four levels of manual verification, including tabulation, primary calibration, recheck and audit, so as to ensure the data quality.

Base information

collect time:1981/01/01-1990/12/31

collect place:Nanxiaohegou watershed, houguanzhai Township, Xifeng District, Qingyang City, Gansu Province

altitude:1050.0m- 1423.0m

data size:794.5 KiB

data format:

Coordinate system: WGS84

Keywords

Theme:Precipitation excursion, Precipitation excerpt, Precipitation excursion, Maximum precipitation, Daily evaporation
	Subject:Hydrometeorology 
	Time:1981 1982 1983 1984 1985 1986 1987 1988 1989 1990
	Place:Qingyang,  Xifeng,  Nanxiaohegou,  Gansu Province

Space scope

|  | north： 35.75 |  |
| --- | --- | --- |
| west：107.62 |  | east：107.51 |
|  | south： 35.68 |  |

Service information

contacts:

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Yellow River Water Conservancy Commission Xifeng Soil and Water Conservation Scientific Experimental Station

author:YUAN Jing

manager:YUAN Jing

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Yellow River Water Conservancy Commission Xifeng Soil and Water Conservation Scientific Experimental Station. Runoff data set of Nanxiaohegou watershed in Loess Plateau Gully Region, 1981-1990. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.HWXFZhan.2020.64.