Precipitation observation data set of wangmaogou watershed in the first sub region of Loess Hilly and gully region from 1991 to 2000

DOI：10.12072/ncdc.HWSDZhan.db0017.2020

CSTR：11738.11.ncdc.HWSDZhan.2020.114

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

This data set is the precipitation observation data of wangmaogou watershed rainfall station from 1991 to 2000. SM1 ordinary rain gauge with diameter of 20cm and dsj2 siphon self recording rain gauge are used for parallel observation. When the self recording rain gauge fails, ordinary rain gauge is used instead. The data include daily precipitation data of rainfall station, precipitation extract data and maximum precipitation table of each period (extracted by minutes). The rainfall stations are shown in the basic information table of Jiuyuangou basin.

Data source description

Instrument continuous observation Manual timing observation

Data processing method

Precipitation observation instruments include SM1 ordinary rain gauge with a diameter of 20cm and dsj2 siphon type self recording rain gauge. The SM1 type adopts the 8-hour and 20-hour two-stage system for annual observation, mainly observing the starting and ending time of precipitation, and making statistics of precipitation. When the intermittent time of precipitation is more than 15 minutes, it is observed as two precipitation before and after the interval, and when the intermittent time is less than 15 minutes, it is recorded as one precipitation observation. If a precipitation crosses 8 hours and 20 hours, it should be measured at 8 hours and 20 hours. Dsj2 type adopts 24 section system for flood season observation. In case of failure of self recording rain gauge, the observation data of ordinary rain gauge is used for supplement.

Data quality description

The instrument calibration shall be carried out before the start of annual observation of rain gauge. In the process of data entry, there are four levels of manual verification, including tabulation, primary proofreading, rechecking and auditing, so as to ensure the data quality.

Base information

collect time:1991/01/01-2000/12/31

collect place:

altitude:2.0m- Nonem

data size:263.3 KiB

data format:

Coordinate system: WGS84

Keywords

Theme:Daily precipitation, Precipitation intensity, Maximum precipitation, Period precipitation
	Subject:Hydrometeorology 
	Time:1991 1992 1993 1994 1995 1996 1997 1998 1999 2000
	Place:Suide,  Yulin,  Wang Maogou,  Shaanxi Province

Space scope

|  | north： 37.59 |  |
| --- | --- | --- |
| west：110.34 |  | east：110.38 |
|  | south： 37.57 |  |

Service information

contacts:Min Yu Fang

phone:09314967596

mailbox:myf@lzb.ac.cn

Contact information

contributors:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

author:LEI Xin

manager:GUO Yumei , Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation). Precipitation observation data set of wangmaogou watershed in the first sub region of Loess Hilly and gully region from 1991 to 2000. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2020. https://cstr.cn/CSTR:11738.11.ncdc.HWSDZhan.2020.114.