Ozone daily data set of  Beijing Shangdianzi atmosphere  watch station 2018

DOI：10.12072/ncdc.SDZRAW.db0005.2021

CSTR：11738.11.ncdc.SDZRAW.2020.22

Share type:Login to Access

Category:Atmosphere

Datasets description

For the ozone concentration data collected at Shangdianzi station in 2018, the first step is to conduct observer level quality control; secondly, with the support of data quality control software, hourly data is generated through expert level quality control; and then daily data set is synthesized from hourly data. The daily data set includes collection time, device name, data value, datavaluemax, datavaluetmx, datavaluemin, datavaluetmn and other information, which can be used for daily change analysis of ozone data of main air pollution gases.

Data source description

The t9i ozone analyzer produced by Thermo Fisher company was used to collect the concentration data of nitrogen dioxide at minute level. The data unit was ppb and the data format was excel (*. XLS).

Data processing method

First, the original data is filtered by singular value, zero span check and other processes to eliminate the problematic observation data. Then the data are corrected according to the calibration curve, and the hourly and daily mean values are calculated according to the effective data.

Data quality description

The data has undergone two levels of quality control: observer level and expert level, and the data quality is good.

Base information

collect time:2018/01/01-2018/12/31

collect place:Atmospheric background station of Shangdianzi District, Shangdianzi Village Back Hill, Gaoling Town, Miyun District, Beijing

altitude:293.1m- 293.5m

data size:50.5 KiB

data format:Excel

Coordinate system: WGS84

Keywords

Theme:Shangdianzi, ozone, atmospheric background
	Subject:Atmospheric chemistry 
	Time:2018
	Place:Shangdianzi Village,  Beijing,  Gaoling Town

Space scope

|  | north： 41.00 |  |
| --- | --- | --- |
| west：117.07 |  | east：117.13 |
|  | south： 40.63 |  |

Service information

contacts:Min Yu Fang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:SHI Qingfeng,  MA Zhiqiang

author:Quan Weijun ,  MIN Yufang

manager:Quan Weijun , SHI Qingfeng

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

SHI Qingfeng, MA Zhiqiang. Ozone daily data set of  Beijing Shangdianzi atmosphere  watch station 2018. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.SDZRAW.2020.22.