2014 Lenglongling Area GPS control point data set

DOI：10.12072/ncdc.DIMaC.db0012.2020

CSTR：11738.11.ncdc.DIMaC.2020.96

Share type:Apply to Access

Category:Glacier

Datasets description

This data set is aimed at Lenglongling area. The data set includes the serial number, longitude, latitude and elevation of the control points. The data set contains 485 records, mainly through the purchase of the national first-class control points in the region, the use of national first-class control points for static measurement, synchronous measurement time of more than 1 hour, through the solution to obtain the glacier area control points; through dynamic differential GPS dynamic joint survey. In the data set, longitude, unit degree (°), latitude, unit degree (°), elevation, that is, the glacier surface elevation in the current year, unit meter (m).

Data source description

This data set is mainly measured data, and the main data sources are static measurement of national first-class control points and dynamic differential GPS dynamic joint survey.

Data processing method

Through purchasing the first level national control points in the region, the global position system (GPS) of Trimble 5800 model is used to synchronously measure the time of more than 1 hour, and the time is measured through the Trimble Geomatics office The software is used to calculate and obtain the control points in the glacier area; then the dynamic differential GPS is used to obtain the dynamic joint measurement, and the dynamic differential GPS measurement process is standardized and standardized, and strictly rechecked.

Data quality description

4-6 stations of China Survey X20 or T5 GNSS are used for joint survey, the synchronous measurement time is more than 1 hour, and 1-3 national control points are used for joint survey; the result calculation accuracy is controlled within 2cm

Base information

collect time:2014/08/01-2014/08/31

collect place:Lenglongling

data size:40.8 KiB

data format:

Coordinate system: WGS84

Keywords

Theme:Lenglongling, control point, GPS
	Subject:
	Time:2014
	Place:Lenglongling

Space scope

|  | north： 37.77 |  |
| --- | --- | --- |
| west：101.20 |  | east：102.20 |
|  | south： 37.37 |  |

Service information

contacts:Li Hongxing

phone:0931-4967592

mailbox:lihongxing@lzb.ac.cn

Contact information

contributors:Cao Bo,  Cui hang

author:WU Kunpeng

manager:Cao Bo

Project information

number:2013FY111400，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Cao Bo, Cui hang. 2014 Lenglongling Area GPS control point data set. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2020. https://cstr.cn/CSTR:11738.11.ncdc.DIMaC.2020.96.