Data set of multi-scale surface fluxes and meteorological elements in Haihe River Basin: Daxing station eddy correlator (2008-2010)

DOI：10.12072/ncdc.westdc.db3829.2023

CSTR：11738.11.ncdc.Westdc.2020.424

Share type:Apply to Access

Category:Meteorology

Datasets description

The data set includes the observation data of eddy correlator from January 1, 2008 to December 31, 2010. The site is located in Weishanzhuang, Daxing District, Beijing. The underlying surface is corn / wheat, melon and fruit. The longitude and latitude of the observation point are 116.4271e, 39.6213n and 20m above sea level. The observation data released by the eddy correlator include: date / time, wind direction WDIR (?), horizontal wind speed wnd (M / s), standard deviation of lateral wind speed STD uuy (M / s), ultrasonic virtual temperature TV (℃), water vapor density H2O (g / m3), carbon dioxide concentration CO2 (mg / m3), friction velocity ustar (M / s), obkhov length, sensible heat flux HS (w / m2), latent heat flux Le (w / m2) ）, carbon dioxide flux FC (mg / (M2S)), quality mark of sensible heat flux QA ﹣ HS, quality mark of latent heat flux QA ﹣ Le, quality mark of carbon dioxide flux QA ﹣ FC. (76 & lt; Δ st & lt; 100, ITC & lt; 100); 7: (Δ st & lt; 250, ITC & lt; 250; 8: (Δ st & lt; 1000, ITC & lt; 1000); the rest is 9). The meaning of data time, for example, 0:30 represents the average of 0:00-0:30; data is stored in *. XLS format. For observation test or station information, please refer to Jia et al. (2012), and for data processing, please refer to Liu et al. (2013).

Data source description

The acquisition frequency of the eddy correlator is 10Hz, the frame height is 3M, the ultrasonic direction is 295 degrees (0 degrees in the north direction), and the distance between the ultrasonic anemometer and the CO2 / H2O analyzer is 18cm.

Data processing method

The published data is the 30 minute data obtained from the post-processing of the original 10 Hz data collected by the edire software. The main processing steps include: outlier elimination, delay time correction, coordinate rotation (plane fitting), frequency response correction, ultrasonic virtual temperature correction and density (WPL) correction, etc.

Data quality description

The quality evaluation of each flux value is mainly the test of atmospheric stability (Δ st) and turbulence similarity (ITC). The 30min flux value output by edire software was also screened: (1) data in case of instrument error; (2) data in 1H before and after precipitation; (3) data with loss rate greater than 3% in every 30min of 10Hz original data; (4) observation data with weak turbulence at night (U * less than 0.1M / s). The average period of observation data is 30 minutes, 48 data in a day, and the missing data is marked as - 6999.

Base information

collect time:2008/01/01-2010/12/31

collect place:

data size:8.5 MiB

data format:

Coordinate system: 无

Keywords

Theme:Latent heat flux, CO2 flux, Eddy correlator, Sensible heat flux
	Subject:
	Time:2008 2010
	Place:Haihe River Basin,  Daxing District,  Beijing

Space scope

|  | north： 39.62 |  |
| --- | --- | --- |
| west：116.42 |  | east：116.42 |
|  | south： 39.62 |  |

Service information

contacts:Li Hongxing

phone:0931-4967592

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Liu shaomin,  Xu ziwei

author:Xu ziwei

manager:Xu ziwei

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Liu shaomin, Xu ziwei. Data set of multi-scale surface fluxes and meteorological elements in Haihe River Basin: Daxing station eddy correlator (2008-2010). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2020. https://cstr.cn/CSTR:11738.11.ncdc.Westdc.2020.424.

Article citation:

Liu Shaomin, Xu Ziwei, Zhu Zhongli, Jia Zhenzhen, Zhu Mingjia. Measurements of evapotranspiration from eddy-covariance systems and large aperture scintillometers in the Hai River Basin, China[J]. Journal of Hydrology, 2013, 487: 24-38.

Jia Zhenzhen, Liu Shaomin, Xu Ziwei, Chen Yujie, Zhu Mingjia. Validation of remotely sensed evapotranspiration over the Hai River Basin, China[J]. Journal of Geophysical Research, 2012, 117, (21).