Runoff sediment observation dataset of Qiaozigou watershed runoff station in the third sub-region of Loess Hills and Gullies area from 2011-2020

DOI：10.12072/ncdc.HWTSZhan.db0021.2020

CSTR：11738.11.ncdc.HWTSZhan.2020.59

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

<pre><code> &lt;pre&gt;&lt;code&gt; This data set is the runoff and sediment observation data of qiaozigou watershed in the third sub region of Loess Hilly and gully region from 2011 to 2020. Qiaozigou watershed is a small watershed for parallel observation and test of governance and non governance, qiaozidongou watershed is a governance watershed, qiaozixigou watershed is a non governance watershed. The data set includes the measured discharge results table, flood hydrological elements extract table, successive flood test results table, daily average discharge table, daily average sediment concentration table and daily average suspended sediment transport rate table.

Data source description

<pre><code> &lt;pre&gt;&lt;code&gt; &amp;lt;p&amp;gt;Manual fixed point and fixed time observation&amp;lt;/p&amp;gt;

Data processing method

<pre><code> &lt;pre&gt;&lt;code&gt; For runoff observation, the buoy method is used to measure the flow velocity, and the velocity area method is used to calculate the flow; sediment sampling and flow measurement are carried out simultaneously, and the sampling times are the same as the flow measurement times. The water side one point method is used to take sediment samples, and the replacement method is used to measure the sediment concentration.

Data quality description

<pre><code> &lt;pre&gt;&lt;code&gt; According to the provisions of "soil and water conservation test specification" (sd239) and "soil and water conservation test specification" (sl419), the data are observed, sorted and compiled by professional technicians.

Base information

collect time:2011/01/01-2020/12/31

collect place:Qiaozigou, Luoyugou watershed, Qinzhou District, Tianshui City, Gansu Province

altitude:1235.0m- 1707.2m

data size:397.6 KiB

data format:

Coordinate system: 无

Keywords

Theme:Annual sediment discharge, annual runoff, total flood volume, flood sediment discharge, soil loss per unit area, runoff coefficient, daily average flow, daily average sediment concentration, daily average suspended sediment transport rate, maximum flow rate
	Subject:
	Time:2011 2012 2013 2014 2015 2016 2017 2018 2019
	Place:Tianshui,  qiaozigou,  Gansu Province

Space scope

|  | north： 34.63 |  |
| --- | --- | --- |
| west：105.71 |  | east：105.73 |
|  | south： 34.60 |  |

Service information

contacts:

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:

author:Liu Xiao

manager:

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

. Runoff sediment observation dataset of Qiaozigou watershed runoff station in the third sub-region of Loess Hills and Gullies area from 2011-2020. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2022. https://cstr.cn/CSTR:11738.11.ncdc.HWTSZhan.2020.59.