Precipitation Observation Dataset of Wangmaogou Basin in the First Sub area of Loess Hilly and Gully Region from 2001 to 2010

DOI：10.12072/ncdc.HWSDZhan.db0004.2020

CSTR：11738.11.ncdc.HWSDZhan.2020.97

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

This data set is the precipitation observation data of wangmaogou watershed rainfall station from 2001 to 2010. Dsj2 siphon rain gauge and SM1 ordinary rain gauge with diameter of 20cm are used for parallel observation. When the self recording rain gauge fails, ordinary rain gauge is used instead. The data include station network layout map, daily precipitation scale, precipitation extract table and maximum precipitation table of each period (1). For the basic information of rainfall stations in wangmaogou basin, please refer to the situation description of wangmaogou watershed from 2001 to 2010.

Data source description

Instrument continuous observation, artificial timing observation

Data processing method

Precipitation observation instruments include dsj2 siphon rain gauge and SM1 ordinary rain gauge with diameter of 20cm. Dsj2 self recording rain gauge adopts 24 segment system for flood season observation, SM1 ordinary rain gauge adopts 8:00 and 20:00 two-stage system for annual observation. In case of failure of self recording rain gauge, observation data of ordinary rain gauge are used for supplement. It mainly observes the beginning and ending time of precipitation and statistics of precipitation. When the intermittent time of precipitation is more than 15min, it is observed as two precipitation before and after the interval. When the intermittent time is less than 15min, it is recorded as a precipitation observation. If a precipitation crosses 8:00 and 20:00, it should be measured at 8:00 and 20:00.

Data quality description

The instrument calibration shall be carried out before the start of annual observation of rain gauge. In the process of data entry, there are four levels of manual verification, including tabulation, primary proofreading, rechecking and auditing, so as to ensure the data quality.

Base information

collect time:2001/01/01-2010/12/31

collect place:Wangmaogou watershed, Suide County, Yulin City, Shaanxi Province

data size:236.7 KiB

data format:

Coordinate system: 无

Keywords

Theme:Annual rainfall, daily rainfall, interval rainfall, maximum rainfall
	Subject:Hydrometric survey 
	Time:2001 2002 2003 2004 2005 2006 2007 2008 2009 2010
	Place:Wang Maogou,  Suide,  Shaanxi Province,  Shaanxi Province

Space scope

|  | north： 37.59 |  |
| --- | --- | --- |
| west：110.34 |  | east：110.38 |
|  | south： 37.57 |  |

Service information

contacts:

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

author:LEI Xin

manager:GUO Yumei , Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation). Precipitation Observation Dataset of Wangmaogou Basin in the First Sub area of Loess Hilly and Gully Region from 2001 to 2010. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2020. https://cstr.cn/CSTR:11738.11.ncdc.HWSDZhan.2020.97.