Data set of glacier surface elevation change in youyifeng area of Altai mountain from 1959 to 2008

DOI：10.12072/ncdc.DIMaC.db0043.2020

CSTR：11738.11.ncdc.DIMaC.2020.113

Share type:Apply to Access

Category:Glacier

Datasets description

This data set aims at the area of friendship peak in Altai mountain, including glaciers and surrounding areas in the area of friendship peak in Altai mountain. Using DEM from topographic map, SRTM (2000) and DEM extracted from remote sensing image, the elevation change of glacial surface in the center of glaciation can be obtained through difference calculation. Only for the elevation change of glacier surface in Youyi peak area of Altai mountain from 1959 to 2008, it is not supplemented and improved, and it is fully open sharing. The dataset contains two files, elevationchange1959-2000 and elevationchange2000-2008. The unit of measurement of the field is meter (m). The data set mainly includes remote sensing image extraction data, which only represents the glacier surface elevation change at the acquisition time of remote sensing image. The time is 1959-2000 and 2000-2008. The data collected in this data set are mainly from the area of friendship peak in Altai mountain, including glaciers and surrounding areas.

Data source description

This data set is mainly interpretation data, and the main data sources are topographic map DEM, SRTM (2000) and remote sensing image DEM.

Data processing method

Using DEM from topographic map, SRTM (2000) and remote sensing image, the elevation change of glacier surface in the center of glaciation was obtained by difference calculation.

Data quality description

After the accuracy test of DEM in different periods, the cosine relationship between slope direction and elevation difference is used for plane and elevation correction; then the relationship between maximum curvature and elevation difference is used to reduce the error caused by different resolutions, and the registration and relative accuracy evaluation of DEM in two periods are carried out to obtain the elevation change of glacier in different periods

Base information

collect time:1959/01/01-2008/12/31

collect place:

data size:3.2 MiB

data format:

Coordinate system: WGS84

Keywords

Theme:Altai Friendship Peak, DEM, Ice Elevation Change
	Subject:physical geography 
	Time:1959-2008
	Place:Ice Elevation Change,  DEM,  Altai Mountain Friendship Peak

Space scope

|  | north： 49.17 |  |
| --- | --- | --- |
| west： 87.57 |  | east： 87.89 |
|  | south： 49.03 |  |

Service information

contacts:Li Hongxing

phone:0931-4967592

mailbox:lihongxing@lzb.ac.cn

Contact information

contributors:LIU Shiyin,  Wei junfeng

author:WU Kunpeng

manager:WU Kunpeng , LIU Shiyin

Project information

number:2013FY111400，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

LIU Shiyin, Wei junfeng. Data set of glacier surface elevation change in youyifeng area of Altai mountain from 1959 to 2008. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.DIMaC.2020.113.