Tianjin Jinhai Lake water nanofiltration desalination rate data set (July-September 2020)

DOI：10.12072/ncdc.MDBW.db0018.2021

CSTR：11738.11.ncdc.MDBW.2021.16

Share type:Login to Access

Category:Others

Datasets description

After the lake water of Jinhai Lake was pre-treated by precision filtration-ultrafiltration, the pre-treated effluent was pumped by a high-pressure pump (CRN3-29, Grundfos, China) into the desalination system fitted with different quantities of nano-filtration membranes, which were made of commercialized polyamide 4040 rolled-type membrane modules (VNF2, Guiyang Times Warden Co., Ltd.). After the pressure and flow rate were stabilized for half an hour, the salinity data of the produced water was recorded and the desalination rate was calculated. The operating pressure was 1.3MPa, the salt content of influent water was 7.5g/L, the water temperature was 30±2℃, the influent flow rate was 2m3/h, and the number of membrane modules was 6-8. The measurement is accurate to two decimal places. The desalination rate of the nanofiltration system is higher than 80%, reaching the assessment index of the task book. This dataset is mainly applied to the study of brackish water desalination.

Data source description

Experimentally obtained.

Data processing method

Operating pressure 1.3 MPa, feed water salinity 7.5 g/L, water temperature 30 ± 2 ℃, feed water flow rate 2 m3/h when the experiments to obtain the nanofiltration system produced water salinity, was calculated to obtain the desalination rate.

Data quality description

(1) Data production process: After the lake water of Jinhai Lake was pre-treated by precision filtration-ultrafiltration, the pre-treated effluent was pumped by a high-pressure pump (CRN3-29, Grundfos, China) into the desalination system fitted with different quantities of nano-filtration membranes, which were made of commercialized polyamide 4040 rolled-type membrane modules (VNF2, Guiyang Times Warden Co., Ltd.). After the pressure and flow rate were stabilized for half an hour, the salinity data of the produced water was recorded and the desalination rate was calculated.   (2) Methods: Operating pressure 1.3MPa, salt content of feed water 7.5g/L, water temperature 30± 2℃, feed water flow rate 2m3/h, the number of membrane modules is 6-8 pieces. The measurement is accurate to two decimal places.   (3) Result: the desalination rate of nanofiltration system is higher than 80%, which reaches the assessment index of the task book.

Base information

collect time:2020/07/20-2020/09/20

collect place:Jinhai Lake, Binhai New Area, Tianjin

data size:109.1 KiB

data format:excel

Coordinate system: 无

Keywords

Theme:Nanofiltration, desalination rate, high pressure pump, influent salt content
	Subject:Hydrochemistry 
	Time:2020
	Place:Jinhai Lake

Space scope

|  | north： |  |
| --- | --- | --- |
| west： |  | east： |
|  | south： |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:DU Runhong

author:DU Runhong

manager:DU Runhong

Project information

number:2017YFE0114200，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

DU Runhong. Tianjin Jinhai Lake water nanofiltration desalination rate data set (July-September 2020). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.MDBW.2021.16.