1km permafrost thickness map of Northeast China (2023-2024)

DOI：10.12072/ncdc.nieer.db7273.2026

CSTR：11738.11.NCDC.NIEER.DB7273.2026

Share type:Login to Access

Category:Frozen soil

Datasets description

Data source description

Observation data: including ground temperature observation data from 104 stations (deep hole>20m and shallow hole data).  Environmental factors: precipitation (PRE), surface melting index (TDD), terrain location index (TPI), etc.

Data processing method

Ground temperature gradient modeling: Establish a ground temperature gradient model using deep hole ground temperature data.  Data inversion: Invert the deep ground temperature of shallow hole stations and calculate the depth of permafrost floor for many years.  Spatial mapping: Using the Random Forest algorithm, combined with environmental factors, to spatially map the thickness of permafrost in the entire region.  Accuracy evaluation: The classification accuracy of the random forest model is 0.74; The simulation results show that the standard deviation of the average frozen soil thickness is about ± 10 m.

Data quality description

This data is modeled using machine learning methods to calculate confusion matrix and overall accuracy. The results show that the model has high consistency.

Base information

collect time:2023/01/01-2024/12/31

collect place:Northeast China

data size:2.6 MiB

data format:*.tif

Coordinate system: WGS84

Projection
       WGS_1984_Albers

Keywords

Theme:Permafrost, distribution of permafrost thickness, 1km
	Subject:geography 
	Time:2023 2024
	Place:Northeast China

Space scope

|  | north： 53.56 |  |
| --- | --- | --- |
| west：111.15 |  | east：135.08 |
|  | south： 38.73 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:LIU Guangyue,  Zhao Lin

author:Lu Yingying

manager:Wang Chong

Project information

number:2022FY100700，
      name:Survey of Permafrost Conditions and Freeze-Thaw Damage in the High-Latitude Regions of Northeast China

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

LIU Guangyue, Zhao Lin. 1km permafrost thickness map of Northeast China (2023-2024). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2026. https://cstr.cn/CSTR:11738.11.NCDC.NIEER.DB7273.2026.