Data set of bridge runoff field in Qiaogou watershed, the first sub region of Loess Hilly and gully region in 2022

DOI：10.12072/ncdc.hwsdzhan.db4104.2023

CSTR：11738.11.NCDC.HWSDZHAN.DB4104.2023

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

This dataset contains runoff field data of different geomorphic types in the Qiaogou watershed of the first sub area of the Loess Hilly and Gully Region in 2022. Observe the secondary precipitation, runoff, and sediment yield of runoff generated by different geomorphic types of runoff fields in the Qiaogou watershed. The data includes the results of successive runoff and sediment tests for different geomorphic types of runoff fields in Qiaogou, as well as the measured results of soil moisture content. The runoff field situation can be found in the "2022 Explanation of Different Landform Types of Runoff Fields in Qiaogou"

Data source description

Instrument continuous observation, manual timed observation

Data processing method

1. Rainfall observation: there are SM1 ordinary rain gauge with diameter of 20cm and dsj2 siphon self recording rain gauge. The data of Qiaogou No.5 rainfall station are used for the precipitation of runoff plots of different geomorphic types in Qiaogou. 2.Sediment sampling and treatment: different observation facilities are adopted for runoff and sediment, including triangular weir, triangular groove, water distribution tank and runoff pool. Sand ocean is collected by triangular weir and triangular groove in the process of runoff generation; the water level of runoff pool is read immediately after precipitation and runoff is terminated, and runoff is calculated according to the area and water level of water distribution tank and runoff pool The sediment sample (500cm3) was taken by mixing water evenly, and the replacement method was used for sample disposal. 3.Soil moisture content: in a representative fixed site, after removing the surface weeds, samples were taken at a depth of 30 cm and determined by drying (weighing) method.

Data quality description

In the process of integration and input of observation data, there are four levels of manual verification, which are tabulation, primary calibration, re calibration and audit, to ensure the data quality.

Base information

collect time:2022/01/01-2022/12/31

collect place:Qiaogou River Basin, Suide County, Yulin City, Shaanxi Province

data size:87.0 KiB

data format:

Coordinate system: WGS84

Keywords

Theme:Precipitation intensity, Sediment concentration, Scour volume, Runoff depth, Runoff coefficient, runoff, Precipitation
	Subject:
	Time:
	Place:Qiaogou,  Suide,  Yulin,  Shaanxi Province

Space scope

|  | north： 37.50 |  |
| --- | --- | --- |
| west：110.29 |  | east：110.30 |
|  | south： 37.49 |  |

Service information

contacts:Min Yu Fang

phone:09314967597

mailbox:myf@lzb.ac.cn

Contact information

contributors:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation),  LEI Xin

author: ,

manager:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation) , LEI Xin

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation), LEI Xin. Data set of bridge runoff field in Qiaogou watershed, the first sub region of Loess Hilly and gully region in 2022. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2023. https://cstr.cn/CSTR:11738.11.NCDC.HWSDZHAN.DB4104.2023.