Observation data set of soil moisture content of runoff field in Qiaogou watershed of the first sub region of Loess Hilly and gully region from 2021 to 2022

DOI：10.12072/ncdc.HWSDZhan.db2417.2022

CSTR：11738.11.ncdc.HWSDZhan.db2417.2022

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

This data set is the observation data of soil moisture content in runoff field of different geomorphic types in Qiaogou watershed, the first sub region of loess hilly and gully region from 2021 to 2022. Qiaogou drainage basin is located in Suide County, Yulin City, Shaanxi Province, with a drainage area of 0.45 square kilometers, a leaf shaped drainage basin, a length width of 1.4 km and 0.32 km, a relative height difference of 132 m, and a gully density of 5.4 km ²， It belongs to the first level tributary of Peijiamao Valley (the first level tributary of Wuding River). In this data set, the soil water content of 8 runoff plots of different geomorphic types in Qiaogou is observed. Soil samples are collected every day and measured on rainy days. If there is no precipitation, no measurement is made. For the basic information of runoff field, see the 2021 to 2022 Description of Runoff Field of Different Landforms in Qiaogou. The main observation indicators include: site number, measurement times, sampling date, soil moisture content at the measurement point (%, sampling depth 10-20cm), and precipitation between two measurement times (mm).

Data source description

Manual observation

Data processing method

At representative fixed locations, after removing surface weeds, samples were taken at depths of 10-20cm and 30cm, and measured using the drying (weighing) method

Data quality description

The observation data has been manually checked at four levels in the process of compilation and entry, including tabulation, initial calibration, re calibration and review, to ensure the data quality

Base information

collect time:2021/01/01-2022/12/31

collect place:Qiaogou watershed, Suide County, Yulin City, Shaanxi Province

data size:74.2 KiB

data format:

Coordinate system: WGS84

Keywords

Theme:Soil moisture content, sampling depth, precipitation between two measurements
	Subject:
	Time:2021 2022
	Place:Yulin,  Qiaogou,  Suide,  Shaanxi Province

Space scope

|  | north： 37.50 |  |
| --- | --- | --- |
| west：110.29 |  | east：110.30 |
|  | south： 37.49 |  |

Service information

contacts:Min Yu Fang

phone:0931-4967596

mailbox:myf@lzb.ac.cn

Contact information

contributors:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

author: ,

manager:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation) , LEI Xin

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation). Observation data set of soil moisture content of runoff field in Qiaogou watershed of the first sub region of Loess Hilly and gully region from 2021 to 2022. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2022. https://cstr.cn/CSTR:11738.11.ncdc.HWSDZhan.db2417.2022.