Observation data set of runoff and sediment in Jiuyuangou watershed, the first sub region of Loess Hilly and gully region, 1980-1990

DOI：10.12072/ncdc.HWSDZhan.db0059.2020

CSTR：11738.11.ncdc.HWSDZhan.2021.131

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

This data set is the runoff and sediment observation data of Jiuyuangou River Basin from 1980 to 1990. Through the water level observation and sediment sampling of runoff station section, the runoff and sediment amount during runoff generation are calculated. The data set includes the measured discharge results table of Jiuyuangou River Basin and wangmaozhuang runoff station, the extract table of flood hydrological elements, the result table of successive flood test, the daily average discharge table, the daily average sediment concentration table and the daily average sediment transport rate table.

Data source description

Manual timing observation

Data processing method

1. The water level observation method of runoff station is: read the water gauge number of the flow measurement building on the flow measurement section. In non flood season, 8:00 and 20:00 respectively are observed once a day. If there is rainfall, observe twice at 8:00 and 20:00 respectively when the daily water level changes slowly; observe four times at 2:00, 8:00, 14:00 and 20:00 respectively when the daily water level changes greatly or there are slow peaks and valleys. During the flood season, when the water level changes violently, it shall be observed in multiple of 6 minutes. 2. Flow measurement method of channel diameter flow station: when the water level is higher than the triangular channel, the buoy method is used to measure the flow in the rectangular spillway, and the buoy coefficient is based on the buoy test results of huangweizizhou runoff test station. 3. Flow measurement method of wangmaozhuang runoff station: adopt the calibrated natural section observation and the calibrated push flow formula for flow calculation. 4. Sediment sampling and treatment: the sediment in the flow measurement section is sampled by the thalweg buoy method, and the number of sediment sampling depends on the number of changes in the flood water level. In order to control the change of sediment concentration, the number of sampling is increased during the flood peak. The sample was treated by displacement method.

Data quality description

Manual observation shall be conducted by professional technicians; during data entry, four levels of manual verification shall be conducted, including tabulation, primary calibration, re calibration and audit, so as to ensure data quality.

Base information

collect time:1980/01/01-1990/12/31

collect place:

altitude:2.0m- Nonem

data size:778.4 KiB

data format:

Coordinate system: WGS84

Keywords

Theme:Runoff modulus, Runoff coefficient, Sediment transport rate, water level, Erosion modulus, flow, Sediment concentration, Runoff depth, Runoff, Soil loss
	Subject:Hydrometeorology 
	Time:1980 1981 1982 1983 1984 1985 1986 1987 1988 1989 1990
	Place:Jiuyuangou,  Suide,  Yulin,  Shaanxi Province

Space scope

|  | north： 37.65 |  |
| --- | --- | --- |
| west：110.26 |  | east：110.39 |
|  | south： 37.54 |  |

Service information

contacts:Min Yu Fang

phone:09314967597

mailbox:myf@lzb.ac.cn

Contact information

contributors:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

author:LEI Xin ,  LiuSijun

manager:GUO Yumei , Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation). Observation data set of runoff and sediment in Jiuyuangou watershed, the first sub region of Loess Hilly and gully region, 1980-1990. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2020. https://cstr.cn/CSTR:11738.11.ncdc.HWSDZhan.2021.131.