Heihe integrated remote sensing joint test: omis-ii data set of airborne imaging spectrometer in Zhangye Yingke huazhaizi flight area (June 4, 2008)

DOI：10.12072/ncdc.NIEER.db1726.2022

CSTR：11738.11.ncdc.NIEER.2021.18

Share type:Login to Access

Category:Remote Sensing

Datasets description

This data set was acquired by airborne omis-ii sensor on June 4, 2008 in Zhangye Daman huazhaizi flight area. Because omis-ii is a scanning imaging sensor, the original data is obviously affected by radiation distortion and the change of aircraft attitude, and the spatial position relationship of adjacent pixels in the image is unstable, so the data released here is the data after radiation correction, radiation calibration and geometric coarse correction. The radiation correction adopts the moment match method, which can eliminate the non-uniformity of radiation response, strip noise and smile phenomenon. Radiometric calibration adopts the calibration coefficient measured in the laboratory before flight, and the calibration unit is w / (m ^ 2 · SR · UM). Geometric coarse correction uses POS data obtained synchronously with the image to reconstruct the aerial belt image, and the visual quality of the image has been greatly improved. The geometric coarse correction image can only be matched with other data with geographical coordinates after geometric fine correction by using geometric control points. The geometric control points of each navigation belt are provided here, and the user can carry out geometric fine correction by himself. As an example, the geometric fine correction and atmospheric correction images around Yingke oasis station and huazhaizi desert station are provided here. In addition, because the total scanning field of view of omis-ii sensor reaches 73 ° and the window of the aircraft is small, the left and right ends of the scanning line are blocked by the cabin. Although the image is restored after radiation correction, it is recommended to only use the image in the middle that is not blocked. The omis-ii original data without geometric coarse correction and the synchronously obtained short wave infrared hyperspectral (swphi) original data shall be archived, which can be obtained only after the application is submitted and approved. The processing time of geometric rough correction is October 2008, and the processing time of radiation correction and calibration is January 2010.

Data source description

On June 4, 2008, in Zhangye Daman huazhaizi flight area. The aerial flight test of imaging spectrometer omis-ii was carried out. The main variable of ground synchronous observation is surface temperature. This data can provide a basic data set for surface temperature inversion and verification of airborne satellite remote sensing data.

Data processing method

The processing time of geometric rough correction is October 2008, and the processing time of radiation correction and calibration is January 2010.

Data quality description

Good data quality

Base information

collect time:2008/06/17-2008/06/17

collect place:Heihe River Basin, intensive observation area of huazhaizi desert, hydrological test area of arid area in the middle reaches, intensive observation area of Yingke oasis and intensive observation area of Zhangye City

data size:17.9 GiB

data format:excel

Coordinate system: WGS84

Keywords

Theme:Imaging spectrometer omis-ii, airborne, ground remote sensing
	Subject:physical geography 
	Time:2008
	Place:Heihe River Basin,  huazhaizi desert intensive observation area,  drought hydrological test area in the middle reaches,  Yingke oasis intensive observation area,  Zhangye intensive observation area

Space scope

|  | north： 39.00 |  |
| --- | --- | --- |
| west：100.20 |  | east：100.50 |
|  | south： 38.50 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:LI Xin,  Ma mingguo

author:Ma mingguo

manager:Ma mingguo

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

LI Xin, Ma mingguo. Heihe integrated remote sensing joint test: omis-ii data set of airborne imaging spectrometer in Zhangye Yingke huazhaizi flight area (June 4, 2008). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.NIEER.2021.18.