Vegetation coverage data set of Heihe national key prevention area in Qilian Mountains in 2024

DOI：10.12072/ncdc.hhstbc.db6952.2025

CSTR：11738.11.NCDC.HHSTBC.DB6952.2025

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

<p>The vegetation coverage data set of Heihe national key prevention area in Qilian Mountain in 2024 includes the vegetation coverage statistical tables of Ejina Banner, Inner Mongolia Autonomous Region, Yongdeng County, Tianzhu County, Ganzhou District, Sunan County, Minle County, Linze County, Gaotai County, Jinta County, Menyuan Hui Autonomous County and Qilian County in Gansu Province in 2024, based on the processing of satellite remote sensing image with spatial resolution of 2m, The storage format is xlsx.</p>

Data source description

<ol> The data sources of land use are ZY-3 and Gao FEN-1 satellite images, which are mainly obtained from the information center of the Ministry of water resources. 2. Vegetation data sources are ZY-3 and Gao FEN-1 satellite images, which are mainly obtained from the information center of the Ministry of water resources. 3.1:50000 DEM is mainly obtained from the information center of the Ministry of water resources.

Data processing method

<ol> Based on the thematic maps of land use, vegetation coverage and slope, the soil erosion intensity was graded by ArcGIS software. 2. The land use processing method is based on ecognition software platform, using the method of object-oriented computer automatic classification and manual visual interpretation to extract the land use data of the study area year by year. Finally, three methods are used to verify the accuracy of the data: field sample point survey, high-resolution image recognition and Google Earth sample point recognition. 3. The processing method of vegetation coverage is based on remote sensing estimation, and the normalized vegetation index (NDVI) is used to estimate the vegetation coverage by pixel dichotomy model. First, the NDVI of each pixel is calculated by using the near-infrared and red band data of multispectral images. Then, the model is used to calculate the vegetation coverage of the whole region, and the vegetation coverage is classified according to the classification rules. Finally, the land use type data obtained from remote sensing interpretation and the vegetation coverage data obtained from remote sensing estimation are used for superposition operation, The vegetation coverage information of each pixel was obtained. 4. Slope data processing method is based on 1:50000 DEM extraction.

Data quality description

<ol> Based on the thematic maps of land use, vegetation coverage and slope, the soil erosion intensity was graded by ArcGIS software. 2. The land use processing method is based on ecognition software platform, using the method of object-oriented computer automatic classification and manual visual interpretation to extract the land use data of the study area year by year. Finally, three methods are used to verify the accuracy of the data: field sample point survey, high-resolution image recognition and Google Earth sample point recognition. 3. The processing method of vegetation coverage is based on remote sensing estimation, and the normalized vegetation index (NDVI) is used to estimate the vegetation coverage by pixel dichotomy model. First, the NDVI of each pixel is calculated by using the near-infrared and red band data of multispectral images. Then, the model is used to calculate the vegetation coverage of the whole region, and the vegetation coverage is classified according to the classification rules. Finally, the land use type data obtained from remote sensing interpretation and the vegetation coverage data obtained from remote sensing estimation are used for superposition operation, The vegetation coverage information of each pixel was obtained. 4. Slope data processing method is based on 1:50000 DEM extraction.

Base information

collect time:2024/01/01-2024/12/31

collect place:National key prevention area of Heihe River in Qilian Mountain

data size:3.3 MiB

data format:

Coordinate system: 无

Keywords

Theme:Statistical table of vegetation coverage
	Subject:
	Time:2024
	Place:Heihe national key prevention area of Qilian Mountain

Space scope

|  | north： |  |
| --- | --- | --- |
| west： |  | east： |
|  | south： |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:

author:

manager:

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

. Vegetation coverage data set of Heihe national key prevention area in Qilian Mountains in 2024. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2025. https://cstr.cn/CSTR:11738.11.NCDC.HHSTBC.DB6952.2025.