Runoff and sediment observation data set of wangmaogou watershed in the first sub region of Loess Hilly and gully region from 2011 to 2020

DOI：10.12072/ncdc.HWSDZhan.db0040.2020

CSTR：11738.11.ncdc.HWSDZhan.2020.91

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

<pre><code> &lt;p&gt;This data set is the runoff and sediment observation data of a runoff station in wangmaogou watershed from 2011 to 2020. Through the water level observation and sediment sampling of runoff measurement section of runoff station, the runoff and sediment amount during runoff generation are calculated. The data set includes flow measurement section diagram of runoff station, extract table of flood hydrological elements, result table of successive flood test, daily average discharge table, daily average sediment concentration table and daily average sediment transport rate table. See the basic information table of runoff stations in wangmaogou basin for the situation of runoff stations.

Data source description

<pre><code> &lt;p&gt;Manual timing observation&lt;/p&gt;

Data processing method

<pre><code> &lt;p&gt;1. The water level observation method of runoff station is: wangmaogou runoff station uses the water gauge on the side wall of the building to measure the water level. In the non flood season, the water level is observed once a day at 8:00 and 20:00 in case of rainfall; the observation is conducted four times at 2:00, 8:00, 14:00 and 20:00 when the daily water level changes greatly or has a slow peak and valley. When the water level changes violently in flood season, the observation shall be made in multiples of 6 minutes. The flow measurement method of wangmaozhuang runoff station: the calibrated natural section observation by manual arrangement is adopted, and the calibrated flow formula is used for flow calculation. Sediment sampling and treatment: the midstream buoy method is used to sample the sediment at the flow measurement section. The frequency of sediment sampling depends on the frequency of flood water level change, and the sampling frequency is increased to control the change of sediment concentration at flood peak. The sample was treated by displacement method.

Data quality description

<pre><code> &lt;p&gt;Manual observation shall be conducted by professional technicians; during data entry, four levels of manual verification shall be conducted, including tabulation, primary calibration, re calibration and audit, so as to ensure data quality. &lt;/p&gt;

Base information

collect time:2011/01/01-2020/12/31

collect place:Wangmaogou watershed, Suide County, Yulin City, Shaanxi Province

altitude:2.0m- Nonem

data size:316.9 KiB

data format:

Coordinate system: WGS84

Keywords

Theme:Runoff, Sediment concentration, Sediment transport rate, Runoff coefficient, Runoff modulus, Soil loss, water level, Erosion modulus, Runoff depth, flow
	Subject:
	Time:2011 2012 2013 2014 2015 2016 2017 2018 2019 2020
	Place:Yulin,  Suide,  Wang Maogou,  Shaanxi Province

Space scope

|  | north： 37.59 |  |
| --- | --- | --- |
| west：110.34 |  | east：110.38 |
|  | south： 37.57 |  |

Service information

contacts:Min Yu Fang

phone:09314967597

mailbox:myf@lzb.ac.cn

Contact information

contributors:Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

author:LEI Xin

manager:GUO Yumei , Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation)

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Suide administration and Supervision Bureau of water and soil conservation of the Yellow River (Suide scientific experimental station of water and soil conservation). Runoff and sediment observation data set of wangmaogou watershed in the first sub region of Loess Hilly and gully region from 2011 to 2020. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2022. https://cstr.cn/CSTR:11738.11.ncdc.HWSDZhan.2020.91.