Soil water and heat data set of nearly naturalized Pinus massoniana plantation (2016-2021)

DOI：10.12072/ncdc.Eco-Hydro.db2413.2022

CSTR：11738.11.ncdc.NIEER.db2413.2022

Share type:Open Access

Category:Ecology

Datasets description

Data source description

This dataset is an automatic observation dataset for soil water and heat, consisting of daily monitoring data on air temperature, relative humidity, soil temperature, soil volumetric moisture content, and soil heat flux of Pinus massoniana plantations that have undergone near natural transformation from October 2016 to December 2021. Using data collected from an automatic meteorological observation station in the artificial forest of Pinus massoniana, which is being transformed into a near natural environment in the South Asian tropics, to conduct dynamic monitoring and research on soil water and heat. The station uses the CR1000 data collector produced by Campbell Company, with a data collection step of 1 minute for each element. Sensor type: Air temperature sensor (1000 Ω PRT, Campbell)、 Air relative humidity sensor (VAISALA HUMICAP H-chip, Campbell), soil temperature sensor (109 type, Campbell), soil moisture sensor (CS616 type, Campbell)、 Soil heat flux sensor (HUKSEFLUX HFP01 type, Campbell)；

Data processing method

This dataset is a reprocessed dataset based on long-term atmospheric and soil monitoring data from the National Positioning Observation and Research Station of the Friendship Pass Forest Ecosystem in Guangxi. The original data processing method refers to the "Ground Meteorological Observation Specification";

Data quality description

Select and set up sample sites through industry experts to ensure that observation points are typical and representative. The Guangxi Friendship Pass Ecological Positioning Research Station is equipped with technical and scientific personnel with long-term field work experience, who regularly maintain instruments and equipment and collect data to ensure the quality and integrity of observation data. The quality control methods for the raw data involved in this dataset are carried out in accordance with the relevant provisions of the Long term Observation Specification of the China Ecosystem Research Network (CERN);

Base information

collect time:2016/10/01-2021/12/31

collect place:Guangxi Friendship Pass Forest Ecosystem National Positioning Observation and Research Station

data size:252.4 KiB

data format:.xlsx

Coordinate system: 无

Keywords

Theme:Soil water and heat dynamics, soil temperature, soil volume water content, Masson Pine Plantation
	Subject:
	Time:2016 2017 2018 2019 2020 2021
	Place:None

Space scope

|  | north： 22.04 |  |
| --- | --- | --- |
| west：106.88 |  | east：106.86 |
|  | south： 22.04 |  |

Service information

contacts:Li Hongxing

phone:0931—4967592

mailbox:lihongxing@lzb.ac.cn

Contact information

contributors:,  ZHENG Lu,  YANG Baoguo,  YANG Kun,  MIN Huilin,  CHEN Lin,  ZHANG Pei,  PANG Shengjiang,  SHU Weiwei,  LI Hua

author:

manager:

Project information

number:桂科202227，
      name:

number:桂科AD20159094，
      name:

number:桂科22-035-130-03，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

, ZHENG Lu, YANG Baoguo, YANG Kun, MIN Huilin, CHEN Lin, ZHANG Pei, PANG Shengjiang, SHU Weiwei, LI Hua. Soil water and heat data set of nearly naturalized Pinus massoniana plantation (2016-2021). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2022. https://cstr.cn/CSTR:11738.11.ncdc.NIEER.db2413.2022.

Article citation:

刘士玲, 郑路, 杨保国, 杨坤, 闵惠琳, 陈琳, 张培, 庞圣江, 舒韦维, 李华. 2016–2021年南亚热带近自然化改造马尾松人工林土壤水热动态数据集[J]. 中国科学数据（中英文网络版）, 2023, 8, (3).