Heihe River eco hydrological remote sensing experiment: specific emissivity data set in the middle reaches of Heihe River Basin (2012)

DOI：10.12072/ncdc.nieer.db3007.2023

CSTR：11738.11.ncdc.NIEER.2021.1794

Share type:Login to Access

Category:Remote Sensing

Datasets description

This data is a typical surface specific emissivity data set observed in Yingke oasis, wetland, Gobi and desert in the middle reaches of Heihe River. The data were observed from May 25, 2012 to July 18, 2012. Instruments: Portable Fourier transform infrared spectrometer (102f) and handheld infrared thermometer of Beijing Normal University Measurement method: use 102f to measure the radiation values of cold blackbody, warm blackbody, observation target and gold plate respectively. Using the radiation value of cold and warm blackbody, 102f is calibrated to eliminate the influence of instrument emission. The specific emissivity and object temperature are inversed by using the iterative inversion algorithm based on smoothness. The band range of specific emissivity is 8-14 μ m. Resolution 4cm-1. This data set contains the original radiation curves (ASCII format) and recording files of cold blackbody, warm blackbody, measured target and gold plate obtained by 102f. The processed data is 8-14 μ M specific emissivity curve (ASCII format). Some data are matched with thermal imager photos of the measured target, scenes taken by digital cameras and images of the measured object

Data source description

The data were observed from May 25, 2012 to July 18, 2012. Portable Fourier transform infrared spectrometer (102f) and handheld infrared thermometer of Beijing Normal University

Data processing method

Measurement method: use 102f to measure the radiation values of cold blackbody, warm blackbody, observation target and gold plate respectively. Using the radiation value of cold and warm blackbody, 102f is calibrated to eliminate the influence of instrument emission. The specific emissivity and object temperature are inversed by using the iterative inversion algorithm based on smoothness. The band range of specific emissivity is 8-14 μ m. Resolution 4cm-1

Data quality description

Good data quality

Base information

collect time:2012/05/25-2012/07/18

collect place:Heihe River Basin, experimental area of artificial oasis in the middle reaches, desert, Gobi, wetland, farmland

data size:382.0 MiB

data format:word,cbx,dwx,emx,sax,wbx

Coordinate system: WGS84

Keywords

Theme:Specific emissivity, thermal imager, handheld infrared thermometer, infrared spectrometer 102f, ground-based remote sensing observation, atmospheric radiation, atmospheric temperature, surface temperature, airborne, ground remote sensing
	Subject:physical geography 
	Time:2012
	Place:Middle reaches artificial oasis experimental area,  Heihe River Basin,  wetland,  desert,  Gobi,  farmland

Space scope

|  | north： 38.86 |  |
| --- | --- | --- |
| west：100.37 |  | east：100.37 |
|  | south： 38.86 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Ma mingguo

author:Ma mingguo

manager:Ma mingguo

Project information

number:91125004，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Ma mingguo. Heihe River eco hydrological remote sensing experiment: specific emissivity data set in the middle reaches of Heihe River Basin (2012). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.NIEER.2021.1794.

Article citation:

Xin Li, Shaomin Liu, Qin Xiao, Mingguo Ma, Rui Jin, Tao Che, Weizhen Wang, Xiaoli Hu, Ziwei Xu, Jianguang Wen, Liangxu Wang. A multiscale dataset for understanding complex eco-hydrological processes in a heterogeneous oasis system.[J]. Scientific data, 2017, 4.