Heihe River eco hydrological remote sensing experiment: channel flow measurement data set in the middle reaches of Heihe River Basin (2012)

DOI：10.12072/ncdc.NIEER.db0089.2020

CSTR：11738.11.ncdc.nieer.2020.1600

Share type:Login to Access

Category:Hydrology

Datasets description

This data set contains the channel flow measurement data on May 22, 2012 (the second round of summer irrigation), June 18, 2012 (the third round of summer irrigation), July 16, 2012 (the first round of autumn irrigation) and August 8, 2012 (the second round of autumn irrigation). The original data of channel flow observation is in Excel format, including channel flow velocity, channel section area, water level and water temperature. Through the data preprocessing process, the original data are converted into channel flow and irrigation volume entering each community. See "channel flow measurement data processing document" for data processing process

Data source description

According to the flow observation of the bucket canal and the coarse canal, the actual water volume entering the minimum irrigation unit (community) is obtained to provide reference data for the optimal irrigation water distribution test; Provide water volume data for vortex, biophysical parameters and other observation tests; It provides a reference for the investigation data of each canal in Yingke irrigation area

Data processing method

Observation instrument: Portable flow watch, origin: Switzerland, velocity observation accuracy: 0.1m/s; Water gauge, observation accuracy: 1cm.    Observation principle: the instrument is a mechanical current meter, which calculates the fluid velocity according to the speed of water turbine. The flow velocity of 0.6 times the central water level of the channel is observed by the current meter, and the channel flow is calculated through the fluid section area calculated by the channel engineering section parameters and water level at the observation point.

Data quality description

Good data quality

Base information

collect time:2012/05/22-2012/08/08

collect place:Middle reaches of the Heihe River

data size:39.6 KiB

data format:Excel

Coordinate system: 无

Projection
       /

Keywords

Theme:Irrigation volume, rivers and canals, ground synchronous observation
	Subject:physical geography 
	Time:2012
	Place:Heihe River Basin,  middle and middle reaches of artificial oasis experimental area

Space scope

|  | north： 38.89 |  |
| --- | --- | --- |
| west：100.35 |  | east：100.40 |
|  | south： 38.85 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Ge Yingchun

author:Ge Yingchun

manager:Ge Yingchun

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Ge Yingchun. Heihe River eco hydrological remote sensing experiment: channel flow measurement data set in the middle reaches of Heihe River Basin (2012). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.nieer.2020.1600.