Heihe integrated remote sensing joint test: surface roughness data set of biandukou intensive observation area during the pre-test period (2007)

DOI：10.12072/ncdc.NIEER.db1798.2022

CSTR：11738.11.ncdc.NIEER.2021.1728

Share type:Login to Access

Category:Remote Sensing

Datasets description

This dataset contains: (1) Sample plot 1; （2) Sample plot 2; （3) GPS point coordinates and plot description of biandukou quadrat; （4) Rough - pre test flat mouth 1; （5) Rough - pre test flat mouth 2; This data can provide basic data for the development and verification of microwave remote sensing algorithms. The data set mainly includes the original photos of roughness measurement at each sampling point of biandukou quadrat 1 and biandukou quadrat 2 during the pre-test in the upstream cold area, and the calculation results of surface height standard deviation (CM) and correlation length (CM). The roughness data is collected by digital photography, and then the top of each spoke in the photo and the four corners of the board are manually digitally sampled under ArcView software to obtain the image coordinate value. After geometric correction, the height of each spoke is calculated, and then the surface height standard deviation and surface correlation length are calculated according to the formula.

Data source description

Each quadrat of flat mouth is 3grid × 3grid, 30m for each grid × 30m。 The calculation formula is shown in pages 234-236, Volume II, microwave remote sensing. In the roughness data file, the first is the sample name, and then the data body includes 4 columns (number, file name, standard deviation and correlation length). Each file name, i.e. TXT file, corresponds to a sampling photo, and the standard deviation (CM) and correlation length (CM) represent the roughness. This is followed by the length of 101 spokes in each photo, which belongs to the intermediate result to check and correct.

Data processing method

The roughness data is collected by digital photography, and then the top of each spoke in the photo and the four corners of the board are manually digitally sampled under ArcView software to obtain the image coordinate value. After geometric correction, the height of each spoke is calculated, and then the surface height standard deviation and surface correlation length are calculated according to the formula. The calculation formula is shown in pages 234-236, Volume II, microwave remote sensing   In the roughness data file, the first is the sample name, and then the data body includes 4 columns (number, file name, standard deviation and correlation length). Each file name, i.e. TXT file, corresponds to a sampling photo, and the standard deviation (CM) and correlation length (CM) represent the roughness. This is followed by the length of 101 spokes in each photo, which belongs to the intermediate result to check and correct.

Data quality description

Good data quality

Base information

collect time:2007/10/17-2007/10/18

collect place:Heihe River Basin, upstream cold region, hydrological test area, biandukou intensive observation area

data size:506.3 MiB

data format:jpg

Coordinate system: WGS84

Keywords

Theme:Surface roughness, geodesy, digital sampling
	Subject:physical geography 
	Time:2007
	Place:Hydrological test area in upper cold region,  Heihe River Basin,  Flat mouth encrypted observation area

Space scope

|  | north： 38.97 |  |
| --- | --- | --- |
| west：100.59 |  | east：100.44 |
|  | south： 38.04 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:JIN Rui

author:JIN Rui

manager:JIN Rui

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

JIN Rui. Heihe integrated remote sensing joint test: surface roughness data set of biandukou intensive observation area during the pre-test period (2007). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2022. https://cstr.cn/CSTR:11738.11.ncdc.NIEER.2021.1728.