Integrated remote sensing joint experiment of Heihe River: spectral measurement data set of typical features in biandukou intensive observation area

DOI：10.12072/ncdc.NIEER.db1883.2022

CSTR：11738.11.ncdc.NIEER.2021.1741

Share type:Login to Access

Category:Remote Sensing

Datasets description

From March 7, 2008 to March 21, 2008, the ASD spectral data of typical features were measured in the Bian Du Kou encrypted observation area. Through the grid spectrum measurement results of ground features, the spatial variation characteristics and laws of ground feature subpixel spectrum are explored, and the spectral characteristics of ground features in cold and arid areas are analyzed. On this basis, it provides data for the spectral model database of typical ground object target and background model, and provides basic data for remote sensing image simulation and spatial scale conversion of remote sensing data. ASD FieldSpec frtm (Boulder, Co, USA) was used for measurement. The spectral range is 350nm-2500nm. The spectral resolution is 3nm in visible near-infrared band and 10nm in short wave infrared band. The survey time, features and contents are as follows: On March 7, 2008, the instrument tested the spectra of dry soil, wet soil, straw, snow and grassland in biandukou area, and arranged quadrats for observation. On March 21, 2008, the surface was covered with about 10cm snow. The spectral characteristics of the snow at biandukou were measured, and no quadrat was arranged. On March 17, 2008, the observation data were synchronized with TM. Sampling method: select the center of the area in a flat and homogeneous area to lay the quadrat. Select 90m respectively × 90m、450m × 450m is used as the grid sampling area of three different scales. At 90m × 90m、450m × 9 in 450m quadrat area × 9 grid division, so that the three quadrats are divided into 81 10m × 10m、50m × 50m sub grid. The specific meaning of the three grids is: 30m × 30m、150m × 150m is the basic resolution of satellite sensor. On this basis, the contribution of single pixel and eight neighborhood pixels to the central pixel is considered. Each pixel is divided into 9 sub pixels to explore the law between the sub-pixel spectrum and the whole pixel spectrum. Due to the long distance between sample points, 90m × The 90m quadrat is realized by laying the quadrat with measuring rope and marker post, with a height of 450m × The 450m quadrat is realized by laying control points with GPS. The sampling method is profile measurement, which represents the spectrum of the whole quadrat pixel by measuring the central profile of each sub quadrat, and replaces the full sampling of the sub quadrat by measuring the statistical average of the spectrum for many times. The survey route adopts patrol mode to ensure the quasi synchronization of spectrum measurement. The measurement data is in ASCII format and can be opened with Notepad, WordPad and other software. The first 5 lines of the file are the file header, which describes the relevant information of the data; The next two columns of data, one for wavelength and one for reflectance (percent reflectance). The file in the original data is in ASD's own format and opened with ASD viewspec software.

Data source description

ASD FieldSpec frtm (Boulder, Co, USA) was used for measurement. The spectral range is 350nm-2500nm. The spectral resolution is 3nm in visible near-infrared band and 10nm in short wave infrared band.   The survey time, features and contents are as follows:   On March 7, 2008, the instrument tested the spectra of dry soil, wet soil, straw, snow and grassland in biandukou area, and arranged quadrats for observation. On March 21, 2008, the surface was covered with about 10cm snow. The spectral characteristics of the snow at biandukou were measured, and no quadrat was arranged. On March 17, 2008, the observation data were synchronized with TM.

Data processing method

Spectral data measurement:  （ 1) Before the survey, input the coordinates of the survey points into GPS or use the survey rope to determine the survey points.  （ 2) Arrive at the test site half an hour before the start of synchronous measurement, check the status of instruments and equipment, and assign tasks to measurement participants.  （ 3) Start the machine 15 minutes before measurement for preheating, and start measurement after stabilizing the dark current value of the instrument.  （ 4) Record weather conditions, site conditions and surveyors. Set the file storage path, set the average times of spectrum, dark current and reference board sampling times. Remove the dark current and optimize the integration time.  （ 5) During the walking survey of ground objects, pay attention that the body and its own shadow are not within the field of view of the spectrometer. Measure the reference plate every 1 minute or so (adjusted according to the weather conditions)  （ 6) Record the measurement results, take photos of the ground features, export the measurement data, and turn off the instrument and laptop.   Sampling method: select the center of the area in a flat and homogeneous area to lay the quadrat. Select 90m respectively × 90m、450m × 450m is used as the grid sampling area of three different scales. At 90m × 90m、450m × 9 in 450m quadrat area × 9 grid division, so that the three quadrats are divided into 81 10m × 10m、50m × 50m sub grid. The specific meaning of the three grids is: 30m × 30m、150m × 150m is the basic resolution of satellite sensor. On this basis, the contribution of single pixel and eight neighborhood pixels to the central pixel is considered. Each pixel is divided into 9 sub pixels to explore the law between the sub-pixel spectrum and the whole pixel spectrum. Due to the long distance between sample points, 90m × The 90m quadrat is realized by laying the quadrat with measuring rope and marker post, with a height of 450m × The 450m quadrat is realized by laying control points with GPS. The sampling method is profile measurement, which represents the spectrum of the whole quadrat pixel by measuring the central profile of each sub quadrat, and replaces the full sampling of the sub quadrat by measuring the statistical average of the spectrum for many times. The survey route adopts patrol mode to ensure the quasi synchronization of spectrum measurement.

Data quality description

Good data quality

Base information

collect time:2008/03/07-2008/03/21

collect place:Heihe River Basin

data size:148.9 MiB

data format:ascll,txt

Coordinate system: WGS84

Keywords

Theme:Ground object spectrum, ASD spectrometer, reflectance
	Subject:physical geography 
	Time:2008
	Place:Heihe River Basin,  Biandukou intensive observation area,  Hydrological test area in upper cold region

Space scope

|  | north： 38.97 |  |
| --- | --- | --- |
| west：100.59 |  | east：100.44 |
|  | south： 38.04 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Zhao Shaojie,  Ren Huazhong

author:Zhao Shaojie

manager:Zhao Shaojie

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Zhao Shaojie, Ren Huazhong. Integrated remote sensing joint experiment of Heihe River: spectral measurement data set of typical features in biandukou intensive observation area. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2022. https://cstr.cn/CSTR:11738.11.ncdc.NIEER.2021.1741.