Runoff and sediment observation data set of Nanxiaohegou watershed in the gully area of the Loess Plateau from 1956 to 1980

DOI：10.12072/ncdc.HWXFZhan.db0012.2020

CSTR：11738.11.ncdc.HWXFZhan.2020.67

Share type:Login to Access

Category:Soil and Water Conservation

Datasets description

This data set is the runoff and sediment observation data of Huaguoshan reservoir outlet, 18mu platform, Yangjiagou and dongzhuanggou of Nanxiaohegou watershed in the gully area of the Loess Plateau from 1956 to 1980. The experimental observation section of the runoff station is the basher water measuring weir and the triangular channel water measuring weir, and the observation method is manual observation. It mainly includes daily discharge data, daily average sediment concentration data, daily average sediment transport rate data, excerpt data of flood hydrological elements, and successive flood test results data of Nanxiaohegou watershed.

Data source description

Manual observation.

Data processing method

Water level observation: in flood season and non flood season, the basic flow shall be observed manually and regularly. In non flood season, the basic flow shall be observed once at 8:00 and 20:00 respectively. In flood season, the basic flow shall be observed once at 2:00, 8:00, 14:00 and 20:00 respectively. In flood season, the basic flow shall be observed once from June to September. During the flood period or the period of rapid water level change, on the premise of reading and controlling the water level at the important turning points of rise, peak, fall and other water level rise and fall, the survey times shall be arranged evenly according to the water level change, at least 3 times near the peak, generally at least 11 times for a single peak, and once 30 minutes to 1.0 hours after the fall, which requires that each peak, valley and complete water level change process can be measured. Sediment Observation: one point method by water edge is adopted manually. In flood period, the number of sand taking should be based on the principle of controlling the change process of sediment concentration. When the water and sediment peaks are inconsistent or the sediment concentration changes violently, the number of measurements should be increased appropriately. All water samples are treated by displacement method (1) Flow calculation: calculate the flow with the observed water level, the calculation formula is: basher weir: q = 3.7h1.59, Yangjiagou triangular groove measuring weir: q = 0.49-2.68h + 7.09h? 2, dongzhuanggou triangular groove measuring weir: q = 1.078-4.54h + 7.96h? 2 Q: Flow, unit: m3 / S; H: water level, unit: M. (2) Calculation of sediment concentration: P = WS / V * 1000 calculation of sediment concentration, P: measured sediment concentration, unit: kg / m3; V: water sample volume, unit: cm3; WS: dry sand weight in water sample, unit: G.

Data quality description

In the process of integration and input of observation data, four levels of manual verification are carried out, including tabulation, primary calibration, re calibration and audit, so as to ensure the data quality.

Base information

collect time:1956/01/01-1980/12/31

collect place:Nanxiaohegou watershed, houguanzhai Township, Xifeng District, Qingyang City, Gansu Province

altitude:1050.0m- 1423.0m

data size:1.6 MiB

data format:

Coordinate system: 无

Keywords

Theme:Average flow rate, Erosion modulus, Average sediment concentration, Runoff modulus, Average sediment transport rate, Runoff depth
	Subject:Hydrometric survey 
	Time:1956 1957 1958 1959 1960 1961 1962 1963 1964 1965 1966 1967 1968 1969 1970 1971 1972 1973 1974 1975 1976 1977 1978 1979 1980
	Place:Qingyang City,  Xifeng,  Nanxiaohegou watershed,  Gansu Province

Space scope

|  | north： 35.75 |  |
| --- | --- | --- |
| west：107.62 |  | east：107.51 |
|  | south： 35.68 |  |

Service information

contacts:Min Yu Fang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Yellow River Water Conservancy Commission Xifeng Soil and Water Conservation Scientific Experimental Station

author:YUAN Jing

manager:YUAN Jing

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Yellow River Water Conservancy Commission Xifeng Soil and Water Conservation Scientific Experimental Station. Runoff and sediment observation data set of Nanxiaohegou watershed in the gully area of the Loess Plateau from 1956 to 1980. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.HWXFZhan.2020.67.