Heihe River eco hydrological remote sensing experiment: observation data set of freeze-thaw experiment in the upper reaches of Heihe River Basin (November 10-14, 2013)

DOI：10.12072/ncdc.nieer.db3492.2023

CSTR：11738.11.ncdc.NIEER.2021.35

Share type:Apply to Access

Category:Remote Sensing

Datasets description

This data set includes the continuous observation data set of bright temperature observed by vehicle mounted microwave radiometer and surface temperature and humidity measured synchronously in aro grassland, aro Township, Qilian County, Qinghai Province from November 10 to 14, 2013. The surface temperature and humidity include the observed soil temperature and soil moisture data at the soil depth of 1cm, 3cm, 5cm, 10cm, 15cm and 20cm by the temperature sensor and 0-5cm by the humidity sensor. The time frequency of routine observation of soil temperature and humidity is 5 minutes. Heihe River eco hydrological remote sensing experiment: observation data set of freeze-thaw experiment in the upper reaches of Heihe River Basin (November 10-14, 2013) Data details: 1. Time: November 10-14, 2013 2. Data: Heihe River eco hydrological remote sensing test: observation data set of freeze-thaw experiment in the upper reaches of Heihe River Basin (November 10-14, 2013) 3. Brightness temperature: observed by vehicle mounted multi frequency passive microwave radiometer, including 6.925, 10.65, 18.7 and 36.5ghz V polarization and H polarization data Soil temperature: the sensor installed on dt80 and dt85 is used to measure, in which the sensor connected to dt80 measures the soil temperature of 1cm, 5cm, 10cm and 20cm, and the probe connected to dt85 measures the soil temperature of 1cm, 3cm, 5cm, 10cm and 15cm Soil humidity: use h-probe sensor to measure 0-5cm soil humidity. The probe can measure 0-5cm soil temperature at the same time 1. Data size: 16.7M 2. Data format:. Xls

Data source description

This data set includes the continuous observation data set of bright temperature observed by vehicle mounted microwave radiometer and surface temperature and humidity measured synchronously in aro grassland, aro Township, Qilian County, Qinghai Province from November 10 to 14, 2013. The surface temperature and humidity include the observed soil temperature and soil moisture data at the soil depth of 1cm, 3cm, 5cm, 10cm, 15cm and 20cm by the temperature sensor and 0-5cm by the humidity sensor. The time frequency of routine observation of soil temperature and humidity is 5 minutes

Data processing method

Measure with sensors mounted on dt80 and dt85

Data quality description

Good data quality

Base information

collect time:2013/11/10-2013/11/14

collect place:Heihe River Basin, grassland, upstream cold region, hydrological test area, aro

data size:17.0 MiB

data format:

Coordinate system: WGS84

Keywords

Theme:Soil, soil temperature, airborne ground remote sensing, microwave radiometer, soil moisture, water content
	Subject:physical geography 
	Time:2013
	Place:Heihe River Basin,  grassland,  upstream cold region,  hydrological test area

Space scope

|  | north： 38.45 |  |
| --- | --- | --- |
| west：100.97 |  | east：100.97 |
|  | south： 38.45 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Zhao Shaojie,  Ma mingguo

author:Zhao Shaojie

manager:Zhao Shaojie

Project information

number:91125004，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Zhao Shaojie, Ma mingguo. Heihe River eco hydrological remote sensing experiment: observation data set of freeze-thaw experiment in the upper reaches of Heihe River Basin (November 10-14, 2013). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.NIEER.2021.35.

Article citation:

Tao Che, Xin Li, Shaomin Liu, Hongyi Li, Ziwei Xu, Junlei Tan, Yang Zhang, Zhiguo Ren, Lin Xiao, Jie Deng, Rui Jin, Mingguo Ma, Jian Wang, Xiaofan Yang. Integrated hydrometeorological, snow and frozen-ground observations in the alpine region of the Heihe River Basin, China[J]. Earth System Science Data, 2019, 11, (3).