Tianjin Jinhai Lake Lake Pretreatment Effluent Water Quality Dataset (July 20-23, 2020)

DOI：10.12072/ncdc.MDBW.db0021.2021

CSTR：11738.11.ncdc.MDBW.2021.13

Share type:Login to Access

Category:Others

Datasets description

2020 collected water samples of Tianjin Jinhai Lake water after pre-treatment of the effluent water, the water quality report obtained by the third-party testing organization National Center for Supervision and Inspection of Seawater and Brackish Water Product Quality Utilization Testing. The precision filtration system adopts bag filter element with filtration precision of 5μm and meltblown filter element in series, and the precision filter operating pressure is 0.3MPa. The ultrafiltration system adopts polyvinylidene fluoride tubular membrane (average pore size of 0.1μm, inner diameter of the membrane tube is 8mm, Tianjin Hepel), and the ultrafiltration operating pressure is 0.13MPa, with the recirculation flow rate of 43~45m3/h, and the flow rate of the produced water is controlled at 2m3/h. The test includes calcium ion, magnesium ion, potassium, sodium, chloride, sulfate, manganese, iron, total organic carbon, chemical oxygen demand, total hardness, total alkalinity, total dissolved solids, particle size distribution. According to the “Standard Test Methods for Drinking Water” GB/T 5750-2006, “Determination of Potassium and Sodium in Water Quality Flame Atomic Absorption Spectrophotometry” GB/T 11904-1989, “Determination of Calcium and Magnesium Ions in Industrial Circulating Cooling Water EDTA Titration Method” GB/T 15452-2009 and other standardized test methods and Procedure.

Data source description

Experimentally obtained.

Data processing method

Water quality testing by a third-party testing organization.

Data quality description

(1)Data production process:Collecting samples of produced water from the lake water of Jinhai Lake after pretreatment by the precision filtration system or precision filtration-ultrafiltration system, and obtaining them by the third-party testing organization. The precision filtration system adopts bag filter element with filtration precision of 5 μm and meltblown filter element in series, and the operating pressure of the precision filter is 0.3 MPa. The ultrafiltration system adopts polyvinylidene fluoride tubular membrane (average pore size of 0.1 μm, inner diameter of the membrane tube is 8 mm, Tianjin HaiPuLu), and the operating pressure of the ultrafiltration is 0.13 MPa, and the recirculation flow rate is 43~45 m3/h, and the flow rate of the produced water is controlled at 2 m3/h.   (2)Methods and standards: According to “Standard Test Methods for Drinking Water” GB/T 5750-2006, “Determination of Potassium and Sodium in Water Quality Flame Atomic Absorption Spectrophotometry” GB/T 11904-1989, “Determination of Calcium and Magnesium Ions in Industrial Circulating Cooling Water EDTA Titration Method” GB/T 15452-2009. The test methods and processes are standardized in the standards.

Base information

collect time:2020/07/20-2020/07/23

collect place:Jinhai Lake, Binhai New Area, Tianjin

data size:1.7 MiB

data format:pdf

Coordinate system: 无

Keywords

Theme:Brackish water, precision filtration, ultrafiltration, water quality, total dissolved solids, hardness, total organic carbon, particle size
	Subject:Hydrochemistry 
	Time:2020
	Place:Jinhai Lake

Space scope

|  | north： |  |
| --- | --- | --- |
| west： |  | east： |
|  | south： |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:DU Runhong

author:DU Runhong

manager:DU Runhong

Project information

number:2017YFE0114200，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

DU Runhong. Tianjin Jinhai Lake Lake Pretreatment Effluent Water Quality Dataset (July 20-23, 2020). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2021. https://cstr.cn/CSTR:11738.11.ncdc.MDBW.2021.13.