Glacier-level and gridded mass change in the rivers' sources in the eastern Tibetan Plateau (1970s-2000)

DOI：10.12072/ncdc.nieer.db6680.2024

CSTR：11738.11.NCDC.NIEER.DB6680.2024

Share type:Open Access

Category:Hydrology

Datasets description

This dataset focuses on glacier mass change in the Eastern Tibetan Plateau's (ETPR) source rivers from the 1970s to 2000. It provides data at two resolutions: high-resolution glacier-level data (30 m) and lower-resolution gridded data (0.1 and 0.5 degrees). The high-resolution data offers valuable insights for individual glaciers, including elevation change over 30 years, annual mass balance, and associated uncertainty for 13,117 glaciers. To calculate mass change, we compared digital elevation models (DEMs) derived from 1970s topographic maps (aerial photogrammetry) with the Shuttle Radar Topography Mission (SRTM) data from 2000. Co-registration, bias correction, and quality control procedures ensured data accuracy. We evaluated the DEM-based elevation differences using ICESat-2 and KH-9 data. The results demonstrate good quality, particularly at lower altitudes, similar to KH-9 data. However, high-altitude results have higher uncertainty due to limitations in capturing highly reflective snow surfaces on steep terrain using aerial photos. This dataset is a valuable resource as it covers 72% of ETPR glaciers. This comprehensiveness makes it ideal for calibrating parameters in mass balance simulations at various scales. Furthermore, the dataset can be used to assess changes in glacier hydrological effects before and after 2000.

Data source description

We employed a total of 718 historical topographic maps including 142 at a scale of 1:50,000 and 576 at a scale of 1:100,000 compiled from aerial photos taken from 1957 to 1983 by the Chinese Military Geodetic Service. In this study, we use SRTM DEM (no void filled version) with a resolution of 1 arc second (~30 m) refer to the glacier surface in the year of 1999 suggested by previous studies (e.g., Gardelle et al., 2013; Mcnabb et al., 2019). In this study, we acquired the KH-9 images covering Xixiabangma Mountain，and ICESat-2 data.

Data processing method

To generate the grid product, we took the following steps: 1) Firstly, we spatially merged the elevation differences, elevation errors, and timestamps of all effective glaciers. 2) For each grid cell, we calculate the average glacier elevation difference, elevation error, and timestamp within that grid cell. These averages are designated as grid cell values. 3) For each grid cell, we calculate the NMAD and pixel count of the height difference. Calculate the grid scale resampling error using formula (6). 4) Combining the grid scale height difference and timestamp, apply formula (7) to calculate the grid scale mass balance. 5) Combining the grid scale elevation variation error and timestamp, apply formula (8) to calculate the initial grid scale mass balance error. Finally, the final mass balance error is obtained by combining the resampling error.

Data quality description

The data quality is good.

Base information

collect time:1970/01/01-2000/12/31

collect place:Eastern Qinghai Tibet Plateau

data size:529.9 MiB

data format:NetCDF

Coordinate system: 无

Keywords

Theme:Glacier (ice sheet), glacier elevation changes, mass balance, remote sensing of surface freeze-thaw cycles/states, cryosphere remote sensing products
	Subject:geography 
	Time:1970 1971 1972 1973 1974 1975 1976 1977 1978 1979 1980 1981 1982 1983 1984 1985 1986 1987 1988 1989 1990 1991 1992 1993 1994 1995 1996 1997 1998 1999 2000
	Place:Rivers at the source of the eastern Qinghai Tibet Plateau

Space scope

|  | north： 40.00 |  |
| --- | --- | --- |
| west： 75.00 |  | east：105.00 |
|  | south： 27.00 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:LIU Shiyin,  Zhu yu

author:Zhu yu ,  LIU Shiyin

manager:Zhu yu , LIU Shiyin

Project information

number:41991234，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

LIU Shiyin, Zhu yu. Glacier-level and gridded mass change in the rivers' sources in the eastern Tibetan Plateau (1970s-2000) . National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2024. https://cstr.cn/CSTR:11738.11.NCDC.NIEER.DB6680.2024.

Article citation:

Zhu Y, Liu S, Wei J, Wu K, Bolch T, Xu J, Guo W, Jiang Z, Xie F, Yi Y, Shangguan D, Yao X, Zhang Z. Glacier-level and gridded mass change in the rivers’ sources in the eastern Tibetan Plateau (ETPR) from 1970s to 2000[J]. Earth System Science Data Discussions, 2024, 2024: 1-28. 
            DOI: 10.5194/essd-2024-255.