2021-2024Runoff and Sediment Observation Dataset of Qiaozigou Basin Gully Flow Station in the Third Sub district of Loess Hilly and Gully Region

DOI：10.12072/ncdc.hwtszhan.db2707.2023

CSTR：11738.11.ncdc.hwtszhan.db2707.2023

Share type:Apply to Access

Category:Soil and Water Conservation

Datasets description

This dataset is the runoff and sediment observation data of the Qiaozigou watershed in the third sub area of the Loess Hilly and Gully Region from 2021 to 2024. The Qiaozigou watershed is a small watershed for parallel observation and testing of governance and non governance. The Qiaozigong watershed is a governance watershed, and the Qiaozigxi watershed is a non governance watershed. The flow observation adopts the buoy method to measure the flow velocity, and the flow velocity area method to calculate the flow rate; Sediment sampling and flow measurement are carried out simultaneously, with the same number of sampling and flow measurement times. Sand samples are taken using the water edge one point method, and sediment concentration is measured using the displacement method. This dataset includes a table of measured flow results, a daily average flow rate table, a daily average sediment concentration table, and a daily average suspended sediment transport rate table.

Data source description

<p>Manual fixed point and fixed time observation

Data processing method

<p>Flow observation: the flow measuring structures of Qiaozidonggou and Qiaozidigou runoff stations are triangular broad-crested weirs and trapezoidal broad-crested weirs respectively. When there is little water, the flow is measured by the volume method with the receiver, and the flow is calculated by the buoy method with the buoy coefficient of 0.85. Generally, it is observed twice at 8:00 and 20:00 every day. The number of measurements in flood season depends on the increase of water conditions. It is required to be able to measure each flood peak, valley and complete flood process. Sediment concentration observation: take sand samples by the one-point method at the water edge, and the sampling times are the same as the flow measurement times. The replacement method is adopted for sand sample treatment.

Data quality description

<p>According to the provisions of "Specification for Water and Soil Conservation Test" (SD239) and "Specification for Water and Soil Conservation Test" (SL419), professional technical personnel shall observe, sort out and compile the data. Observation data shall be measured, calculated, sorted and analyzed according to the requirements of "four follow-up", and errors and unreasonable records of observation shall be timely handled and corrected, and notes shall be made. During the data compilation and entry process, four levels of manual verification, such as tabulation, initial calibration, recheck and review, are carried out. The data compilation is carried out in strict accordance with the relevant observation and compilation specifications and procedures, and in accordance with the process of first calibration, second calibration, review, recheck and review to ensure the quality of data results.

Base information

collect time:2021/01/01-2024/12/31

collect place:Qiaozigou watershed, Qinzhou District, Tianshui City, Gansu Province

altitude:1235.0m- 1707.2m

data size:202.9 KiB

data format:

Coordinate system: 无

Keywords

Theme:Annual sediment transport, annual runoff, total flood, flood sediment transport, soil loss per unit area, runoff coefficient, daily average flow, daily average sediment concentration, daily average suspended sediment transport rate, maximum flow
	Subject:hydrology 
	Time:2021 2022 2023 2024
	Place:Luoyugou,  Tianshui,  Gansu Province

Space scope

|  | north： 34.63 |  |
| --- | --- | --- |
| west：105.71 |  | east：105.73 |
|  | south： 34.60 |  |

Service information

contacts:

phone:0931-4967592

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:ZHANG Xiaoyu

author:Liu Xiao ,  FAN Yijia

manager:ZHANG Xiaoyu

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

ZHANG Xiaoyu. 2021-2024Runoff and Sediment Observation Dataset of Qiaozigou Basin Gully Flow Station in the Third Sub district of Loess Hilly and Gully Region. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2023. https://cstr.cn/CSTR:11738.11.ncdc.hwtszhan.db2707.2023.