Yellow River Bayangaole-to-Toudaoguai Station Ice Thickness Dataset (2019-2020)

DOI：10.12072/ncdc.iceflood.db2364.2022

CSTR：11738.11.ncdc.iceflood.db2364.2022

Share type:Login to Access

Category:Hydrology

Datasets description

This dataset contains ice thickness data for the Yellow River from Bayangaole to Toudaoguai Station for the years 2019-2020, obtained through simulation using an ice thickness model. Bayangaole, Sanhuhekou, Baotou, and Toudaoguai are the existing hydrological stations along the Inner Mongolia section of the Yellow River. This dataset consists of 1 Excel file.

Data source description

The data in this dataset was obtained through simulation using a thermodynamic analysis method. The data was collected at the location of the numerical simulation, which is Hefei, and covers the period from 2020 to 2021. The data resources were obtained through simulation using an ice thickness model, which took into account multiple parameters in hydraulic and thermal conditions during the simulation process. The output results of the model are controlled within a reasonable accuracy range.

Data processing method

An ice thickness calculation model based on the degree-day method using Stefan's formula calculates the heat flux under the ice cover by introducing the Colburn analogy method. Factors such as river discharge, flow velocity, water level, riverbed slope, and ice cover roughness are incorporated into the thermodynamic equation. Prototype observation data from four stations along the Bayangaole to Toudaoguai section of the river were used to calibrate the relevant parameters of this ice thickness calculation model. After calibration, the model can calculate ice cover thickness data using air and water temperature data. The model originates from the paper "Simulation Study on Ice Cover Thickness in the Inner Mongolia Section of the Yellow River".

Data quality description

The calculators are researchers in river ice at Hefei University of Technology. The relevant calculation process has been written up in papers and published, ensuring reliable data with good quality.

Base information

collect time:2019/01/01-2020/12/31

collect place:Bayangaole, Toudaoguai

data size:37.6 KiB

data format:Excel

Coordinate system: 无

Keywords

Theme:Yellow River, ice thickness
	Subject:hydrology 
	Time:2019 2020
	Place:Bayan Gaole,  Toudaoguai

Space scope

|  | north： 40.31 |  |
| --- | --- | --- |
| west：107.03 |  | east：111.06 |
|  | south： 40.27 |  |

Service information

contacts:Li Hongxing

phone:0931-4967592

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:WANG Jun,  hou zhixing

author:WANG Jun

manager:WANG Jun

Project information

number:2018YFC1508401，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

WANG Jun, hou zhixing. Yellow River Bayangaole-to-Toudaoguai Station Ice Thickness Dataset (2019-2020). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2022. https://cstr.cn/CSTR:11738.11.ncdc.iceflood.db2364.2022.