Multi-scale surface flux and meteorological elements observation dataset in the Hai River Basin (Guantao site-automatic weather

DOI：10.12072/ncdc.westdc.db3831.2023

CSTR：11738.11.ncdc.Westdc.2020.428

Share type:Apply to Access

Category:Meteorology

Datasets description

The data set includes the observation data of automatic meteorological station from January 1, 2008 to December 31, 2010. The site is located in hezhai village, Guantao County, Hebei Province. The underlying surface is corn / wheat and cotton. The longitude and latitude of the observation point are 115.1274e, 36.5150n and 30m above sea level. The data issued by the automatic weather station include: date / time, air temperature and humidity observation (ta_12.5m, Rh_12.5m (℃,%), wind speed (ws_12.7m) (M / s), wind direction (wd_12.7m) (℃), pressure (HPA), precipitation (rain) (mm), four component radiation (DR, ur, dlr_cor, ulr_cor, RN) (w / m2), surface radiation temperature (irt_1, irt_) (℃), soil heat flux (gs_1, gs_2) (w / m2) Multi layer soil moisture (MS UU 2cm, Ms UU 5cm, Ms UU 10cm, Ms UU 20cm, Ms UU 40cm, Ms UU 60cm, Ms UU 100cm) (%) and multi layer soil temperature (TS UU 0cm, TS UU 2cm, TS UU 5cm, TS UU 10cm, TS UU 20cm, TS UU 40cm, TS UU 60cm, TS UU 80cm, TS UU 100cm) (℃). For observation test or station information, please refer to Jia et al. (2012), and for data processing, please refer to Liu et al. (2013).

Data source description

The acquisition frequency of the automatic weather station is 10s, and it is output once in 10min. The observation elements include air temperature, relative humidity (12.5m), heading to the north; wind speed (12.7m), wind direction (12.7m), heading to the north; air pressure (installed in the waterproof box); rainfall (16m); net radiation / four component radiation (15.6m), heading to the South; infrared surface temperature (15.7m), arm heading to the south, probe heading to the vertical downward direction

Data processing method

The soil temperature and humidity probe is buried 2m to the south of the meteorological tower. The buried depth of the soil temperature probe is 0cm, 2cm, 5cm, 10cm, 20cm, 40cm, 60cm, 80cm and 100cm. The buried depth of the soil moisture sensor is 2cm, 5cm, 10cm, 20cm, 40cm, 60cm and 100cm. The soil heat flow plate (2 pieces) is buried underground 2 Cm, one is buried under the vegetation, the other is buried between trees.

Data quality description

Processing and quality control of observation data: (1) 1440 data (every 10min) shall be ensured every day, and if there is any missing data, it shall be marked by - 6999; (2) the time with duplicate records shall be eliminated; (3) the data that obviously exceeds the physical meaning or the measuring range of the instrument shall be deleted; (4) the format of date and time shall be unified, and the date and time shall be in the same column. For example, time: 10:30, June 10, 2010

Base information

collect time:2008/01/01-2010/12/31

collect place:

data size:32.8 MiB

data format:

Coordinate system: 无

Keywords

Theme:Soil heat flux, pressure, Air temperature and humidity, soil moisture , automatic weather station, wind speed, Surface radiation temperature
	Subject:
	Time:2008 2010
	Place:Guantao County,  Hebei Province,  Haihe River Basin

Space scope

|  | north： 36.50 |  |
| --- | --- | --- |
| west：115.12 |  | east：115.12 |
|  | south： 36.50 |  |

Service information

contacts:Li Hongxing

phone:0931-4967592

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:Liu shaomin,  Xu ziwei

author:Xu ziwei

manager:Xu ziwei

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

Liu shaomin, Xu ziwei. Multi-scale surface flux and meteorological elements observation dataset in the Hai River Basin (Guantao site-automatic weather . National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2020. https://cstr.cn/CSTR:11738.11.ncdc.Westdc.2020.428.

Article citation:

Liu Shaomin, Xu Ziwei, Zhu Zhongli, Jia Zhenzhen, Zhu Mingjia. Measurements of evapotranspiration from eddy-covariance systems and large aperture scintillometers in the Hai River Basin, China[J]. Journal of Hydrology, 2013, 487: 24-38.

Jia Zhenzhen, Liu Shaomin, Xu Ziwei, Chen Yujie, Zhu Mingjia. Validation of remotely sensed evapotranspiration over the Hai River Basin, China[J]. Journal of Geophysical Research, 2012, 117, (21).