Heihe River Integrated Remote Sensing joint test: ENVISAT ASAR ground synchronous observation data set in EBO encrypted observation area during the pre-test period (October 18, 2007)

DOI：10.12072/ncdc.NIEER.db1801.2022

CSTR：11738.11.ncdc.NIEER.2021.1779

Share type:Login to Access

Category:Remote Sensing

Datasets description

This data can provide a ground data set for the development and verification of active microwave remote sensing inversion algorithm and forward model of soil moisture and freeze-thaw state. On October 18, 2007, the ground synchronous observation test of ENVISAT ASAR data was carried out in EBU sample 1 and EBU sample 2. ENVISAT ASAR data is AP mode, VV / VH polarization combination mode, and the transit time is about 11:17bjt. Both EBAO sample 1 and EBAO sample 2 are 3grid × 3grid, 30m for each grid × 30m square, a total of 25 sampling points (including center point and corner point). Simultaneously with the satellite transit, the wet soil moisture meter is used to measure the real part of soil volume water content, soil conductivity, soil temperature and soil complex dielectric constant at each sampling point; The hand-held infrared thermometer is used to obtain the surface radiation temperature; The soil is taken with a ring knife and dried to obtain the weight water content, soil bulk density and volume water content. At the same time, some parameters of vegetation were investigated, mainly including vegetation height, coverage and vegetation water content This data can provide a ground data set for the development and verification of active microwave remote sensing inversion algorithm and forward model of soil moisture and freeze-thaw state.

Data source description

On October 18, 2007, the ground synchronous observation test of ENVISAT ASAR data was carried out in EBU sample 1 and EBU sample 2.

Data processing method

ENVISAT ASAR data is AP mode, VV / VH polarization combination mode, and the transit time is about 11:17bjt. Both EBAO sample 1 and EBAO sample 2 are 3grid × 3grid, 30m for each grid × 30m square, a total of 25 sampling points (including center point and corner point). Simultaneously with the satellite transit, the wet soil moisture meter is used to measure the real part of soil volume water content, soil conductivity, soil temperature and soil complex dielectric constant at each sampling point; The hand-held infrared thermometer is used to obtain the surface radiation temperature; The soil is taken with a ring knife and dried to obtain the weight water content, soil bulk density and volume water content. At the same time, some parameters of vegetation were investigated, mainly including vegetation height, coverage and vegetation water content.

Data quality description

Good data quality.

Base information

collect time:2007/10/18-2007/10/19

collect place:Heihe River Basin. EBO intensive observation area. Hydrological test area in the upstream cold area

data size:38.0 KiB

data format:excel

Coordinate system: WGS84

Keywords

Theme:Aerospace remote sensing, soil, microwave remote sensing, surface radiation temperature, surface radiation characteristics, soil temperature, soil moisture / moisture content, ground verification information, airborne ground remote sensing
	Subject:physical geography 
	Time:2007
	Place:EBO intensive observation area,  upstream cold region hydrological test area,  Heihe River Basin

Space scope

|  | north： 38.97 |  |
| --- | --- | --- |
| west：100.59 |  | east：100.44 |
|  | south： 38.04 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:JIN Rui

author:JIN Rui

manager:JIN Rui

Project information

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

JIN Rui. Heihe River Integrated Remote Sensing joint test: ENVISAT ASAR ground synchronous observation data set in EBO encrypted observation area during the pre-test period (October 18, 2007). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2022. https://cstr.cn/CSTR:11738.11.ncdc.NIEER.2021.1779.