Test data set of mechanical properties and mineral composition of typical loess in Gansu Province, Qinghai Province and some areas of Ningxia Hui Autonomous Region (2019-2020)

DOI：10.12072/ncdc.LANDSLIDE.db2035.2022

CSTR：11738.11.ncdc.LANDSLIDE.db2035.2022

Share type:Apply to Access

Category:Hydrology

Datasets description

The data set includes the tests of clay mineral types and total mineral analysis of different soil properties in different regions of the Loess Plateau and the tests of mechanical property indexes such as shear strength, tensile strength and compressive strength of Malan Loess in Lanzhou, Gansu Province under different water content with strong disturbance of human engineering activities, so as to obtain various mechanical strength parameters.

Data source description

PW 3710 X-ray diffractometer of Philips company in the Netherlands was used to test and analyze the mineral composition.

Data processing method

After air drying, take about 10 g of all samples for analysis of total mineral composition. The diffraction mode of the test adopts cu-k target radiation, the tube pressure is 35 kV and the tube flow is 25 mA, and the scanning speed is 0.03 ° / min θ The data obtained in the range of 5 ° ~ 45 ° were used for result analysis. After the test, use the PDF card in the special MDI jade software to compare and fit the width and height of the diffraction peak of the XRD diffraction pattern of the test sample, and then confirm the mineral composition and analyze the characteristics of the mineral structure based on the characteristic peak of the target mineral.

Data quality description

The operation specification conforms to the standard process, and the data quality is good.

Base information

collect time:2019/01/01-2020/12/31

collect place:Lanzhou City, Tianshui City, Longnan City, Xining City, Guyuan City, Xiji County

data size:3.7 MiB

data format:

Coordinate system: 无

Keywords

Theme:Malan loess, mechanical properties, mineral composition
	Subject:physical geography 
	Time:2019 2020
	Place:Lanzhou City,  Tianshui City,  Longnan City,  Xining City,  Guyuan City

Space scope

|  | north： 36.28 |  |
| --- | --- | --- |
| west：101.04 |  | east：105.92 |
|  | south： 33.92 |  |

Service information

contacts:Min Yufang

phone:0931-4967596

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:

author:ZHANG Fanyu

manager:

Project information

number:2018YFC1504702，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

. Test data set of mechanical properties and mineral composition of typical loess in Gansu Province, Qinghai Province and some areas of Ningxia Hui Autonomous Region (2019-2020). National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2022. https://cstr.cn/CSTR:11738.11.ncdc.LANDSLIDE.db2035.2022.