The soil-water characteristic curves of different loess under different temperature conditions

Share type:Apply to Access

Category:Others

Datasets description

Data source description

The four loess samples were from in-situ sampling (Lanzhou: N:36.11, E:103.794; Suide: N:37.48, E:110.492; Guyuan: N:35.96, E:105.76; Tianshui: N:34.92, E:105.678).  The testing method used was the filter paper method, and the filter paper used was the Whatman No. 42 filter paper.  The enverinonment chamber used for temperautre control were: isotemp chamber (-35~+60°C with accuracy of ±0.1°C), isotemp freezer (-26~+10°C with accuracy of ±0.5°C), and oven (Room Temp+10~+200°C with accuracy of ±1.0°C).

Data processing method

The calibration equations used for the filter paper were based on Haghighi et al. (2012), Jin et al. (2020), and Zeng et al. (2023).  Haghighi, A., Medero, G. M., Marinho, F. A., Mercier, B., & Woodward, P. K. (2012). Temperature effects on suction measurement using the filter paper technique. Geotechnical Testing Journal, 35(1), 83-90.  Jin, F.Y., Wang, Y.J., Sun, D.A., & Zeng, L.L. (2020). Temperature effect on the water retention curve of lime-treated soil during curing. Journal of Civil and Environmental Engineering, (03), 24-31.  Zeng, Z.T., Cui, Z.Q., Sun, D.A., Yao, Z., & Pan, B. (2023). Temperature effect on water retention capacity of Nanning expansive soil and its microscopic mechanism. Rock and Soil Mechanics, (08), 2177-2185.

Data quality description

The filter paper calibration equations used are relatively accurate, and the measured soil-water characteristic curve demonstrates high reliability, providing data support for interpreting the physical and mechanical behaviors of the four types of loess.

Base information

collect time:2024/01/01-2024/12/31

collect place:Lanzhou, Suide, Guyuan, Tianshui

data size:36.0 KiB

data format:.xlsx

Coordinate system: 无

Keywords

Theme:Loess, Soil-water characteristic curve, Temperature
	Subject:Engineering Geology 
	Time:2024
	Place:Lanzhou,  Suide,  Guyuan,  Tianshui

Space scope

|  | north： |  |
| --- | --- | --- |
| west： |  | east： |
|  | south： |  |

Service information

contacts:Li Hongxing

phone:0931-4967592

mailbox:ncdc@lzb.ac.cn

Contact information

contributors:ren jun ping

author:ren jun ping

manager:ren jun ping

Project information

number:2022YFC3003401，
      name:

Citations and annotations

In order to protect the rights and interests of the platform's scientific and technological resources, expand the services of the platform center, and enhance the application potential of scientific and technological resources,resource users are requested to mark and quote the research results (including published papers, treatises, data products, unpublished research reports, data products, etc.) generated by the use of resources in the following manner.

For the achievements published in English, please refer to the following specifications：

The dataset is provided by National Cryosphere Desert Data Center(https://www.ncdc.ac.cn).

Data reference:

ren jun ping. The soil-water characteristic curves of different loess under different temperature conditions. National Cryosphere Desert Data Center(http://www.ncdc.ac.cn), 2026.